3GPP TSG RAN WG4 Meeting #117	R4-2521276

Dallas, USA., 17th – 21st, Nov. 2025

# Title: 	On R20 low-low band CA via switching

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.3

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, a new WID on enhancement of low NR band carrier aggregation via switching was approved.

The objectives are shown below:

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

The reasons are similar with Rel-19 LBCA switching, which is there exists some challenges to implement the low-low band CA band combination due to poor antenna implementation when sharing one common antenna, or cost/size issue when separated antennas are used.

In last meeting, there are some agreements included and meanwhile, there are still some open issues which need to be discussed. In this contribution, we gives some discussions on some open issue for the enhancement of low-low band CA via switching based on [3].

# 2	Discussion

Comparing with the Rel-19 LBCA switching, additional switching case is introduced. Thus there are three cases up to Rel-20 for LBCA switching.

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

For Rel-19 LBCA switching, it is the switching case between Case 1 and Case 2, which is switchedDL option, and For Rel-20 enhanced LBCA switching, it is the switching case between Case 1 and Case 3, which is dual DL option. It should be noted that there is no switching happened between Case 2 and Case 3.

The different switching scenarios are applied to the following FDD+SDL LBCA combination:

1. Rel-19 LBCA switching mechanistic (i.e. single DL switched) applies to CA_n5-n29, CA_n12-n29, CA_n14-n29, CA_n28-n67 and CA_n29-n71 according to the CR[2].

2. Rel-20 LBCA switching mechanistic (i.e. dual DL switched) applies to CA_n12-n29, CA_n28-n67 according to the WID

It should be noted that more LBCA combination will be introduced via basket WID. All band combination are comprised of single carrier in each band.

It can be seen that for CA_n12A-n29A and CA_n28A-n67A can support both single DL switched and dual DL switched options. However, we think it may not support both switching options for all FDD+SDL due to large Rx frequency separation between the two band such as CA_n5A-n29A.

The frequency information for CA_n12A-n29A and CA_n28A-n67A can be found in table 1 from TR38.768.

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n12A-n29A |  | (1) |
| CA_n28A-n67A |  | (2) |
| NOTE 1: Aggregation of n12 + n29 DL with no UL on n12 and depending on UE RF front end capability is targeted for switching Case 3 NOTE 2: Aggregation of n28 + n67 DL with no UL on n28 and depending on UE RF front end capability is targeted for switching Case 3 |  |  |

For the UE RF reference architecture, it was agreed:

Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states - UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information

The UE RF reference architecture for Rel-19 switching scenarios was not discussed, but it can be found in the corresponding WID motivation paper, which is for Rel-19 and Rel-20 LBCA switching are illustrated in Figure 1.

Figure 1. Rel-19 LBCA switching scenario

It can be found that the UE RF architecture are different between Rel-19 and Rel-20 LB CA switching, the main different is that a combined filter shall be used to support dual DL receiving in both SDL carrier 2 and FDD DL carrier 1 for Rel-20 dual DL switching, while for Rel-19 single DL switching, single band filter is used for either SDL carrier2 or FDD DL carrier 1.

Observation 1: The UE RF architecture are different between Rel-19 and Rel-20 LB CA switching.

The open issues for the UE RF architecture are:

- Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

For a combined filter to support SDL n67+ full n28 DL, the total supported maximum channel bandwidth is 60MHz (i.e. 20MHz for n67 + 40MHz for full n28 DL). In general, for a normal SAW filter, the relative channel bandwidth is about 3%~6%, then the supported channel bandwidth is about 23MHz to 46MHz, which cannot achieve 60MHz. However, for a high performance SAW filter such as TC-SAW (Temperature Compensation SAW), the relative channel bandwidth can be up to 5%~10%, then the supported channel bandwidth is about 38.4MHz to 76.8MHz. Considering a typical 8% relative channel bandwidth, the supported channel bandwidth is 61.44MHz which is also larger than 60MHz. Although the cost of TC-SAW is higher than normal SAW, it would be still feasible to use a combined filter to support SDL n67+ full n28 DL.

Observation 2: A high performance SAW filter would be needed to support SDL n67+ full n28 DL.

For the UE architecture, in our understanding, it can be applied for both full band n28 and partial n28 operation to be combined into a common filter with band n67 although there exists overlap between SDL n67 DL and FDD n28 UL as there are only SUL+FDD DL carriers and no Tx on FDD carrier in case 3.

Observation 3: The above UE RF architecture can be applied for both full band n28 and partial n28 operation.

Proposal 1: With a high performance SAW filter, it is feasible to use a combined band n28+n67 DL filter to support 60MHz channel bandwidth and the full band n28 implementation

For CA_n12A-n29A, there is only 1MHz separation between n29 DL upper edge and n12 DL lower edge. It would not possible to use two separate band filters (i.e. duplexer) to combine n12 and n29 DL in the case of the frequency of the two bands is very close. Due to there is no incumbent narrowband service in the band gap between n29 and n12 (i.e. 728~729 MHz) in Canada [4], the above example UE reference architecture can be applied.

For some other scenario like CA_n5A-n29A, CA_n14A-n29A, CA_n29A-n71A, the frequency gap for these low low CA scenarios are large, then it requires two separate filters for the two DL band. It is illustrated in Figure 3.

Figure 2. Main path for the architecture of LBCA switching scenarios have large frequency gap

Proposal 2: Two separate filters for the two DL band is needed for the LBCA switching scenario with large separation between two DL bands.

REFSENS:

The SDL band, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. For inter-band CA, the reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Obviously, it is not applied to R20 LBCA switching due to there are no Tx on both FDD carrier and SDL carrier.

The REFSENS values for SDL band are defined as equation-based scaling with RB number, it is the same with TDD approaches without considering the self-interference from Tx carrier.

For R20 LBCA switching, we think the existing REFSENS requirements for FDD single carrier with Tx/Rx and SDL carrier should be kept. For REFSENS requirements for FDD single carrier with Rx only, relaxed values can be considered.The relaxed value may be -100+10log10(NRB/25), which is same with other FDD band like n1 supporting 5MHz who has no impact from its Tx due to large separation.

Proposal 3: For FDD carrier with Tx/Rx, the existing REFSENS requirements can be reused. For FDD carrier with Rx only, relax REFSENS requirements should be consider without Tx impacts.

Proposal 4: For SDL carrier, the existing REFSENS requirements can be reused.

Proposal 5: For SDL+FDD carrier use wideband receiver filter, the reference sensitivity is defined to be met with all downlink component carriers active without the uplink carriers active.

For the co-existence issue in the WF:

1. For the UE-UE co-existence: In the specification, there is spurious emission for UE co-existence requirements. However, there is neither E-UTRA 29 (i.e. NR n29) as protected band in n12 nor E-UTRA 67 (i.e. NR n67) in the TS38.101-1. The frequency of n29 is close to n12, then the ACLR/SEM could be used as a metric to restrict the emission from band n12 (especially high part) falls into the n29. If not, the possible interference can also be alleviated by NW scheduling, i.e. the band n12 high part is not scheduled in the case of n29+n12 DL exists, or by the physical distance between the two UEs. Similarity as n28-n67. For the letter one, the possible interference can also be alleviated by restricting n28 to the low part range (i.e. 703MHz~733MHz).

2. For the BS-BS co-existence: For single band n12, there is a NOTE 5 to describe the additional spurious emissions requirements for NR BS operating in n29, see below:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 12 or NR Band n12 | 699 – 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.2.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5). |

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band.

It was already identified that the specific solutions may be required to fulfil the spurious emissions limits for NR band n29 BS to co-existence with NR band n12. However, there are no solutions specified in the specification. Thus the solutions would be depends on BS implementation such as using high performance Rx filter in n29+n12 DL.

For single band n28, due to the overlaps with n67, it would not possible for a BS to use high performance Rx filter to reject the n28 UL in the overlapping frequency range. Thus one of the solution is to restrict n28 to the low part range (i.e. 703MHz~733MHz).

3. Blocking: For CA_n12A-n29A and CA_n28A-n67A, we think new notes should be added in out of band blocking Table 7.6.3-2 in TS38.101-1, similar as note 10 for CA_n20A-n28A, which is to consider the whole DL frequency range of the two DL bands.

NOTE 10	For a UE supporting CA_20A-28A and higher order band combinations in which CA_20A-28A is a subset, the requirements for Band n20 and Band n28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band n28 and FDL_high by Band n20.

With the above consideration, we have the following observations and proposals:

Observation 4: For BS-BS co-existence for n29 and n12, specific solutions may be required to fulfil the spurious emissions limits, and it would depend on BS implementations.

Proposal 6: To limit the n28 operation to the Region 1 range 703-733/758-788 for CA_n28A-n67A.

Proposal 7: For CA_n12A-n29A and CA_n28A-n67A, two new NOTEs in out of band blocking Table 7.6.3-2 in TS38.101-1 should be added to include whole DL frequency range of the two DL bands.

In the feature list [5], the switching cases are included:

It was agreed that new UE capability will be introduced and the capability shall be per band pair per band combination. Thus for R20 dual DL switching, the new switching cases should be informed to RAN2 to update the signalling (existing one or introduce new signalling) if necessary, which depends on RAN2 decision.

Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination

Proposal 8: Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating.

# 3 Conclusion

In this contribution, we give some discussions on the R20 enhancement FDD-SDL LBCA downlink carrier switching.

The observations, proposals and conclusions are:

Observation 1: The UE RF architecture are different between Rel-19 and Rel-20 LB CA switching.

Observation 2: A high performance SAW filter would be needed to support SDL n67+ full n28 DL.

Observation 3: The above UE RF architecture can be applied for both full band n28 and partial n28 operation.

Proposal 1: With a high performance SAW filter, it is feasible to use a combined band n28+n67 DL filter to support 60MHz channel bandwidth and the full band n28 implementation

Observation 4: For BS-BS co-existence for n29 and n12, specific solutions may be required to fulfil the spurious emissions limits, and it would depend on BS implementations.

Proposal 2: Two separate filters for the two DL band is needed for the LBCA switching scenario with large separation between two DL bands.

Proposal 3: For FDD carrier with Tx/Rx, the existing REFSENS requirements can be reused. For FDD carrier with Rx only, relax REFSENS requirements should be consider without Tx impacts.

Proposal 4: For SDL carrier, the existing REFSENS requirements can be reused.

Proposal 5: For SDL+FDD carrier use wideband receiver filter, the reference sensitivity is defined to be met with all downlink component carriers active without the uplink carriers active.

Proposal 6: To limit the n28 operation to the Region 1 range 703-733/758-788 for CA_n28A-n67A.

Proposal 7: For CA_n12A-n29A and CA_n28A-n67A, two new NOTEs in out of band blocking Table 7.6.3-2 in TS38.101-1 should be added to include whole DL frequency range of the two DL bands.

Proposal 8: Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating.

# 4	Reference

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei).

[2] R4-2515156, Way Forward for [116bis][320] NR_low_band_CA_switching, Apple

[3] R4-2513997, Discussion on R20 low-low band CA via switching, ZTE Corporation, Sanechips

[4] R4-RP-252702, Motivation for low-band carrier aggregation via switching enhancement in Rel-20,TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T

[5] R4-2511883, Rel-19 RAN4 UE feature list for NR (version 2), CMCC