3GPP TSG-RAN WG4 Meeting #117										    R4-2521330

Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Title: 	On RRM requirements for NR MIMO Phase 6 Enhancements

Agenda Item:	7.10.3

Document for:	Discussion

Introduction

In last meeting, a WF was approved on NR MIMO phase 6 [1].

In this document, we provide our views on the RRM requirements of NR MIMO phase 6.

Discussion

- Topic #2: Enhancing UL capacity and coverage

Issue 2-1-1: Whether to introduce additional RRM requirements for SRS antenna port switching in cross-slot SRS

It is proposed that RAN4 hold on any discussion on defining new or modified RRM requirements for this scenario until there is further progress and clearer technical direction from RAN1.

The fundamental design aspects, such as the applicable scenarios and UE behavior for cross-slot SRS antenna port switching, are still under discussion in RAN1,

Therefore, we recommend waiting for RAN1, at least including the following aspects:

- Permissible SRS port switching patterns in cross-slot scenarios

- Slot boundary handling for phase continuity and power control

Once stable, RAN4 can then efficiently assess the specific RRM impacts and define appropriate requirements.

Proposal 1: Wait for RAN1 progress before defining new RRM requirements.

Issue 2-1-2: Whether to introduce additional RRM requirements for SRS carrier switching in cross-slot SRS

The current RAN1 agreements on cross-slot SRS focus on single-carrier scenarios, so we don't yet see a strong use case for SRS carrier switching. Since the applicability of multi-carrier cross-slot SRS are still undefined, it's too early to assess any potential RRM impacts. RAN1 should clarify:

- Whether cross-slot SRS supports simultaneous multi-CC transmission

- Impact on PUSCH scheduling and UL timing alignment

Proposed 2: Defer discussion until RAN1 expands scope to multi-carrier cross-slot SRS.

- Topic #3: Enhancing DL CSI Acquisition

Issue 3-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

The introduction of early triggering for SRS, CSI-RS, and CSI reporting during the transition from IDLE/INACTIVE to CONNECTED mode represents a significant enhancement aimed at accelerating link adaptation. However, the fundamental procedures and associated timelines for this early triggering mechanism are currently under active discussion in RAN1.

RAN4 should await further progress in RAN1 on finalizing the technical details for early SRS/CSI-RS/CSI triggering, including how and when the UE receives configurations, reports capabilities via MSG3, and processes the trigger in MSG4—must be solidified before RAN4 can meaningfully assess the impact on RRM timelines.

Alternatively, considering that RAN4 has never defined specific RRM requirements for the IDLE/INACTIVE to CONNECTED transition delay itself, a strong argument can be made that no new timeline requirements are necessary for this early triggering feature. The existing framework may be sufficient, and the benefits of early triggering can be realized without introducing new RAN4 mandates.

Therefore, we propose that RAN4 either:

- Postpone discussion until RAN1 agreements are stable, or

- Decide not to introduce new timeline-related requirements for this use case.

Proposal 3: RAN4 either to postpone discussion until RAN1 agreements are stable, or decide not to introduce new timeline-related requirements for this use case.

Issue 3-2-1: Applicable scenarios for early triggering during SCell activation

RAN4 needs to first clarify the practical use cases and expected benefits of introducing early triggering mechanisms for aperiodic CSI-RS and aperiodic SRS-AS during SCell activation. The applicability likely depends on specific SCell activation scenarios, such as activation of known or unknown cells, or activation with and without TCI state indications.

- Known TCI State: UE can immediately measure aperiodic CSI-RS after activation command.

- Unknown TCI State: Requires SSB-based beam selection first, potentially negating early triggering benefits.

Furthermore, RAN4 should discuss and agree on the specific SCell activation cases (e.g., based on existing delay requirements in clauses 8.3.2, 8.3.12, etc.) for which early SRS/CSI/CSI-RS triggering is applicable.

Proposal 4: RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering, and pprioritize known-TCI-state scenarios for discussion.

Issue 3-2-3: RRM impact due to early SRS/CSI-RS/CSI triggering for SCell activation

In our view, there is no need to define new timeline specifically for early CSI/CSI-RS triggering. The existing SCell activation delay requirements already account for the time until the first valid CSI report, and early triggering appears to be an optimization within this timeline.

When early SRS is triggered, THARQ (for acknowledging the activation command) and Tactivation_time (for downlink synchronization) should remain unchanged. Any CSI/CSI-RS/SRS measurement and report must be conducted after the UE has completed downlink synchronization on the SCell. RAN4 can further discuss whether the component TCSI_reporting needs to be updated when the completion event is an SRS transmission instead of a CSI report. We also recommend waiting for further RAN1 progress on the technical details of the triggering mechanism via the SCell activation command before finalizing RRM requirements.

Proposal 5: For early CSI/CSI-RS triggering for SCell activation, no need to define new timeline

Proposal 6: For early SRS triggering SCell activation, THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

Proposal 7: Wait for RAN1 further agreements on the early CSI/CSI-RS/SRS triggering via SCell activation.

Conclusions

Based on above analysis, we have following observations and proposals.

- Topic #2: Enhancing UL capacity and coverage

Issue 2-1-1: Whether to introduce additional RRM requirements for SRS antenna port switching in cross-slot SRS

Proposal 1: Wait for RAN1 progress before defining new RRM requirements.

Issue 2-1-2: Whether to introduce additional RRM requirements for SRS carrier switching in cross-slot SRS

Proposed 2: Defer discussion until RAN1 expands scope to multi-carrier cross-slot SRS.

- Topic #3: Enhancing DL CSI Acquisition

Issue 3-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

Proposal 3: RAN4 either to postpone discussion until RAN1 agreements are stable, or decide not to introduce new timeline-related requirements for this use case.

Issue 3-2-1: Applicable scenarios for early triggering during SCell activation

Proposal 4: RAN4 to discuss the applicable SCell activation cases for early SRS/CSI/CSI-RS triggering, and pprioritize known-TCI-state scenarios for discussion.

Issue 3-2-3: RRM impact due to early SRS/CSI-RS/CSI triggering for SCell activation

Proposal 5: For early CSI/CSI-RS triggering for SCell activation, no need to define new timeline

Proposal 6: For early SRS triggering SCell activation, THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

Proposal 7: Wait for RAN1 further agreements on the early CSI/CSI-RS/SRS triggering via SCell activation.

- Reference

[1]. R4-2514825, WF on NR MIMO phase 6 RRM.