3GPP TSG-RAN WG4 Meeting #117															    R4-2521331

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.6

Source:	OPPO

Title:	Views on RAN4 6G RRM

Document for:	Discussion

Introduction

According to the approved 6G SID [1] and WF [2] in last meeting, companies are encouraged to provide analysis and views on at least RAN4 driven non-AI RRM topics.

### Recommendation for 6G RRM contribution preparation for RAN4#117 meeting Strongly encourage companies to structure contribution and categorize proposals based on the following categories for RAN4#117: - Measurement gap(MG) and interruption - Including MG and interruption related proposals - RRM framework: Measurement capability/delay/overhead/accuracy/quantity/unified-measurement - Mobility related RRM - RRM related energy efficiency - Spectrum aggregation related RRM - MIMO and mTRP operation related RRM - NTN related RRM - Initial access related RRM - Other PHY signal/channel/procedure related RRM

In this contribution, we will provide our initial considerations on RRM aspects for 6G.

Overview of 6G RRM

The discussion of 6G physical layer signal/channel/procedure, coverage and energy saving schemes have been started in RAN1 from August meeting. Based on the functionality of physical layer and protocol stack timeline, RAN4 will discuss how UE performs transmission and reception including measurement, and investigate what are the minimum RRM requirements for UE.

In our view, RAN4 can start the study of RRM impact by RAN1/2 new design for 6G as early as possible, especially physical layer signal/procedure, coverage and energy saving schemes. The outcome of RAN4 study can also help RAN1/2’s design to become more reasonable and feasible.

For instance, once RAN1 figured out the reference signals for measurement and RAN2 designed RRC reconfiguration and procedures for handover, then RAN4 would define and test the delay and interruption time. In this case, RAN4 can convert RAN1/RAN2 designs into testable metrics, ensure UE performance and consistency across diverse scenarios, and adjust test boundaries for new features.

The RRM impact of RAN1/2 new design for 6G needs to be studied as early as possible in RAN4, especially physical layer signal/procedure, coverage and energy saving schemes.

Meanwhile, RAN4 can also trigger some RAN4 led work items, e.g., Measurement Gap and interruption, RRM framework, which have less impact to RAN1/2. How to select other topics which are more RAN4 centric needs to be discussed in RAN4. In our view, the selected features can be used as baseline for 6G discussion in RRM area.

From our side, the following RRM functions are suggested with higher priority to be studied in RAN4, at least including:

- unified RRM measurement, e.g., in order to simplify UE capabilities and RRM requirements for supporting both L3 and L1/L2 triggered mobility

- measurement GAP and interruption, e.g., unified design for diverse gaps like pre-MG/NCSG/concurrent gap, and complicated gap skipping or collision rules.

- spectrum aggregation operation, e.g., RRM impact due to enhanced CA or SCMC (single cell multi-carriers) being discussed in other RAN WGs

- UE and network energy savings, e.g., support of RRM measurement relaxation, on-demand measurement, DRX, LP-WUR based measurement relaxation/offloading, low-capability device types (like Redcap UE, IoT-like device in 5G) and other solutions are not precluded depending on RAN1/2 conclusion

Suggest RAN4 to firstly focus on the following topics with more supporting companies and less dependency on other WGs.

Measurement GAP and interruption

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement

Regarding the observations, it is worthy to consider a harmonized design for 6G RRM. Comprehensive assumptions for RRM measurement, need to be considered in 6G Day1. RAN4 needs to focus on the most important RRM KPIs, such as measurement delay and accuracy, mobility latency, carrier aggregation latency, interruption, etc.

RAN4 to investigate a harmonized design for 6G RRM, focusing on the most important RRM KPIs, such as measurement delay and accuracy, mobility latency, carrier aggregation latency, interruption, etc.

Besides, RF and baseband assumption also needs to be studied in RAN4 since new spectrum for 6G will be introduced. The impact due to new spectrum allocation (e.g., FR3) can be studied parallelly in RAN4 RF and RRM. For example, antenna & RF architecture, frequency bands & bandwidth for 6G UE are going to be studied in RAN4 RF, which can also trigger the discussion of RRM impact, which may impact on the discussion of RRM measurement delay or measurement gap length, e.g.,

- how UE Rx branches (e.g., 1Rx or 2Rx) affect measurement delay and accuracy,

- whether Rx beam sweeping is needed for FR3,

- which RF tuning time for FR1 or FR2 can be used for FR3.

Study RRM impact of 6G new spectrum based on new UE RF/baseband assumption.

Measurement GAP and interruption

Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is considered as part of RAN4 RRM 6G study. The detailed scope will be further decided. Interruption enhancement due to gap-less measurement will be part of the MG discussion.

In this clause, we provide our view on measurement gap and interruption for 6G RRM.

Measurement GAP

- FFS followings in RAN4#117 (other sub-topics are not precluded) - based on the majority views from companies, RAN4 RRM to discuss which MG related sub-topics can be prioritized from the following candidate list, and then discuss the solutions for the selected topics in 6G SI: - MG pattern reduction from 5G (10 companies: Apple, MTK, QC, OPPO, Sony, Xiaomi, vivo, Ericsson, Samsung, Nokia) - Gap-less measurement and its side conditions (10 companies: MTK, Samsung, CMCC, ZTE, NTT DCM, HW, Ericsson, Nokia, QC, Xiaomi) - In scenarios with and without an available RF chain. - Unified MG (7 companies: Apple, OPPO, LGE, Xiaomi, ZTE, Ericsson, QC), e.g., - Unified MG configuration - Unified MG and scheduling restriction - Unified MG for different feature related measurements - Adapative MG operation, e.g., activation/deactivation/cancellation/skipping (7 companies: Apple, OPPO, CMCC, Xiaomi, ZTE, Ericsson, LGE) - Using which 5G MG enhancement features to 6G day 1 (OPPO, CMCC, Xiaomi, vivo) - E.g., needforGap, NCSG, concurrent MG, preconfigured MG and etc. - MG sharing(Apple, OPPO, vivo), e.g., - among intra-frequency, inter-frequency, and inter-RAT measurement (including L3 and L1 measurement) - UE assisted MG configuration (Apple, QC) - e.g., MG requesting by UE (Apple, QC) - Multi-CC measurements in MG (Apple, CMCC, ZTE(searcher related)) - Optimization on MGL and RF tuning/retuning (Apple, MTK, QC) - MG applicability for per-UE, per-FR, per-CC, or per-CC group (OPPO, Xiaomi, ZTE(granularity))

Due to UE’s hardware restrictions (e.g., RF tuning, transceiver or other Rx processing units’ adjustment), UE may not be able to maintain connection with the serving cell when performing measurements, especially when the target measurement resources is not contained within the active BWP. In this case, gap (with explicit location and length) is a good choice to enable measurement and meanwhile avoid unknown data loss. Clearly, the gap plays an important role in measurement and will be used in 6G.

NR R15 and R16 focus on the fundamental design of measurement gaps. NR Rel-17 MGE WID introduces the following three new GAPs, including Pre-MG, concurrent MG and NCSG.Fom RRM perspective, R17 only defines requirements for each individual feature and further the joint operations for case 1 con-MG with Pre-MG and case 2 con-MG with NCSG are considered in Rel-18. However, the RRC signalling in RAN2 already considered the joint operation from R17 and these is a mis-alignment between RAN2 configuration and RAN4 RRM requirements.

As below is a brief summary for measurement gap enhancement in RAN4 from R17 to R19. Devise types of gaps and different solutions for gap activation/gap collision/gap skipping are discussed and specified in different work items in RAN4, which make current RRM requirements related to measurement gap in TS38.133 quite obscure and difficult to understand.

The features of RAN4 measurement gap enhancement

    - Unified GAP and adaptive operation

As 6G is expected to be more efficient and flexible than 5G NR, a unified GAP design covering all above enhanced features should be studied, including new joint combinations. UE may need to support a unified GAP configuration or capability to cover different types of MG, like pre-configured gap, parallel configured gap, and small gaps. In this way, less capability reporting and network configuration overhead can be achieved.

And the benefits of different gaps can be considered again in 6G GAP. We observed the benefits of different type of gaps.

- Pre-MG: support UE autonomous or NW-controlled MG (de)activation in case of active BWP switching, MO addition/removal, and SCell activation/deactivation and so on. In this way, the frequent RRC signalling to reconfigure MG can be avoided and the related latency can be reduced.

- Concurrent MG: support multiple concurrent gaps in parallel, where each of them is associated with different MOs. Compared with single per-UE gap or single per-FR gap for each FR, con-MG is beneficial for the case when the target MOs are mis-aligned in the time domain. For example, if MG is required for MO1 (160ms periodicity and 0ms offset) and MO2 (160ms periodicity and 40ms offset), then NW have to configure a MG with 40ms periodicity as shown in Figure 2(a) without con-MG, and half of MG occasions are redundant. For UE supporting con-MG, NW can configure two MG with 160ms periodicity as shown in Figure 2(b).

Concurrent Gap

- NCSG: achieve a good trade-off between measurement/mobility and data throughput by reducing the interruption due to measurements via vacant RF chains. Compared with normal MG, only two small interruptions (1ms or 0.75ms) are allowed in VIL and data transmission could be maintained during ML.

Unified Gap

To enable more efficient and flexible measurement, unified GAP needs to be studied to cover different types of gaps, including pre-configured gap, parallel configured gap, and small gaps.

However, there also existing some problems of 5G gaps. We provide several examples as below, and these issues need to be avoided in 6G gap design.

- Issue #1: The deactivated SCell is measured with NCSG by default, which is a compromised solution

NCSG is required for deactivated SCell measurement. When the SCell is activated, this SCell may be measured without GAP, depending on whether the SSB is contained in the active BWP. In this case, the fast switch between NCSG and SMTC (e.g., implemented by Pre-NCSG activation), or transition between NCSG and MG is preferred. In NR, RAN4 discussed this scenario in case of con-MG with NCSG, and reached a conclusion that the deactivated SCell is measured with NCSG by default, which is a compromised solution.

- Issue #2: Fixed MGL may be not efficient

when MO1 with 1ms SMTC duration and MO2 with 5ms SMTC duration are expected to be measured in the same MG, network can only configure a MG with 6ms MGL to cover both MOs in 5G NR. For the MG occasion measuring MO1, 4ms duration are wasted. To improve efficiency, the MGL adapted with the MO can be considered.

- Issue #3: Pre-MG activation/deactivation based on per BWP indication is quite complicated

The Pre-MG (de)activation mechanism is highly dependent on the BWP. For NW-controlled mechanism, the (de)activation bit is configured for each BWP and UE is required to check this information for every active DL BWP. For UE autonomous mechanism, Pre-MG should be activated if UE cannot perform the measurement without gap, regardless whether some or all target SSBs are contained in the active DL BWPs. The current Pre-MG may not be flexible enough for 6G. Especially, Pre-MG status indication is carried in each BWP configuration and the (de)activation is quite complicated. We think per-UE signalling based scheme may be more reliable and easier. In addition, dynamic signalling like MAC CE or DCI can be considered, which is already supported in NR, e.g., pre-configured MG activated by MAC CE in R18 Positioning WI.

- Issue #4: DCI based dynamic Gap cancelling is not perfect

R19 XR agreed on DCI based gap cancelling solution. But RAN4 debated for a long time on the cons and pros of semi static and dynamic GAP skipping. RAN4 should be involved earlier in 6G and study the scheme of GAP skipping or cancelling, and evaluate the feasibility and RRM impact of different skipping rules.

In summary, RAN4 needs to come up with a solution to enable efficient adaptation among all supported GAPs for UE in 6G. In our view, the following aspects can be considered in RAN4:

- Unified GAP configuration and efficient GAP change or gap adaptation

- Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

- Semi static and dynamic GAP skipping or cancelling

- Unified solution for gap collision

Study the following aspects for unified GAP design:

Unified GAP configuration and efficient GAP change or gap adaptation

Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

Unified solution for GAP cancelling schemes

Unified solution for GAP collision

    - GAP pattern reduction

As per gap patterns, 4 MG patterns with MGL= {3ms, 6ms} and MGRP= {40ms, 80ms} are defined in LTE. NR introduced up to 26 MG patterns and 24 NCSG patterns, including all the LTE gaps for inter-RAT LTE measurements, and 20ms/160ms periodicity considering the flexible SSB/CSI-RS design.

Besides, FR2 specific MGL (e.g., 5.5ms) are introduced due to the short RF tuning time. The gap patterns in 6G should match with target measurement objectives. As mentioned in [1], 5G NR and 6G interworking and coexistence should be supported, which means the gap patterns in 6G should cover at least 6G measurement and 5G NR measurement.

The gap patterns in 6G should match with target measurement purposes, including both 5G NR measurement and 6G measurement.

NR measurement gap patterns

| MG patterns |  | MGRP | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | MGL | 3ms | #10 | #2 | #3 | #11 |
|  |  | 4ms | #6 | #7 | #8 | #9 |
|  |  | 6ms | #4 | #0 | #1 | #5 |
| FR2 | MGL | 1.5ms | #20 | #21 | #22 | #23 |
|  |  | 3.5ms | #16 | #17 | #18 | #19 |
|  |  | 5.5ms | #12 | #13 | #14 | #15 |

On the other hand, it is unrealistic to define many gap patterns perfectly matching with for every SSB/CSI-RS configuration. Too many gap patterns will also lead to complicated gap applicability rules as defined in clause 9.1.2 of TS 38.133. In our view, 6G should avoid this situation and strive to reach a small set of gap patterns. During last meeting, about 10 companies proposed to reduce gap patterns and we also think this is a worthy direction to study. Then, the question is how to achieve it.

One approach to reduce the number of gap patterns is to focus on the most common and typical use cases. For 5G NR measurement, NR gap patterns should be supported, but not all of them. In our view, the mandatory NR gap patterns are generally aligned with typical measurement objective configuration in practice and should be prioritized. The remaining non-mandatory gap patterns could be deprioritized or even excluded. In Rel-15, MG pattern ID 0 and 1 are mandatorily supported without UE capability report, and MG pattern ID 13, 14, 17, 18 and 19 are additionally supported for FR2 with UE capability report. In Rel-16, MG pattern ID 2, 3 and 11 are mandatory for NR only measurement in NR SA and NR-DC scenarios. If only NR mandatory gap patterns highlighted in Table 1 are considered, the number of gap patterns could be reduced to 10.

Besides, some of gap patterns are quite similar and could be merged. For example, MG pattern ID 0 with MGL=6ms and MG pattern ID 13 with MGL=5.5ms, the 0.5ms MGL difference comes from RF switch time for FR1 and FR2. And this is more obvious for NCSG, for example NCSG ID 0 and NCSG ID 13 are the same from the perspective of ML and VIRP. If the same RF tuning time for all FRs is assumed for gap pattern design, the gap patterns could also be reduced. The gap applicable rules could be simplified as well since there is no need to distinguish FR.

Reduce NR gap patterns for inter-RAT NR measurement in 6G

Option 1: prioritize NR mandatory gap patterns

Option 2: study FR-agnostic gap patterns assuming 0.5ms RF switch time for all FRs

As for the RF tuning time in 6G, we prefer to reuse the values in 5G. Some companies proposed to optimize RF tuning time, e.g., reduce RF tuning time to several OFDM symbols. But we don’t think the RF and processing hardware will be improved too much compared with 5G. Besides, 6G aims to support new FR3 and a more complicated RF structure may be considered for UE. There is no need to reduce the RF tuning time. If single value for RF tuning time is agreed for all FRs, 0.5ms for NR FR1 can be considered as baseline.

For gap pattern design, prefer to reuse 0.5ms as RF tuning time as baseline.

For 6G measurement, the above NR gap patterns can be applicable as well or some new 6G-specific gap patterns

may be required. The details are dependent on the reference signal or SMTC design in RAN1 and should be postponed after more conclusions are reached. For example, to achieve energy efficiency and power saving gain, one approach is to extend the periodicity of SSB/SMTC in 6G, e.g., to 320ms or even longer. Then, 6G specific gap patterns aligning with the new SSB/SMTC design should be introduced.

Study 6G-specific gap patterns, at least considering new design of reference signals or MTC.

    - GAP applicability

Another aspect is about gap type. Firstly, we think per-UE gap should be considered as baseline for 6G, especially for low-capability UE with single searcher or FFT processing unit. NR discussed other options such as per frequency layer, per SMTC group or per FR, and ended up with per-FR gap as the trade-off between flexibility and complexity. Independent searchers are assumed for per-FR gap and the measurement in one FR will not cause interruption to the other FR. We are open to discuss per-FR or even per-CC, per-CC group gap. But it relies on the UE structure and FR design in 6G, which are not clear so far. It is better to wait for more progress.

Consider per-UE gap as baseline, and open to discuss per-FR, per-CC (group) gap.

    - GAP sharing

When network signals “01”, “10” or “11” with RRC parameter MeasGapSharingScheme [2][16]and the value of X is defined as in table 9.1.2.1-1, and - Kintra = 1 / X * 100, - Kinter = 1 / (100 – X) * 100, When network signals “00” indicating equal splitting gap sharing, X is not applied. The RRC parameter MeasGapSharingScheme shall be applied to the calculation of carrier specific scaling factor as specified in clause 9.1.5.2.1. - Table 9.1.2.1-1: Value of parameter X for EN-DC measurement gap sharing measGapSharingScheme; Value of X (%). ‘00’; Equal splitting. ‘01’; 25. ‘10’; 50. ‘11’; 75. NOTE: It is left to UE implementation to determine which measurement gap sharing scheme in the table to be applied, when MeasGapSharingScheme is absent and there is no stored value in the field..

When multiple MOs are expected to be measured in a gap, gap sharing is considered. As shown above, an RRC parameter MeasGapSharingScheme is used to configure the gap sharing ratio between intra-frequency and inter-frequency carriers, where inter-frequency carriers include inter-frequency and inter-RAT in NR R15. The reason is that intra-frequency mobility is the most typical scenario and should be separately considered. In the later releases, more types of measurements like PRS and L1-RSRP measurements are further supported to be measured within gap.

However, the gap sharing scheme is not updated accordingly and we only consider these new measurement type as inter-frequency carriers. In 6G, more measurement types should be considered for gap sharing scheme. One example is shown as Table 2, where the sharing ratios for intra-frequency, inter-frequency, inter-RAT NR and L1-RSRP inter-frequency measurement could be explicitly configured. Moreover, the last column is left as others for the potential new measurement types in the later releases, like PRS measurement or sensing, and some code-points could be reserved for them.

An example of gap sharing scheme

| Gap sharing | Intra-frequency | Inter-frequency | Inter-RAT NR | L1-RSRP inter-frequency measurement | Others (e.g., PRS, sensing) |
| --- | --- | --- | --- | --- | --- |
| ‘000’ | Equal splitting |  |  |  |  |
| ‘001’ | 25 | 25 | 25 | 25 | 0 |
| ‘010’ | 50 | 20 | 20 | 10 | 0 |
| ‘011’ | 75 | 10 | 10 | 5 | 0 |
| ‘100’ ~ ‘111’ | Reserved |  |  |  |  |

For GAP sharing scheme, consider more measurement types including all measurement categories which needs GAPs.

Interruption

Issue 4-2: Interruption related scope [Way forward] - FFS followings in RAN4#117 (other sub-topics are not precluded) - Proposal 1(QC): - RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR. - RAN4 should study all existing interruption scenarios in 5G NR whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR. - RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission. - Proposal 2(ZTE): - Finer granularity of interruption such as symbol level is foreseen in 6G to benefit the throughput via avoid the vital symbols within a slot used for DCI or UCI. Awareness of the location of interruption by NW could lessen the impact on throughput. - Proposal 3(Ericsson): - RAN4 shall strive for reducing the amount of interruptions in UEs. - The interruptions can be specified at a symbol level. - The existing interruption requirements may need to be revisited for 6G, if the 6G UEs can achieve a better performance. - Proposal 4(Nokia): - RAN4 to aim at removing all UE autonomous interruptions in 6G. - Proposal 5(MTK): - RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures

    - General for interruption

- Based on the proposals in last meeting, we summarize the issues related to interruption, for both gap-less with interruption and other interruption scenarios.

RAN4 to study the following issues for interruption requirements for 6G

Issue#1: Whether the existing interruption scenarios in 5G NR are still applicable to 6G

Issue#2: Whether it’s possible to remove autonomous interruptions in 6G

Issue#3: Whether and how to reduce the interruption length

Issue#1: Whether the existing interruption scenarios in 5G NR are still applicable to 6GR

RAN4 should study all existing interruption scenarios in 5G NR whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR.

The followings are interruption scenarios identified in 5G, and specified in current TS38.133. In our view, RF retuning and baseband parameter adjustment are the major reasons to cause interruption from UE side. The existing interruption scenarios in 5G NR can be still applicable to 6G.

- Gap-less Measurement with interruption, including measurement on deactivated PScell/SCell

- Carrier aggregation operation, e.g., SCell/SCG activation

- Carrier switching, e.g., UL TX/DL Rx switching, SRS carrier switching

- Antenna switching, e.g., MIMO layer adaptation

- Physical layer parameter adjustment, e.g., BWP/UCBW/SCS/MIMO layer

- Other RRC parameter adjustment, e.g., DRX/non-DRX transition

The existing interruption scenarios in 5G NR can be still applicable to 6G.

Gap-less Measurement with interruption, including deactivated PScell/SCell measurement

Carrier aggregation operation, e.g., SCell/SCG activation

Carrier switching, e.g., UL TX/DL Rx switching, SRS carrier switching

Antenna switching, e.g., SRS antenna switching

Physical layer parameter adjustment, e.g., BWP/UCBW/SCS/MIMO layer adaptation

Other RRC parameter adjustment, e.g., DRX/non-DRX transition

For the new scenarios of interruption in 6G, it will depend on outcome from RAN1 or RAN2, like new spectrum aggregation, new scenarios of carrier/cell switch, or specific RRC parameter adaptation. On the other hand, it also relies on UE architecture of 6G, and RAN4 need to study whether it is feasible to speed UP or remove the RF tuning time and baseband adjustment like for the parameters including SCS, UE BW, MIMO layers and etc. In our view, it seems not possible to remove, and interruption cannot be avoided.

Study new scenarios in 6G which need interruption based on RAN1/RAN2/RF assumptions, e.g., for new spectrum aggregation, new scenarios of carrier/cell switch, or specific RRC parameter adaptation.

Issue#2: Whether it’s possible to remove autonomous interruptions in 6G

Interruption used to be caused by some UE behavior and effect on victim carriers, e.g., inter-frequency measurement, UL carrier switching, DRX to non-DRX transition which mostly causes UE RF/BB retuning.

We have already tried some solutions to restrict interruption location in time domain or frequency domain. For instance, NCSG configured with visible interruption length of each occasion, are introduced in R17, assuming UE have unused RF chains to do measurements on non-serving carrier frequency. In this case the length and time location are explicitly specified.

But it cannot be applied to all cases, because UE does not always have unused or spare chains or processing units. It is dependent on RF architecture, and UE capability report like NCSG or NFI should be supported.

In 6G, this situation cannot change much. Autonomous interruptions cannot be removed from all scenarios. RAN4 can study whether the configured interruption like periodic NCSG for measurement without gap can be extended to which scenarios.

Autonomous interruptions cannot be removed from all scenarios. RAN4 can study the feasibility of extending NCSG like periodic interruption to which scenarios.

Issue#3: Whether and how to reduce the interruption length

RAN4 should firstly study whether is it possible to reduce the interruption length including RF retuning time for different UE procedures, e.g., from slot level (1 ms/0.5 ms) to OFDM symbol level (about tens us). It will depend on UE/BB architecture of 6G. In our view, we don’t think the RF and processing hardware will be improved too much compared with 5G. Besides, 6G aims to support new FR3 and a more complicated RF structure may be considered for UE. There is no need to reduce the RF tuning time. If single value for RF tuning time is agreed for all FRs, 0.5ms for NR FR1 can be considered as baseline.

From UE vendor perspective, the feasibility and necessity of reducing interruption due to RF tuning and baseband adjustment need to be clarified firstly.

Prefer to reuse the RF tuning time from 5G, considering more complicated 6G RF/BB architecture.

    - Gap-less with interruption and other interruption cases

On the other hand, RAN4 can study for which painful scenarios reducing interruption time can be possible. We provide our views on gap-less with interruption and other interruption cases as below.

- Gap-less with interruption

As for how to define the interruption for gap-less measurement, 5G NR specified two solutions, NCSG and ‘no-gap-with-interruption’, which require the same UE capability but different RRM requirements.

The network can configure NCSG to control and reduce the proportion and duration of interruptions. The interruption location and length for NCSG are specified as VIL, which is known to both UE and network. It applies for the periodic measurement without gap but UE needs to frequently and periodic retuning the RF chain or baseband processing units during VIL.

For example, UE also needs to prepare the baseband for simultaneous data Tx/Rx on the serving cells and measurement on the target carrier during VIL, which is different from no-gap measurement with interruption.

On the other hand, ‘no-gap-with-interruption’ only defines the interruption ratio, leaving the location up to UE implementation. It applies for the case where RF switching or retuning happed at UE side with some flexibility, and UE can choose the best opportunity of adjust RF/baseband within a duration. In the case of the gapless measurement with uncertain interruption, it avoids signaling complexity from network and allows for potentially more optimized interruption parameters for UE.

Therefore, both periodic interruption (network-controlled) and flexible interruption (UE-autonomous) should be allowed for gap-less measurement in 6G, since the different use cases can still happen.

For gap-less measurement, both periodic interruption (network-controlled) and flexible interruption (UE-autonomous) should be allowed for UE.

- Other interruption scenarios

- In case of handover or cell switch, in R18 LTM was introduced with pre-synchronization before cell switch, and handover interruption was reduced via advanced synchronization.

- In case of SRS Carrier Switching or BWP switching, for UEs with fast RF retuning capability, interruption lengths shorter than 1ms can be defined. In synchronous CA, if the target carrier belongs to the same Timing Advance Group as the source, the interruption can be reduced by one slot compared to the asynchronous case.

- In case of dynamic cell on/off, DRX state transitions, and other RRC parameter reconfiguration, how to avoid or minimize unnecessary and prolonged interruptions through signaling, configuration, and timing management need both network and UE cooperation.

Compared with periodic or aperiodic interruption, aperiodic interruption needs more flexibility from UE side, since UE　may need more effort to complete the RF／BB retuning or adjustment. For such kind of aperiodic behavior, interruption is still expected to be defined with an upper limit of ratio, leaving some flexibility for UE implementation.

As the events causing interruption could be triggered by L1/L2/L3 configuration, different processing delays are expected. And the actual time for UE retuning RF/BB is possible to be rearranged and even reduced to avoid the interruption across the whole procedure, e.g., from the handover command to handover complete. RAN4 can study how to reduce the interruption length case by case based on the procedures once they are defined in RAN1/2.

For aperiodic interruption, it is still expected to be defined with an upper limit of ratio, leaving some flexibility for UE implementation.

RAN4 study how to reduce the interruption length case by case based on the procedures once they are defined in RAN1/2.

RRM framework

Issue 5: RRM framework: Measurement capability/delay/overhead/accuracy Agreement: RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is considered as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

General

6G device types are being discussed and considered in RAN1/2. Like normal UE, Redcap UE or IoT-like devices in 5G, could be also considered in 6G. Besides, the classification of device type in 6G may be different, RRM related parameters like MIMO layers, Tx/Rx antennas, UE channel bandwidth, supported duplex-mode, can be the key factors to differentiate different RRM requirements and corresponding applicability for different device types. RAN4 should study different RRM related factors and requirements for different device types in 6G.

Study different RRM related conditions and requirements for different device types in 6G.

In 5G NR, a lot of UE capabilities related to RRM were introduced for UE, like fast/relaxed measurement, with or without scheduling restriction, interruption free or not, support of different baseband processing time. Also, as mentioned above, device type is being discussed in RAN1/2, and all possible UE categories like UE with different Rx, channel bandwidth, stationary or high-speed, or even IoT-like devices. These should be considered as well when RAN4 defines the minimum RRM requirements.

Therefore, RAN4 should study and agree on the principle to define the minimum requirements for UE, and avoid the diverse and complicated UE capabilities in the specification, which may introduce too many corner cases and duplicated test cases, making our spec quite hard to read. 6G RRM requirement should skip some corner cases and leave such issues up to UE implementation.

RAN4 to consider the minimum UE requirements and avoid too many UE capabilities and corner cases in 6G RRM discussion, leaving more flexibility for implementation.

Besides, the table below summarizes the RRM requirements in TS 38.133 from R15 to R19. RAN4 can follow the RRM requirements framework of 5G for 6G study, and pick the useful features from both the first release (i.e., R15) and subsequent releases (e.g., R16 to R20) as the starting points of 6G RRM requirements.

Table 3 can be taken as starting point for 6G study of RRM in 6G Day 1.

RRM requirements framework

| Feature | R15 (Baseline) | R16/R17/R18/R19/R20 (Enhancement) |
| --- | --- | --- |
| Idle-state Mobility | - DRX based cell (re)selection - Detection, evaluation, and measurement - Paging interruption | - Idle CA/DC measurement, e.g., EMR; - RRM measurement relaxation, e.g., low mobility and not-at-cell edge - Fast CA/DC setup, e.g., eEMR - OD-SIB1 impact - LP-WUS/WUR, e.g., measurement relaxation and offloading |
| Connected-State Mobility | Handover: - SSB based on L3 measurements. - Delay and interruption requirements CA operation: - Single SCell (de)activation - SCell addition/removal | Handover: - L1/L2 triggered mobility, e.g., SSB/CSI-RS based L1-RSRP measurement - CHO, CLTM SCell/PScell operation: - PUSCH SCell activation - Direct SCell activation - Multiple SCell activation - SCG activation - SSB-less SCell - On-demand SSB for SCell - MAC-CE based LTM and SCell activation |
| Timing | Timing & Synchronization - UE Transmission Timing: TA range and accuracy. - Measurement Timing: Alignment of RSRP measurement windows with SSB periods (i.e.,160ms). | - Multi-TAG - Reference Cell for SSB-less cell - One shot large UL timing adjustment |
| Scheduling | - BWP switching - TCI state switching - Interruption | - Carrier Switching - Antenna switching - UCBW change |
| Measurement procedure | L3 Measurements: - Measurement gap (MG) - delay and accuracy for intra-/inter-frequency, inter-RAT L1 Measurements: - RLM/BFD/CBD/L1-SINR | - Gap enhancement, e.g., needforgap, concurrent gap, pre-MG, NCSG, MUSIM, positioning gap - SCell BFD - Unified TCI - BWP without restriction |
| Note 1: All potential device types which are discussed in RAN1/2 could be included, e.g., normal UE, Redcap UE or IoT-like devices. |  |  |

Measurement delay/overhead/capability

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Measurement delay/overhead - Searcher number for enhanced simultaneous measurements (OPPO, HW, Samsung, ZTE, vivo, Ericsson) - Rx beam sweeping factor reduction (QC, Ericsson) - Identification/measurement/tracking/reporting delay reduction (QC) - RRM with NW aided measurement priority (Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - Unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes. (Nokia) - SSB evaluation (Samsung) - Intra and inter-frequency definition

For Measurement framework, we think the basic structure of 5G RRM requirements will be not changed too much such as the components of cell reselection for Idle mode UE, and cell identification (i.e., cell detection, evaluation, and measurement) for connected mode UE can be kept unchanged.

Basic UE measurement procedure for 5G Idle and connected mode

The basic structure of measurement procedure for idle and connected mode can be reused.

    - Issue #1: Searcher assumption

In 5G NR, CSSF is introduced as carrier specific scaling factor for measurements among multi-carriers/cells, especially for outside gap. 2 searchers are assumed for basic CSSF in R15, and 3 searchers for measurement outside gap are introduced in R19 for CSSF enhancement.

For example, the current searcher assumption/usage for CSSF for SA mode can be summarized as follows (just focus on SSB and without considering CSI-RS measurements).

- The 1st searcher is dedicated to PCC. The 2nd searcher is shared among the rest CCs.

- The 2nd searcher is dedicated to PSCC for 50% of the time, and for the other 50% of the searcher is shared with the rest of the SCCs.

- Among the SCCs, the SCC where neighbour cell measurement is also required occupies 50% of the time, which the rest 50% is equally shared with the rest of the SCCs (and further extended to inter-frequency and inter-RAT MOs without gaps in later releases).

The searcher assumption can be discussed at the start of 6G, and it is better to consider all possible assumptions of searchers, like the total number of searchers, the usages of different searchers, and applicable measurements types.

RAN4 can study whether 3 searchers introduced in R19 can be considered as baseline for 6G day 1, owing to support of more spectrum and measurement demands for 6G UE, e.g., FR1/2/3 and inter-RAT 2/3/4/5G measurements. And whether the searchers are separated or shared by L1 and L3 measurement can be also studied in RAN4 if L1 and L3 unified measurements were introduced.

RAN4 should learn the lesson of 5G to avoid the complicated tables of CSSF. Comprehensive searcher assumption will simplify CSSF a lot. The solutions like one measurement per band, 3 searchers can be considered to reuse for 6G.

RAN4 should learn the lesson of 5G to avoid the complicated tables of CSSF.

Enhanced CSSF in R19 for NR can be considered for 6G.

Besides the measurements outside gap, RAN4 can also study the feasibility of CSSF enhancement for measurements within gap. In 5G, all MOs which needs measurement gap, and LTM candidate cells are all equally shared according to the measurement types, e.g., intra-frequency or inter-frequency, configured by gap sharing factor. Possible enhancement to speed up the measurement within gap can be studied in RAN4.

Possible enhancement on CSSF within gap can be studied in RAN4.

    - Issue #2: Rx beam sweeping reduction

In 5G, Rx beam sweeping reduction has been specified for FR2 UE supporting HST and multi-Rx reception. We are not sure FR2 can be firstly deployed in 6G together with FR1. Since the solutions or capabilities of Rx beam sweeping reduction can be considered, and optimization on FR2 is not urgent for 6G, we prefer not to spend too much time on new solutions for Rx beam sweeping reduction in 6G phase1.

Consider to reuse the solutions or capabilities of Rx beam sweeping reduction for FR2, and Not to spend too much time on new solutions for Rx beam sweeping reduction in 6G phase1.

    - Issue #3: Virtual RRM UE group

Virtual RRM UE group is proposed to achieve RRM measurement relaxation or offloading between different UEs, which are configured or reported to be a group. It is a new use case for 6G. We are interested in this concept and the benefit behind it. It can provide a new tool for both UE and network to reduce the measurement efforts and signaling overhead. We are supportive to study and evaluate virtual RRM UE group in 6G Day 1.

However, there still exist some issues to be clarified and discussed in RAN4:

  - Q1: What is the impact to network side, like measurement configuration, report configuration?

  - Q2: What network can do to support UE group based RRM measurement?

  - Q3: Whether is it feasible to sharing RRM group related information among UEs? Like UE coordination through 3GPP communication like Sidelink or Relay, or non-3GPP communication like Bluetooth or Wifi between two UEs?

  - Q4: Whether the measurement results from one UE can reflect the measurement quality of another UE? If no, how to resolve this with additional UE implementation?

Study and evaluate virtual RRM UE group in RAN4, at least considering the following issues:

Q1: What is the impact to network side

Q2: What network can do to support UE group based RRM measurement

Q3: Whether it is feasible to sharing RRM group related information among UEs,　considering 3GPP or non-3GPP D2D communication

Q4: Whether the measurement results from one UE can reflect the measurement quality of another UE? If no, how to resolve this with additional UE implementation

    - Issue #4: Measurement capability

FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Measurement capability - measurement capability for number of cells, beams and frequency layers (MTK, OPPO, Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - RF retuning time, baseband processing time for typical use cases - UE reference architecture for 6G spectrums - Baseline assumptions of RRM requirements for different UE device types

To be safe, UE measurement capabilities for 6G need to cover those for 5G, including the minimum number of measured frequency layers, the minimum number of cells, the assumption of searchers, the maximum resources of measured RSs, support of gap, gap-less with interruption and gap-less without interruption, etc.

In addition, the assumption of searcher and measurement engines for measurement (including outside and within gap) can be reconsidered. For example, RAN4 can study whether 3 searchers introduced in R19 can be considered as baseline for 6G day 1, owing to support of more spectrum and measurement demands for 6G UE, e.g., FR1/2/3 and inter-RAT 2/3/4/5G measurements. And whether the searchers are separated or shared by L1 and L3 measurement can be also studied in RAN4 if L1 and L3 unified measurements were introduced.

UE measurement capabilities for 6G are assumed to be able to cover those for 5G, at least including frequency layers, cells, searchers, RSs, gap-less with interruption and gap-less without interruption.

Unified RRM Measurement

Considering different kinds of RRM requirements for different procedures and UE capabilities in 5G, RAN4 needs to investigate a harmonized design for 6G RRM measurement, to enable better UE performance, and optimize the delay and gap/interruption requirements for 6G RRM.

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Unified measurements - United/integrated cross-layers measurement and/or report between L1 and L3 (Samsung, OPPO, CMCC, LGE, Xiaomi) - United/integrated cross-functions measurement and/or report for L1 (e.g., integration of MIMO and LTM, or integration of RLM/BFD/CBD)(OPPO, CMCC, Xiaomi) - RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement. Unification in terms of reusing L3 measurements for L1 before certain point (e.g., before TCI state activation) (Ericsson)

Unified L1 and/or L3 measurement

In last meeting, companies provide interests on unified cross-layers measurement and/or report between L1 and L3 and unified cross-function measurement and/or report within L1.

Issue #1: Unified L1 and L3 measurement for mobility

In 5G, both L1/L2 triggered mobility and L3 trigger handover are introduced for UE. Even they are both for mobility management, a lot of different UE capabilities and RRM measurement requirements are required. For example, RRM requirements of LTM apply with some restrictions, e.g., LTM candidate cells belong to L3 MOs, cell detection by L3 could be the pre-condition for LTM. Candidate cells can include serving cell.

Comparison of L3 mobility and LTM cell switch

To provide flexibility of mobility management, RAN4 can further study a unified solution to support both L1 and L3 measurement. RAN4 can start from cross-layer harmonization for same function firstly, e.g., L1-RSRP based neighbor cell measurement in LTM and L3-RSRP for mobility.

- Identify the need of unified configuration for same function, e.g., RS, MO, MTC, Report

- Evaluate RRM impact of unified cell switch/handover procedure, e.g., triggers/conditions/reports

- Assess the difference of measurement requirements and measurement restriction

RAN4 to study unified L1 and L3 measurement for mobility, at least including:

Identify the need of unified configuration for same function, e.g., RS, MO, MTC, GAP

Evaluate RRM impact of unified cell switch/handover, e.g., triggers/conditions/reports

Assess the difference of measurement requirements and measurement restriction

Issue #2: Unified L1-RSRP measurement for MIMO and LTM

Besides, RAN4 can also investigate and evaluate cross-function harmonization for same layer, e.g., L1-RSRP in MIMO and L1-RSRP in LTM. It may more rely on RAN1/RAN2 design, e.g., procedure, configuration, event, etc. We can study the feasibility and how to apply or extend ICBM and inter-cell multi-TRP to L1 mobility (LTM), which may also rely on RAN1 and RAN2’s design on MIMO operation and Mobility in 6G SI.

From our side, we slightly prefer to consider the cross-layer harmonization for RRM measurement with higher priority, i.e., L1 and L3 based neighbor cell measurement, and further consider the cross-function harmonization for the same layer, i.e., L1-RSRP in MIMO and LTM. For example, RAN4 can focus on cross-layer L1 and L3 unified measurements in 2025 Q4, and further evaluate the RRM impact of cross-function L1 measurement like MIMO and LTM from 2026 Q1 based on initial assumption or conclusion from other WGs.

Study and evaluate cross-function integration for same Layer — L1-RSRP in MIMO and L1-RSRP in LTM

More rely on RAN1/RAN2 design, e.g., procedure, configuration, event, etc.

Study the feasibility and how to apply or extend ICBM or inter-cell multi-TRP to L1 mobility (LTM)

Study the RRM impact and applicability, e.g., SSB outside SMTC, RTD＜CP, SSB_without_restriction

Mobility related RRM

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to start from mobility related RRM solutions with less RAN1/2-dependency. - RAN4 to identify which of the following topics can be starts directly in RAN4: - Latency and/or interruption reduction for mobility through RAN4-defined components (Apple, MTK(interruption only), QC, LGE, ZTE, HW, vivo, Ericsson, Nokia, CMCC, Samsung) - discuss RRM part reduction during mobility, e.g., L1/L3 measurement, beam sweeping and etc. - discuss the scenarios/conditions for such reduction (known, unknown, or other status) - discuss NW controlled and UE initiated L1/L3 measurement report - Solutions for longer SSB periodicity in mobility (MTK, Samsung, Ericsson) - Early RRC decoding, and/or, DL/UL sync, and/or, early T/F tracking for mobility (MTK, Nokia) - Virtual RRM UE group (Apple, ZTE) - Unified measurement and mobility framework (QC) - E.g., based on 5G LTM - End-to-end handover latency target (QC) - RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness. - Unified UE measurement requirements across cell changes (Nokia) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other mobility related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other mobility related RRM topics once there are sufficient conclusions from other WGs.

In LTE, there are only RRC IDLE and RRC CONNECTED mode. In 5G NR, a new RRC_INACTIVE has been introduced on top of RRC_IDLE and RRC_CONNECTED mode. RAN4 RRM specified corresponding RRM requirements to support all RRC states.

For 6GR, whether to reuse all RRC states from 5G or not, especially RRC_INACTIVE, is still under discussion. All RAN WGs should cooperate on mobility for 6GR (for all RRC states), including related RRM. It is a bit early for RAN4 to start the discussion on RRM requirements, e.g., IDLE cell selection and reselection, CONNECTED handover delay and interruption, before the initial framework and procedure of Mobility are decided.

Whether to reuse all RRC states from 5G, RAN4 needs to wait RAN2 to decide the framework of Mobility.

We agree that RAN4 can start from some RRM aspects of mobility with less RAN1/2-dependency. We have interest on the following topics, which can bring significant gain from both UE and network side.

- Handover Interruption reduction for mobility through RAN4-defined components

- RRM impact for longer SSB periodicity in mobility, e.g., timing and cell search

RAN4 can start to study Mobility related RRM　aspects with less RAN1/2-dependency, like

Handover Interruption reduction for mobility through RAN4-defined components

RRM impact for longer SSB periodicity in mobility, e.g., timing and cell search

In 6G, it also aims to support migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR. From RRM perspective, RAN4 should consider whether/how to support the RRM for mobility between 5GNR and 6GGR.

RAN4 to study mobility related RRM for mobility between 5GNR and 6GGR.

Others

RRM related energy efficiency

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify candidate topics can be studied in RAN4 directly: - Network energy saving: - RRM for new SSB design(e.g., SSB periodicity extension, OD-SSB/OD-SIB1) (vivo, Ericsson, Nokia, ZTE) - SSB-less based RRM (Ericsson, Nokia, ZTE) - UE power saving: - UE type/state based RRM relaxation (Apple, CMCC, Ericsson(scalable set of measurement requirement), Nokia) - LR based solutions for UE power saving (Sony, vivo, Ericsson) - DRX/eDRX based measurement (Ericsson) - Energy sensing based RRM strategy (CATT) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other energy efficiency related RRM topics if there are sufficient conclusions from other WGs - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other energy efficiency related RRM topics once there are sufficient conclusions from other WGs.

From R15 to R19, RRM requirements are continuously iterated to support RAN1/2 led new features, including UE power saving and Network Energy Saving (NES).

Evolution of UE and network energy saving

The discussion of 6G physical layer signal/channel/procedure, coverage and energy saving schemes have been started in RAN1 from August meeting. RAN1 is also considering unified schemes of energy saving from both network and UE side.

Regarding the observations, it is worthy to consider a harmonized design of energy saving from 6G RRM perspective as well. We support to consider UE and network energy saving together, at least the following aspects coming from 5G can be reconsidered, e.g.,

- RRM measurement relaxation based on criteria

- On-demand measurement, extending to PCell or neighbour cell

- DRX based measurement

- LP-WUR based measurement relaxation/offloading,

- Measurement for low-capability device types (like Redcap UE, IoT-like device in 5G)

and other solutions are not precluded depending on RAN1/2 conclusion

Study a harmonized design of UE and network energy saving from 6G RRM perspective. Consider to leverage the solutions from 5G as follows, and other solutions are not precluded depending on RAN1/2 conclusion.

RRM measurement relaxation based on criteria

On-demand measurement, extending to PCell or neighbour cell

DRX based measurement

LP-WUR based measurement relaxation/offloading,

Measurement for low-capability device types

Spectrum aggregation related RRM

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - RAN4 to identify which of the following topics can be starts directly in RAN4 RRM with less RAN1/2 and RAN4 RF session dependency: - SCell activation/deactivation, deactivated SCell measurement, fast carrier setup based on 6G UE implementations (MTK(SCell activation), QC, vivo(activation), Ericsson, Nokia) - RRM conditions and requirements for Single Cell Multi-Carriers (MTK, vivo, Samsung, OPPO) - RRM impacts of DL and UL decoupling (Samsung, Ericsson) - Carrier switch enhancements for UL and DL (MTK) - RRM impacts of realistic SCS for spectrum (Samsung) - relaxation of the requirement on timing alignment between carriers (Ericsson) - RRM for MRSS (Samsung) - To be discussed: - Option 1: RAN4 to set check points to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics if there are sufficient conclusions from other WGs and RAN4 RF session - FFS: Check point timeline - Option 2: RAN4 to check whether or not starting discussion on other spectrum aggregation and CA related RRM topics once there are sufficient conclusions from other WGs and RAN4 RF session

Evolution of CA/DC enhancement

In 5G, CA/DC related evolution are continued from R15 to R20, and related RRM impact are also defined, such as support of different kinds of SCell activation, SCG activation, and IDLE CA/DC measurement including EMR/eEMR.

In practice, RAN1 figured out the reference signals and spectrum aggregation structure, and RAN2 designed the related reconfiguration and procedures, then RAN4 would define and test the RRM requirements related to cell/carrier/TRP operation. However, we think, in 6G, RAN4 can involve to the design earlier, if RAN4 can see the benefit or feasibility issues of new spectrum aggregation structure.

From RRM perspective, single cell with multi-carrier (SCMC) can bring some benefit by reducing the SSB transmission and related cell search and measurements on some of the carriers. Both Network and UE can achieve the power saving gain. We are supportive to study RRM conditions and requirements for Single Cell Multi-Carriers compared to CA.

Illustration of measurement and report in SCMC operation

RAN4 to study RRM impact for Single Cell Multi-Carriers (SCMC).

MIMO and mTRP operation related RRM

- FFS followings in the RAN4#117 meeting (other sub-topics are not precluded): - Proposal 1(Apple): - study RRM impact from mTRP on different carriers, if supported. - Proposal 2(Samsung): - In 6GR, for MIMO operation and multi-TRP, RAN4 RRM to discussion on following aspects: - Multiple Tx and Multiple Rx with/without simultaneously in transmission/reception - TCI states: RRM only support unified TCI states framework - Further harmonized “TR point” and “cell” for mobility and measurement - Proposal 3(vivo): - MIMO/mTRP related RRM requirements (e.g., measurement for beam management, TCI state switching) need be studied based on concrete RAN1 assumptions and progress, and early RAN4 evolvement is needed w.r.t UE implementation constraint (e.g., UE multi-panel, Rx/Tx timing difference). - Proposal 4(Nokia): - RAN4 to study how to enable robust multi TRP operation considering both beam pairs and deployments

In LTE and 5GNR, RAN4 specified many RRM requirements for MIMO and multi-TRP including TCI-States related; Beamforming measurement related including L1-RSRP; L1-SINR; BFD/CBD; ICBM; Single-DCI; Multi-DCI; asymmetric TRPs; etc.

We are not sure which aspects of MIMO will be included in 6G phase 1, and RAN4 should wait for more conclusion from RAN1.

Wait for more input from RAN1 on MIMO and mTRP operation.

multi-TRP operation is introduced to improve the performance of cell-edge. But it is mainly used to for serving cell beam management. Whether it can be extended for cell switch, is still a question. One reason is that multi-TRP is not widely deployment in real field, and another is TRP based measurement report may not be able to reflect the quality of one cell. Based on these, the feasibility and necessity need to be further evaluated.

The feasibility and necessity need to be firstly clarified for multi-TRP based mobility.

NTN related RRM

FFS followings in the RAN4#117 meeting (other sub-topics are not precluded) - Proposal 1(Samsung, LGE): - RAN4 to discuss for harmonized 6G Radio design for TN and NTN, can start from these aspects: - Re-consider which procedures can be common for TN and NTN. - Integrated TN-NTN mobility support. - Re-consider the framework and mechanisms for the different NTN UE types. - Proposal 2(CMCC): - Within the harmonized 6G Radio design for TN and NTN, if both TN measurement and NTN measurement are configured to UE, the measurement priority shall be under network control, the measurement on TN carrier shall have higher priority as the default assumption. - Towards 6G, the baseline UE measurement capability shall be reconsidered, at least the UE capability of parallelSMTC-r17, parallelMeasurementGap-r17, parallelMeasurementWithoutRestriction-r17 need to be inherited as mandatory to 6G. - Proposal 3(Ericsson): - RAN4 shall ensure that NTN RRM requirements remain aligned with the TN RRM framework in 6G, while incorporating updates to address NTN-specific aspects (which have already been introduced in earlier releases or may be introduced in 6G). - RAN4 shall study valid and effective NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation.(LGE) - Proposal 4(Nokia): - NTN should be considered as a day-1 feature in NR and not be relegated to separate subclauses. This includes the measurement framework. - RAN4 to study how to simplify the measurement requirements for NTN. RAN4 to inform RAN1/2 about the conclusions of the study. - Proposal 5(CATT): - For Space-Air-Ground Integrated Network, RAN4 to study the optimization of RLM and access performance with frequent and significant changes in propagation delay, which may involve multi-dimensional optimization at least including frequency layer and spatial layer.

3GPP can consider harmonized 6G Radio design for TN and NTN, but before physical design, mobility procedures and framework are clearly discussed in RAN1/2, it is a bit early to discuss 6G NTN related RRM in RAN4 without enough and clear assumption.

Wait for more progress and input from RAN1/2 on NTN.

Initial access related RRM

FFS followings in the RAN4#117 meeting (other sub-topics are not precluded) - Proposal 1(Samsung): - In 6GR, for initial access, RAN4 RRM to discussion on following aspects: - Whether to specify the RRM requirements for initial cell search. To consider on following aspects: - Whether can find the start point to define such RRM requirement like “power on” - Necessity to specify such RRM requirements if “UE is powered on” happened infrequently. - Part of UE performance in initial cell search can be ensured by other procedures like cell identification; sync raster - RRM requirements are needed for cell selection - Whether to specify the RACH RRM requirements as functionality as correct UE behavior and tests in RRM. - Proposal 2(vivo): - The cell detection should be studied based on 6G SSB design. In addition, RAN4 may discuss whether necessary to introduce cell selection delay requirement whereas the same principle as that of 4G/5G, i.e., no corresponding cell selection delay requirement, can be used as the base.

RAN4 discuss whether it’s necessary to introduce any RRM requirement for initial access or cell selection. In 5G, no such kind of requirements are defined in RAN4. We think for 6G the situation cannot be changed. The issues like how to find the start point to define such RRM requirement like “power on”, how to setup and repeat the test of initial access are hard to be solved. Thus, we prefer RAN4 to skip the discussion of initial access related RRM.

RAN4 to skip the discussion of initial access related RRM.

Other PHY signal/channel/procedure related RRM

FFS followings in the RAN4#117 meeting (other sub-topics are not precluded) - UE Tx timing: - Proposal 1(MTK): - Study PRACH (if introduced) specific timing accuracy requirement. - Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command. - Proposal 2(Nokia): - In 6G, the UE transmit timing requirements would have to address practical aspects from deployments and define UE requirements for these. - Inter-RAT sync: - Proposal 1(MTK): - Synchronized Inter-RAT to be the baseline assumption for 6G to enable inter-RAT measurement and mobility. - Testability: - Proposal 1(QC): - RAN4 should study whether to use TDL, instead of AWGN, in more performance tests of 6G RRM. - RRM-specific Categories: - Proposal 1(QC): - Study how to establish a more intuitive and practical framework for differentiating UE RRM performance beyond baseline RRM requirements and individual capabilities: - The framework aims to reflect real UE implementation-based RRM performance, while RRM requirements continue to serve as the baseline for minimum performance. - The framework aims to cover a limited set of scenarios and cases with field/deployment relevance, within those defined in the RRM requirement specification. - The framework aims to focus on the most important RRM KPIs, such as measurement latency, mobility latency, carrier aggregation latency, interruption, etc. - The framework aims to accommodate different UE implementations across vendors without requiring disclosure of implementation details, unless such details must be known to the network. - The framework aims to minimize impacts on existing 3GPP specifications. - The framework aims to provide relationships to UE capabilities, where applicable. - The framework aims to be maintainable and verifiable with minimal effort. - TCI switching reduction: - Proposal 1(QC): - RAN4 should study the mechanisms to reduce TCI state switch timeline in 6G. - Proposal 2(Nokia): - RAN4 to study the steps involved in TCI switching and identify potential reductions in TCI switching delay for 6G. - Duplexing: - Proposal 1(Samsung): - In 6GR, RAN4 RRM shall collaborate with RAN1 to discuss on how to support multiple types of duplexing including SBFD. - RRM relaxation and simplification for 6G massive IoT: - Proposal 1(Sony): - RAN4 should study the RRM relaxation and simplification for 6G massive IoT, comparing it with legacy IoT devices, to reduce device complexity and improve network/device energy efficiency. - Sensor based RRM: - Proposal 1(CATT): - RAN4 to consider enriching RRM measurement content and introducing sensing information based mobility management. - In addition to link quality measurement such as RSRP/RSRQ/SINR, the measurement content can also include speed, distance, angle, positioning, imaging, or activity detection, etc. - RAN4 to consider RRM impact for multi-functional RAN, where communication and sensing functionalities are jointly supported. - user-centric based RRM: - Proposal 1(CATT): - RAN4 to study the RRM impact for user-centric operation. - Unified UE capability: - Proposal 1(Xiaomi): - Unified modular based UE RRM Capability Definitions – Study common, reusable capability blocks for fundamental UE abilities (e.g., beam-sweeping factor, message-processing footprint) to improve consistency and scalability across RRM procedures. - BWP switch: - Proposal 1(vivo): - For BWP switch, reduction on BWP switch time may need study once BWP design (if there is any) in 6G is clear. Particularly, the reduction on the duration for UE parsing time may be studied if the BWP framework is further simplified. At the same time, RAN4 could study whether the RF retuning time can be further improved or not. - MRTD: - Proposal 1(Ericsson): - When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system. - When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between ARP. - TDD Cell Phase Synchronization: - Proposal 1(Ericsson): - Keep TDD Cell Phase Synchronization requirement the same as in NR NR. - CGI reading: - Proposal 1(Ericsson): - RAN4 should define the CGI reading requirement in 6G first release. - Purpose-based measurement requirements: - Proposal 1(Nokia): - For Connected mode, Idle mode, and Inactive mode, RAN4 to study defining measurement requirements depending on purpose of the configured measurement: mobility or data (CA). - FFS: - Due to the limited TU of the 6G SI for RRM, the topics in “Issue 13: Other PHY signal/channel/procedure related RRM” will not be studied in 6G SI timeline, unless it can be well justified with following criteria in RAN4#117 meeting: - Topics that can be initiated directly in RAN4 - Topics with clear commercial demand for RRM - Topics for fundamental feature in RRM (not incremental enhancement from 5G) - Topics with the strongest support from companies - Topics whose study can address the most critical pain points in 5G RRM - Topics requested from other WGs

We have interest on RRM-specific Categories. The intention is to specify the RRM specification and keep some flexibility of UE. We agree that RRM requirements should continue to serve as the baseline for minimum performance, and 6G RRM should aim to accommodate different UE implementations across vendors without requiring disclosure of implementation details, unless such details must be known to the network.

However, we don’t think for it is easy for RAN4 to define different categories of RRM requirements. And RAN-P is also discussing the device type. It is not clear that what the relationship between device types and RRM-specific categories. Is one device type of UE can only have one category of RRM requirements, or more than one categories? But if too many categories are defined, it seems similar to different combination of UE capabilities.

More clarifications are needed on the principle of RRM specific categories, and the relationship between device types and RRM-specific categories.

More clarifications are needed on the principle of RRM specific categories, and the relationship between device types and RRM-specific categories.

Conclusion

We provide the following observations and proposals.

2. Overview of 6G RRM

- The RRM impact of RAN1/2 new design for 6G needs to be studied as early as possible in RAN4, especially physical layer signal/procedure, coverage and energy saving schemes.

- Suggest RAN4 to firstly focus on the following topics with more supporting companies and less dependency on other WGs.

Measurement GAP and interruption

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement

RAN4 to investigate a harmonized design for 6G RRM, focusing on the most important RRM KPIs, such as measurement delay and accuracy, mobility latency, carrier aggregation latency, interruption, etc

Study RRM impact of 6G new spectrum based on new UE RF/baseband assumption.

3.1.1 Unified GAP and adaptive operation

To enable more efficient and flexible measurement, unified GAP needs to be studied to cover different types of gaps, including pre-configured gap, parallel configured gap, and small gaps.

Study the following aspects for unified GAP design:

Unified GAP configuration and efficient GAP change or gap adaptation

Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

Unified solution for GAP cancelling schemes

Unified solution for GAP collision

3.1.2. GAP pattern reduction

The gap patterns in 6G should match with target measurement purposes, including both 5G NR measurement and 6G measurement.

Reduce NR gap patterns for inter-RAT NR measurement in 6G

Option 1: prioritize NR mandatory gap patterns

Option 2: study FR-agnostic gap patterns assuming 0.5ms RF switch time for all FRs

For gap pattern design, prefer to reuse 0.5ms as RF tuning time as baseline.

Study 6G-specific gap patterns, at least considering new design of reference signals or MTC.

3.1.3 GAP applicability

Consider per-UE gap as baseline, and open to discuss per-FR, per-CC (group) gap.

3.1.4 GAP sharing

For GAP sharing scheme, consider more measurement types including all measurement categories which needs GAPs.

3.2.1 General for interruption

RAN4 to study the following issues for interruption requirements for 6G

Issue#1: Whether the existing interruption scenarios in 5G NR are still applicable to 6G

Issue#2: Whether it’s possible to remove autonomous interruptions in 6G

Issue#3: Whether and how to reduce the interruption length

The existing interruption scenarios in 5G NR can be still applicable to 6G.

Gap-less Measurement with interruption, including deactivated PScell/SCell measurement

Carrier aggregation operation, e.g., SCell/SCG activation

Carrier switching, e.g., UL TX/DL Rx switching, SRS carrier switching

Antenna switching, e.g., SRS antenna switching

Physical layer parameter adjustment, e.g., BWP/UCBW/SCS/MIMO layer adaptation

Other RRC parameter adjustment, e.g., DRX/non-DRX transition

Study new scenarios in 6G which need interruption based on RAN1/RAN2/RF assumptions, e.g., for new spectrum aggregation, new scenarios of carrier/cell switch, or specific RRC parameter adaptation.

Autonomous interruptions cannot be removed from all scenarios. RAN4 can study the feasibility of extending NCSG like periodic interruption to which scenarios.

From UE vendor perspective, the feasibility and necessity of reducing interruption due to RF tuning and baseband adjustment need to be clarified firstly.

Prefer to reuse the RF tuning time from 5G, considering more complicated 6G RF/BB architecture.

3.2.2 Gap-less with interruption and other interruption cases

For gap-less measurement, both periodic interruption (network-controlled) and flexible interruption (UE-autonomous) should be allowed for UE

For aperiodic interruption, it is still expected to be defined with an upper limit of ratio, leaving some flexibility for UE implementation.

RAN4 study how to reduce the interruption length case by case based on the procedures once they are defined in RAN1/2.

4. RRM framework

Study different RRM related conditions and requirements for different device types in 6G.

RAN4 to consider the minimum UE requirements and avoid too many UE capabilities and corner cases in 6G RRM discussion, leaving more flexibility for implementation.

Table 3 can be taken as starting point for 6G study of RRM in 6G Day 1.

- RRM requirements framework

| Feature | R15 (Baseline) | R16/R17/R18/R19/R20 (Enhancement) |
| --- | --- | --- |
| Idle-state Mobility | - DRX based cell (re)selection - Detection, evaluation, and measurement - Paging interruption | - Idle CA/DC measurement, e.g., EMR; - RRM measurement relaxation, e.g., low mobility and not-at-cell edge - Fast CA/DC setup, e.g., eEMR - OD-SIB1 impact - LP-WUS/WUR, e.g., measurement relaxation and offloading |
| Connected-State Mobility | Handover: - SSB based on L3 measurements. - Delay and interruption requirements CA operation: - Single SCell (de)activation - SCell addition/removal | Handover: - L1/L2 triggered mobility, e.g., SSB/CSI-RS based L1-RSRP measurement - CHO, CLTM SCell/PScell operation: - PUSCH SCell activation - Direct SCell activation - Multiple SCell activation - SCG activation - SSB-less SCell - On-demand SSB for SCell - MAC-CE based LTM and SCell activation |
| Timing | Timing & Synchronization - UE Transmission Timing: TA range and accuracy. - Measurement Timing: Alignment of RSRP measurement windows with SSB periods (i.e.,160ms). | - Multi-TAG - Reference Cell for SSB-less cell - One shot large UL timing adjustment |
| Scheduling | - BWP switching - TCI state switching - Interruption | - Carrier Switching - Antenna switching - UCBW change |
| Measurement procedure | L3 Measurements: - Measurement gap (MG) - delay and accuracy for intra-/inter-frequency, inter-RAT L1 Measurements: - RLM/BFD/CBD/L1-SINR | - Gap enhancement, e.g., needforgap, concurrent gap, pre-MG, NCSG, MUSIM, positioning gap - SCell BFD - Unified TCI - BWP without restriction |
| Note 1: All potential device types which are discussed in RAN1/2 could be included, e.g., normal UE, Redcap UE or IoT-like devices. |  |  |

4.2 Measurement delay/overhead/capability

The basic structure of measurement procedure for idle and connected mode can be reused.

Issue #1: Searcher assumption

RAN4 should learn the lesson of 5G to avoid the complicated tables of CSSF.

Enhanced CSSF in R19 for NR can be considered for 6G.

Possible enhancement on CSSF within gap can be studied in RAN4.

Issue #2: Rx beam sweeping reduction

Consider to reuse the solutions or capabilities of Rx beam sweeping reduction for FR2, and Not to spend too much time on new solutions for Rx beam sweeping reduction in 6G phase1.

Issue #3: Virtual RRM UE group

Study and evaluate virtual RRM UE group in RAN4, at least considering the following issues:

Q1: What is the impact to network side

Q2: What network can do to support UE group based RRM measurement

Q3: Whether it is feasible to sharing RRM group related information among UEs,　considering 3GPP or non-3GPP D2D communication

Q4: Whether the measurement results from one UE can reflect the measurement quality of another UE? If no, how to resolve this with additional UE implementation

Issue #4: Measurement capability

UE measurement capabilities for 6G are assumed to be able to cover those for 5G, at least including frequency layers, cells, searchers, RSs, gap-less with interruption and gap-less without interruption.

4.3	 Unified RRM Measurement

RAN4 to study unified L1 and L3 measurement for mobility, at least including:

Identify the need of unified configuration for same function, e.g., RS, MO, MTC, GAP

Evaluate RRM impact of unified cell switch/handover, e.g., triggers/conditions/reports

Assess the difference of measurement requirements and measurement restriction

Study and evaluate cross-function integration for same Layer — L1-RSRP in MIMO and L1-RSRP in LTM

More rely on RAN1/RAN2 design, e.g., procedure, configuration, event, etc.

Study the feasibility and how to apply or extend ICBM or inter-cell multi-TRP to L1 mobility (LTM)

Study the RRM impact and applicability, e.g., SSB outside SMTC, RTD＜CP, SSB_without_restriction

5.	Mobility related RRM

Whether to reuse all RRC states from 5G, RAN4 needs to wait RAN2 to decide the framework of Mobility.

RAN4 can start to study Mobility related RRM aspects with less RAN1/2-dependency, like

Handover Interruption reduction for mobility through RAN4-defined components

RRM impact for longer SSB periodicity in mobility, e.g., timing and cell search

RAN4 to study mobility related RRM for mobility between 5GNR and 6GGR.

6.1	RRM related energy efficiency

Study a harmonized design of UE and network energy saving from 6G RRM perspective. Consider to leverage the solutions from 5G as follows, and other solutions are not precluded depending on RAN1/2 conclusion.

RRM measurement relaxation based on criteria

On-demand measurement, extending to PCell or neighbour cell

DRX based measurement

LP-WUR based measurement relaxation/offloading,

Measurement for low-capability device types

6.2	 Spectrum aggregation related RRM

RAN4 to study RRM impact for Single Cell Multi-Carriers (SCMC).

6.3	 MIMO and mTRP operation related RRM

Wait for more input from RAN1 on MIMO and mTRP operation.

The feasibility and necessity need to be firstly clarified for multi-TRP based mobility.

6.4	 NTN related RRM

Wait for more progress and input from RAN1/2 on NTN.

6.5 Initial access related RRM

RAN4 to skip the discussion of initial access related RRM.

6.6	 Other PHY signal/channel/procedure related RRM

More clarifications are needed on the principle of RRM specific categories, and the relationship between device types and RRM-specific categories.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RANP #108

- R4-2514644, WF on 6G RRM