# 3GPP TSG-RAN WG4 Meeting #117	R4-2521333

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.3.2

Source:	OPPO

Title:	Discussion on test case configuration for low band carrier aggregation via switching

Document for:	Approval

Introduction

Based on the timeline of R19 low band CA work item [1], the majority of performance requirements were discussed in previous meetings, and the related CR on test cases are expected to be submitted in this meeting. In this contribution, we will provide our considerations on the test case configuration [2].

Discussion

Issue 1-1-5: Delay requirement in SDL SCell activation [Way forward]: - FFS: - Proposal 1 (LGE): - RAN4 to define SCell activation delay requirements for fractional case where {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, and the principle of Kp scaling factor could be applied to SCell activation delay. - Proposal 2 (Nokia): - Switching pattern is not applied when UE is in DRX. - Remove the impact of switching pattern for Kp factor for measurements during DRX. - If above bullet is agreeable, inform RAN1 about the agreement in an LS - Proposal 3 (Ericsson): - Proposal 5: RAN4 to agree on the following text in the SCell activation delay requirement for the cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern: - For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies: - if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern. - if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained. - if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

The existing test cases in TS 38.133 spec cover both DRX case and non-DRX case, where DRX cycle is configured as 40ms, 80ms or longer. Based on the agreement reached in the last meeting, the switching pattern periodicity is assumed as 40ms. If both DRX and switching pattern are configured in the test case, the impact of switch pattern may not be reflected. Besides, some companies have concerns on applying switching pattern for UE in DRX and propose to remove the impacts of switching pattern in DRX case, as shown above (highlighted in red). Although no consensus on core requirements is reached so far, we prefer to consider only non-DRX case when defining test cases.

Proposal 1: Focus on on-DRX case when defining test cases.

The next issues to discuss is gap configuration in the test case. For RLM OOS test cast defined in clause A.6.5.1.1, Gap Pattern ID #0 (40ms) with 0ms offset is configured, which is overlapped with the SSB configuration (20ms periodicity and 0ms offset). Then, half of SSB occasions are dropped for gap. If the FDD on duration based on switching pattern is aligned with the dropped SSB occasions, no SSB occasion is available for RLM measurements. If the FDD on duration is aligned with the non-dropped SSB occasions, the measurement delay is same as the existing test case and the impacts of switching pattern cannot be verified. Therefore, we propose to not configure gap in the test cases.

Proposal 2: Not configure gap in the test cases.

Conclusions

In this paper, we provided our views on the performance requirements for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: Focus on on-DRX case when defining test cases.

Proposal 2: Not configure gap in the test cases.

Reference

- RP-243317 New WID on low band carrier aggregation via switching

- R4-2514870 WF for [116bis][202] NR_LBCA_Sw_RRM, Apple