# 3GPP TSG-RAN WG4 Meeting #117	R4-2521337

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.8.4

Source:	OPPO

Title:	Discussion on RRM core requirements for R20 LB CA

Document for:	Approval

- Introduction

RANP #109 meeting agreed a new WID in [1] and the objectives are duplicated below. The majority objectives are similar as R19 LB CA and it is encouraged to reuse R19 mechanism and requirements as much as possible. The main enhanced part is to change case 2 (Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1) to case 3 (Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1). Based on the objectives and the agreements reached in last meeting [2], this contribution will provide our considerations on RRM core requirements for R20 LB CA.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

- Discussion

2.1 General issues

Issue 1-1-2: MRTD assumption - Proposal 1 (ZTE Corporation, Sanechips, Samsung): Reuse the MRTD requirement specified in R19 low band CA via switching - Proposal 1-1 (Samsung): The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching - Recommended WF - Tentative agreement: - For MRTD assumptions in Rel-20 LB-CA via switching: Reuse the MRTD requirement specified in Rel-19 LB-CA via switching, i.e., the maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us

The first issue is whether the MRTD assumption (3us) in R19 LB CA can be reused for R20 LB CA. This assumption is important for RRM requirements and should be discussed firstly. For example, when MRTD is too large, the slot boundary of FDD PCell and SDL SCell may not align well and additional interruption / delay may be required. In our view, there is no much difference between R19 and R20, and it is preferred to reuse the same MRTD assumption.

Proposal 1: Reuse the MRTD requirement specified in Rel-19 LB-CA via switching.

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered Agreement: According to the WID, RANA only defines requirements for CA with FDD PCC and one SDL SCC.

Issue 1-1-3: Band combination assumption - Proposal 1 (OPPO, Samsung): RAN4 will only define requirements for CA with FDD PCC and one SDL SCC - Recommended WF - Tentative agreement: - For Rel-20 LB-CA via switching, RAN4 only defines requirements for CA with FDD PCC and one SDL SCC

As shown above, R19 LB CA precluded the scenario when more carriers are configured. We think R20 is the same as R19 and the above agreement should still apply.

Proposal 2: RAN4 will only define requirements for CA with FDD PCC and one SDL SCC.

Issue 1-1-4: SSB assumption - Proposal 1 (Samsung): To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused: SSB based SDL SCell operation - Recommended WF - Tentative agreement: - For SSB assumptions in Rel-20 LB-CA via switching: Reuse the SSB assumptions for Rel-19 LB-CA via switching as baseline, that is SDL SCell is transmitting SSB

Regarding the reference signal for SDL SCell, we agree to consider SSB which is widely used for measurement.

Proposal 3: R20 LB CA can focus on the scenario when SDL SCell is transmitting SSB.

2.2 SDL SCell related RRM requirements

RRM requirement applicability for R19 LB CA via switching (R19) - Agreement in online session: - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Thursday online (R20) Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC (R20) - Recommended WF - Companies to check whether the following proposal can be agreed - In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Comparing R19 LB CA and R20 LB CA, the major difference is that UE could receive data from FDD carrier 1 when SDL carrier 2 is turned ON. From the perspective of SDL SCell, UE behavior is almost the same, i.e. periodically switch to the SDL SCell. In R19 LB CA, the related measurement delay requirements depend on the overlapping scenario between SSB/SMTC/CSI-RS and switching pattern. For example, if all the SSB/MTC/CSI-RS occasions are available based on the switching pattern, no delay extension is allowed and the legacy requirements can apply. If a fraction of SSB/SMTC/CSI-RS is not available due to the overlapping with PCell on duration, the measurement delay is extended in accordance to the fraction of available occasions. Otherwise, if no SSB/MTC/CSI-RS occasion is available, the UE cannot perform measurement and no requirement apply. We this principle could be reused.

Proposal 4: Reuse the following principle for gap-less L3 measurement and L1 measurement of activated SDL SCC

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Interruption requirement design of deactivated SDL SCell measurement (R19) Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell). [Agreement]: - For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length.

Issue 1-4-1: SDL SCell related RRM requirement (R20) - Recommended WF - For SDL SCell related RRM requirements, RAN4 to further discuss the following requirements identified for Rel-20 LB-CA via switching - SDL SCell activation/deactivation - Wait for more progress on activation/deactivation requirement for Rel-19 LB-CA - Interruption due to deactivated SCell measurement - FFS: SSB-less SCell - Other requirements are not precluded - Wait for RF progress on reference architecture for Case 3

One impact of deactivation SDL SCell measurement is about the interruption. As shown above, both interruption length and miss ACK/NACK ratio are defined in R19 LB CA. In R20 LB CA, the interruption on FDD DL and UL are different and should be separately considered. For interruption length on FDD UL, the same methodology could be applied, that is the interruption length = tuning time + TSMTC_duation + retuning time (in slot level), where the tuning/retuning time is updated based on RAN1 conclusion.

Proposal 5: Interruption length on FDD UL due to deactivated SCell measurement = tuning time + TSMTC_duation + retuning time.

For interruption length on FDD DL, at least the SMTC duration should be excluded since UE is capable to Rx on both FDD DL carrier and SDL carrier. During the switching gap, interruption on FDD DL may be needed but we are open to discuss.

Proposal 6: Interruption length on FDD DL due to deactivated SCell measurement = tuning time + retuning time.

Based on proposal 5 and proposal 6, the miss ACK/NACK rate could be calculated as below.

Proposal 7: Missed ACK/NACK rate due to deactivated SDL measurements are allowed with up to X= YUL+YDLmeasCycleSCell, where YUL is the interruption length on FDD UL, and YDL is the interruption length on FDD DL.

- Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms Issue 1-1-3b: switching pattern based carrier switching in SCell activation Agreement: - TLBCA =1ms for MAC CE based, 0 ms for RRC based. Issue 1-1-4a: Application time of disabling switching pattern in SCell deactivation Agreement: - The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation Issue 1-1-4b: switching pattern based carrier switching in SCell activation Agreement: - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

The other issue is the (de)activation timing of switching pattern during SCell (de)activation procedure. As shown above, an additional time margin TLBCA is considered for the UE processing, e.g. to apply switching pattern at the RF level. During last RAN4 meeting, the exact value of TLBCA is determined, which is 1ms for MAC-CE based SCell activation and 0ms for RRC based SCell activation. UE behavior in R20 may be a little different from that in R19. For example, when SCell is in activation procedure, UE needs to turn off FDD Tx and Rx, and then switch to SDL in R19. However, R20 UE needs to turn off FDD Tx, maintain the FDD Rx and turn on the SDL in R20. We slightly prefer to apply the same value of TLBCA for R20 LB CA, but we are not pretty sure and are open to discussion.

Proposal 8: Reuse TLBCA defined in R19 LB CA for R20 LB CA.

2.3 FDD PCell related RRM requirements

Issue 1-4-1: RLM requirement for PCell [Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure. Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching [Agreement]: The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

For FDD PCell measurement including RLM, BFD, CBD, intra-frequency SSB L3 measurement without gap and L1-RSRP/SINR measurement, R19 LB CA extends the measurement delay requirements considering the switching pattern. However, UE in R20 could still maintain FDD Rx even when SDL is turned on. The measurement delay requirements do not need to be extended and the legacy requirements before R19 LB CA can apply.

Proposal 9: For FDD PCell, the legacy delay requirements without considering R19 LB CA extension should apply for R20 LB CA, including:

- Intra-frequency L3 measurement (gap-less only)

- RLM

- BFD/CBD

- L1-RSRP/SINR

- PL-RS switch delay

- TCI state switch delay

- Conclusion

This contribution gave our considerations on RRM requirements for R20 LB CA and the following proposals.

Proposal 1: Reuse the MRTD requirement specified in Rel-19 LB-CA via switching.

Proposal 2: RAN4 will only define requirements for CA with FDD PCC and one SDL SCC.

Proposal 3: R20 LB CA can focus on the scenario when SDL SCell is transmitting SSB.

Proposal 4: Reuse the following principle for gap-less L3 measurement and L1 measurement of activated SDL SCC

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 5: Interruption length on FDD UL due to deactivated SCell measurement = tuning time + TSMTC_duation + retuning time.

Proposal 6: Interruption length on FDD DL due to deactivated SCell measurement = tuning time + retuning time.

Proposal 7: Missed ACK/NACK rate due to deactivated SDL measurements are allowed with up to X= YUL+YDLmeasCycleSCell, where YUL is the interruption length on FDD UL, and YDL is the interruption length on FDD DL.

Proposal 8: Reuse TLBCA defined in R19 LB CA for R20 LB CA.

Proposal 9: For FDD PCell, the legacy delay requirements without considering R19 LB CA extension should apply for R20 LB CA, including:

- Intra-frequency L3 measurement (gap-less only)

- RLM

- BFD/CBD

- L1-RSRP/SINR

- PL-RS switch delay

- TCI state switch delay

References

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Huawei, RANP #109

- R4-2514824, WF for [116bis][224] NR_LBCA_Sw_enh_RRM, Moderator (Samsung), RAN4 #116Bis