3GPP TSG-RAN WG4 Meeting #117	R4-2521355

# Dallas, USA, 17 - 21 Nov, 2025

Title: 	Simulation results on NR NTN Ku band demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	5.10.3

Document for:	Information

Background

During RAN4#116bis meeting, Way Forward [1] for Rel-19 Demodulation_Part2 was approved. In this contribution, we provide our initial simulation results on NR NTN Ku band demodulation performance requirements.

Simulation results

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | [TBD] | 50 / 15 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | N/A |
| 3-2 | [TBD] | 50 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 1.7 |

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| [2-1] | [50] | [48] | [1] | [8] | [TBD] | NTN-TDLC5-1200 | 1x1 | 1 | 15kHz: 3.1 30kHz: 2.9 |

| CCE to REG mapping type | REG bundle size | Interleave size | Shift index | DCI format | Payload (without CRC) |
| --- | --- | --- | --- | --- | --- |
| Interleaved | 2 | 3 | 0 | 1_0 | 42 |

Conclusions

In this contribution, we provide our initial simulation results on NR NTN Ku band demodulation performance requirements.

Reference

R4-2515041, Way Forward for [116bis][312] Rel-19 Demodulation_Part2, RAN4#116bis, Nokia