3GPP TSG-RAN WG4 Meeting #117	R4-2521359

# Dallas, USA, 17 - 21 Nov, 2025

Title: 	Discussion on IoT NTN TDD demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	6.21.3

Document for:	Discussion

Background

During RAN4#116bis meeting, Way Forward [1] for Rel-19 Demodulation_Part2 was approved. In this contribution, we provide our initial simulation results on IoT NTN TDD demodulation performance requirements.

Discussion

General

Repetition number selection

For the repetition number, we propose to select the value that the total TxDuration not larger than 8ms, i.e. not cross TDD pattern scheduling period considering the long delay for one transmission, especially for NPRACH, maybe user experience for access to the network will be impacted.

Select the repetition number value that the total TxDuration not larger than 8ms, i.e. not cross TDD pattern scheduling period.

- NPUSCH format 2: Option 2: 4 with Tx Duration of 2 (Total Duration of 8 ms)

- NPRACH: Option 1

  - Format 0: Option 1: 4 with 2 Tx duration (Total duration 8 ms)

  - Format 1: Option 1: 2 with 4 Tx duration (Total duration 8 ms)

UE demodulation performance requirements

NPDSCH

| Test number | Bandwidth | Carrier Type | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | 2.5 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | 4 | NTN-TDLA100-10 | 1 | 1x1 | 70% | 3.1 | NB1, NB2 |

SAN demodulation performance requirements

NPUSCH format 1

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 1.0 |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 8 Hz | 4 | 4 | 70% | 0.3 |

NPUSCH format 2

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN TDLA100-1 | 1 | 15KHz | 16 Hz | 8 | 4 | 9.1 |

NPRACH

| Number of TX antennas | Number of RX antennas | TxDuration | Repetition number | Propagation conditions and correlation matrix (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | 8 | NTN-TDLA100-1 | 200 Hz | 9.3 | 9.1 |

Proposals

In this contribution, we share our proposal and provide our initial simulation results on IoT NTN TDD demodulation performance requirements.

- Select the repetition number value that the total TxDuration not larger than 8ms, i.e. not cross TDD pattern scheduling period.

- NPUSCH format 2: Option 2: 4 with Tx Duration of 2 (Total Duration of 8 ms)

- NPRACH: Option 1

  - Format 0: Option 1: 4 with 2 Tx duration (Total duration 8 ms)

  - Format 1: Option 1: 2 with 4 Tx duration (Total duration 8 ms)

Reference

R4-2515041, Way Forward for [116bis][312] Rel-19 Demodulation_Part2, RAN4#116bis, Nokia