3GPP TSG-RAN WG4 Meeting #117	                                                                      R4-2521367

Dallas, US, November 17th-21st, 2025

Title: 	Simulation results for 6Rx demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2

Document for:	Discussion

Background

In this contribution we provide our simulation results for 6Rx according to [1].

- UE performance requirements

PDSCH requirements

Simulation assumptions are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = 0.07179 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Rel-15 Type 1 DMRS 2+2: {1000-1003, 1004, 1006} |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | 6% |
| Test metric |  |  | SNR@70% max TP |

Simulation results are captured in Table 2.1-1:

Table 2.1-2: Simulation results for PDSCH

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | MIMO layers | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 40 / 30 | 6, 490/1024 | TDLA30-10 | 4x6, ULA, α = 0 and β = 0.07179 | 4 | 70 | 17.1 |
| 2 | 40 / 30 | 6,438/1024 | TDLA30-10 | 8x6, ULA, low | 6 | 70 | 17.4 |

CQI requirements

Simulation assumptions are captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for CQI requirements

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4x6 with static channel H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 4,(13) |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration (Note 1) |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1, CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00010001 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | 8 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |
| Note 1: The PMI associated to i2 = 0 is always used as the precoder regardless of the reported i2 value. |  |  |  |  |  |  |  |

Simulation results are captured in Table 2.2-2:

Table 2.2-2: Simulation results for CQI requirements under AWGN channel

| SNR(dB) | Median CQI | BLER for Median CQI | BLER for Median CQI+1 | BLER for Median CQI-1 |
| --- | --- | --- | --- | --- |
| 5 | 5 | 0.94 | 1.00 | 0.00 |
| 6 | 5 | 0.02 | 1.00 | 0.00 |
| 7 | 6 | 1.00 | 1.00 | 0.00 |
| 8 | 6 | 0.01 | 1.00 | 0.00 |
| 9 | 7 | 0.99 | 1.00 | 0.00 |
| 10 | 7 | 0.00 | 1.00 | 0.00 |
| 11 | 8 | 1.00 | 1.00 | 0.00 |
| 12 | 8 | 0.00 | 1.00 | 0.00 |

SDR requirements

Simulation results are captured in Figure 2.3-1 and Table 2.3-1

Figure 2.3-1: Simulation assumptions for SDR test

Table 2.3-1: Simulation summary for SDR test

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 | MCS Table 2 | MCS Table 4 |
| 28 | 19.9 |  |  |
| 27 | 18.8 | 26.0 |  |
| 26 | 17.7 | 24.6 |  |
| 25 | 16.7 | 23.7 | 28.3 |
| 24 | 15.7 | 22.7 | 26.1 |
| 23 | 14.7 | 20.8 | 24.5 |
| 22 |  | 19.7 |  |

- Conclusion

In this contribution we provide simulation results for 6Rx.

- Reference

[1] Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx. Huawei, HiSilicon