3GPP TSG-RAN WG4 Meeting #117	R4-2521375

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on non-collocated CA performance for type 4b UE

Source: 	Huawei, HiSilicon

Agenda item:	6.2.3

Document for:	Discussion

Background

A WF [1] on NonCol_intraB_ENDC_NR_CA_Ph2_demod was agreed. In this contribution we provide our views on open issues.

- Discussion

One issue is Rank/MCS:

Way forward: MCS/rank Down-select the MCS for the stronger CC based on the simulation results - For weaker CC: - Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1) - For stronger CC: - Option 1: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2) - Option 2: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2) - Other options are not precluded. Interested companies are encouraged to provide the simulation results (both alignment and impairment results) in the RAN4#117 to decide the rank/MCS.

Our simulation results are captured as following:

Table 2-1: Ideal simulation results

| Bandwidth (MHz) | Modulation format and code rate | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 40 | QPSK.0.30 | 2x4 | 70 | -4.2 |
| 40 | 256QAM 0.69 | 4x4 | 70 | 19.4 |
| 40 | 256QAM 0.74 | 4x4 | 70 | 20.4 |

Table 2-3: Impairment simulation results

| Bandwidth (MHz) | Modulation format and code rate | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 40 | QPSK.0.30 | 2x4 | 70 | -2.2 |
| 40 | 256QAM 0.69 | 4x4 | 70 | 21.4 |
| 40 | 256QAM 0.74 | 4x4 | 70 | 22.4 |

Considering the criteria is that selecting the MCS which make the SNR difference between two CCs close to but smaller than 25dB, i.e. ≤25dB, we propose MCS22

Proposal 1: RAN4 to consider MCS 22 for stronger CC.

One issue is antenna connection:

Way forward: Antenna connection Discuss whether to capture the antenna connection for Type 4b UE test in TS 38.101-4 5.1.1. - Example: - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration.

The architecture of Type4b UE could be described as that Type 4b UE is equipped with 8 antennas, 4 of which are connected to one CC and the other 4 antennas are connected to another CC. Similar to antenna connection rules defined for inter band CA requirements (see 5.1.1.7.6), it should also be up to UE’s declaration and AP configuration which 4 out of 8Rx should connected to same CC for this non-collocated intra band CA test. Here are antenna connection rules for inter band CA requirements with 8Rx copied defined in the spec:

##### 5.1.1.7.6 Applicability rule and antenna connection for CA tests with 8 RX Within the CA configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, 4 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.3.1 are applied. Within the CA configuration if any of the PCell and/or the SCells is a 2Rx supported RF band, 2 out of the 8Rx should be connected with data source from system simulator, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied. Within the CA configuration if any of the PCell and/or the SCells is a 8Rx supported RF band, all 8Rx should be connected with data source from system simulator. Requirements from Clause5.2A.4.1 are applied. For 8Rx capable UEs, the 2Rx supported RF bands, 4Rx supported RF bands and 8Rx supported RF bands are up to UE’s declaration.

The proposed applicability rule for antenna connection for this test is that:

- For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4b UE in Clause xxxx, 4 out of 8Rx should be connected with one data source from system simulator and the other 4 out of 8Rx should be connected with another data source from system simulator, depending on UE’s declaration and AP configuration.

Proposal 2: Define the applicability rule for antenna connection as following:

- For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4b UE in Clause xxxx, 4 out of 8Rx should be connected with one data source from system simulator and the other 4 out of 8Rx should be connected with another data source from system simulator, depending on UE’s declaration and AP configuration.

- Conclusion

In this contribution we provide our views on open issues for non-collocated CA performance for type 4b UE. The proposals are:

Proposal 1: RAN4 to consider MCS 22 for stronger CC.

Proposal 2: Define the applicability rule for antenna connection as following:

- For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4b UE in Clause xxxx, 4 out of 8Rx should be connected with one data source from system simulator and the other 4 out of 8Rx should be connected with another data source from system simulator, depending on UE’s declaration and AP configuration.

- Reference

[1] WF on NonCol_intraB_ENDC_NR_CA_Ph2_demod. Ericsson