3GPP TSG-RAN WG4 Meeting #117	R4-2512377

# Dallas, US, November 17th-21st, 2025

Title: 	Discussion on performance requirements for SCM

Source: 	Huawei, HiSilicon

Agenda item:	7.5.3

Document for:	Discussion

Background

A WF[1] on performance requirements for SCM was agreed in last meeting,  where many open issues are left. This contribution provides our related views.

- Discussion

Frequency generalization for rCDL channel modeling

One issue is frequency generalization for rCDL channel modeling:

- Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

It has been agreed to define CDL channel based on fixed doppler regardless frequency. We propose to only consider 10Hz, since only CDL with 10Hz doppler has been aligned in R19 SI. Higher doppler make lead to the risk of non-alignment simulation results.

Proposal 1: RAN4 to only consider maximum Doppler fD =10Hz

Angle scaling for rCDL

One issue is angle scaling for rCDL, following angle scaling procedure for rCDL has been agreed:

- Agreement: - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details. - Agreement on the details: - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies. - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model - Additional rCDL Channel Models based on different AS are not precluded

We propose to only consider tabulated desired AS values ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 and not consider additional rCDL channel Models based on different AS.

Proposal 2: RAN4 to only consider tabulated desired AS values ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 and not consider additional rCDL channel Models based on different AS.

Further rCDL-C1 channel alignment

One issue is whether to further align rCDL-C1 implementations:

- Candidate options: - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4. - Option 2: Not to do further rCDL-C1 channel alignment

We see it’s value to align the channel statistic apart from SNR points since it can ensure the CDL implementation is indeed aligned across companies. But it would be challengeable to finish this work in time considering the limited time budget. We prefer to postpone it to 6G SI for more comprehensive study.

Proposal 3: RAN4 to postpone the further rCDL channel alignment to 6G SI.

Duplex mode and SCS

One issue is duplex mode and SCS:

- Agreement: - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

It has been agreed to use single channel based on 3.5GHz frequency for R20 performance requirements, it’s FFS whether this channel can be applicable for FDD since the frequency difference between FDD and TDD bands could be significant. We prefer only to consider TDD requirements in 5G and further study the feasibility of FDD performance and address the frequency dependent issues in 6G.

Proposal 4: RAN4 to only consider TDD in R20.

UE antenna modelling

One issue is UE antenna modelling, the options are:

- <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901: - #1: Candiadate location for UE antenna placements - #2: Directional antenna assumption for UE - Further discuss the two options in the next meeting: - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901. - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

From a technical perspective, option 1, which has been defined as default antenna placement and assumption in latest version of TS 38.901 could make the antenna modelling more practical and better reflect the antenna implementation. The different placement and radiation pattern will result in different antenna correlation and performance.  But we understand the concern from some companies that more alignment work is needed. We are OK to not consider this in R20 requirements definition and postpone it to 6G SI for more comprehensive study.

Proposal 5: RAN4 to postpone the update of UE antenna modelling to 6G SI.

Delay scaling after cluster truncation

One issue is delay scaling after cluster truncation:

- Candidate options: - Option 1: Consider omitting the post-truncation delay scaling step in rCDL model derivation. Methods other than linear scaling and/or metrics other than RMS delay spread could be investigated. - Option 2: Reuse the Rel-19 SI approach captured in TR38.753

As pointed out from one company in email discussion, the antenna element pattern is omitted when performing delay scaling. I found CDL channel and TDL channel have exact same normalized delay distribution, take CDL-A/TDL-A as an example:

Our understanding is that above phenomenon implies antenna radiation pattern is not considered when RAN1 calculate RMS delay for CDL channel, otherwise the scaling factor would be different and different normalized delay will be got. So it would be better to follow RAN1’s procedure and not to consider AEP for delay scaling. Aligning the scaling factor considering antenna radiation pattern is challengeable and time-consuming. Furthermore, we don’t see any technical reason to change RMS delay spread based methods which is already widely used.

Proposal 6: RAN4 to reuse the Rel-10 SI approach captured in TR 38.753.

PMI bias

One issue is PMI bias, the candidate options are:

- Candidate options: - Option 1: RAN4 shall exploit approaches to address the PMI bias issue - Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 - Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD) - Other options are not precluded - Option 2: No need to consider time-wise evolution for PMI reporting tests - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

For option1a, beam steering defined in current spec is applied to legacy PMI test, where the intention is to make the cluster of TDL channel vary to verify UE’s capability to perform PMI selection correctly under channel cluster with different direction. Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel, which is measured in the deployment, thereby the practicality will be destroyed.

Observation 1: Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel, which is measured in the deployment, so that the practicality will be destroyed.

For option 1b, time variant mean angles will also destroy the inherent angular distribution which is measured in the field and also change the doppler shift. Since the doppler parameters estimation is performed in each TRS slot, varying doppler shift within one TRS period will degrade the performance.

Observation 2: Time variant mean angles will also destroy the inherent angular distribution which is measured in the field and also change the doppler shift. Since the doppler parameters estimation is performed in each TRS slot, varying doppler shift within one TRS period will degrade the performance.

Now we propose a new approach which gives spatially varying CDL channel model more physical significance. The idea is that we are changing the BS bearing angle (alpha) during the PMI test. Considering the cluster direction is defined based on global system coordinate, changing the angle of BS LCS relative to GCS will result in different optimal PMI precoder for the strongest cluster.

For example, figure 2-1 shows PSD (power spectrum distribution) plots for different Base station bearing angle with fixed 10° downtilt angle and 0° slant angle. It can be shown that the PSD is quite different for different BS bearing angle, which will result in different best PMI.

Observation 4: Changing BS bearing angle (alpha) can effectively change power spectrum distribution, resulting in different desired PMI.

| BS: alpha=0°, Beta=10, Gama=0 | BS: alpha=90°, Beta=10, Gama=0 |
| --- | --- |
| BS: alpha=-150°, Beta=10, Gama=0 | BS: alpha=150°, Beta=10, Gama=0 |

- Figure 2-1: Power spectrum distribution for different BS bearing angle.

We propose to apply time varying BS bearing angle to PMI test, which can be described as following:

α=α0+∆α*k

α0 is the random start value with the uniform distribution, ∆α is the step of phase variation, k is the linear increment. To ensure the relative even PMI distribution, it’s proposed to limit the α to [-76°, 14°]. So the α could be described as following:

α=modα0+∆α*k,90°-76°

It’s mentioned that changing the angle of LCS relative to GCS will change the antenna radiation power of each cluster, resulting in different spatially response for each cluster which has impact on result of reduction of CDL channel and may could have inconstant signal power. For simplicity, it’s proposed to introduce omni antenna radiation pattern for BS to eliminate these impacts.

We did a simulation to measure PMI statistic based on Singler panel Type I codebook for such approach under rCDL-C1 channel. In the simulation, ∆α=0.1 is assumed. For codebook configuration, 8Tx with Rank2 was assumed. We calculate the optimal precoder for 3600 consecutive slots. As a reference, we also measured PMI statistic for legacy rCDL-C1 fixed without bearing angle. The results are shown in Figure 2-2:

Figure 2-2: PMI statistics for legacy rCDL-C1 and rCDL-C1 with bearing angle varying

From simulation results shown in Figure 2-2 we can observe that:

Observation 5: Legacy rCDL-C1 channel results in bias PMI statistics with optimal PMI index always being (4,0,0). With Time varying BS bearing angle, all PMI are covered with more even distribution.

In figure 2-3, we compare Follow PMI, Random PMI and fixed PMI with (13,0,0) performance. It can be observed that even with relative most frequent PMI (13,0,0), the performance is still much worse than follow PMI.

Figure 2-3: PMI performance for follow PMI, Random PMI and Fixed PMI with proposed time varying BS bearing angle

Based on above discussion, we have following proposals:

Proposal 7: RAN4 to consider time varying BS bearing angle (denoted as α) for PMI test to address PMI bias issue with following formula:

α=α0+∆α*k

α0 is the random start value with the uniform distribution, ∆α is the step of phase variation, k is the linear increment.

To ensure the relative even PMI distribution, it’s proposed to limit the α to [-76°, 14°]. So the α could be described as following:

- α=modα0+∆α*k,90°-76°

- Proposal 8: RAN4 to consider omni Base station antenna radiation pattern for PMI test.

Regarding option3, we prefer not to define PMI requirements in 5G if PMI bias issues cannot be reasonably addressed.

Proposal 9: Not to define PMI requirements in 5G if PMI bias issues cannot be reasonably addressed.

Precoding, MCS and test metric

One issue is precoding, MCS and test metric, the options are:

- Agreement and candidate options: - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP. - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on: - Option a: Based on per CW thput with Different MCS per CW - Option b: Based on total thput with different MCS per CW - Option c: Based on total thput with same MCS per CW - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

Regarding option a, we think it is pointless to define per CW throughput since in the market the KPIs are always measured based on per UE performance. We don’t see the any UE baseband processing between option b and option c, option b need much effort on MCS selection and evalution, therefore we prefer option c.

Proposal 10: RAN4 to select option c, the test metric is defined based on total throughput with same MCS per CW.

Test applicability for SCM SU-MIMO requirements

One issue is test applicability rules for SCM SU-MIMO requirements:

- Candidate options: - Proposal 1: Introduce new declaration for the SCM testing. RAN4 needs to discuss whether to define one single declaration on SCM requirements or define multiple declarations for each individual SCM test cases. - Proposal 2: Introduce new optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules - Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

Regarding proposal 2, we don’t think UE receiver will change if channel model changes from TDL to CDL. Therefore “SCM compatible MIMO receiver” is not reasonable.   We prefer proposal 3, if requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements. We think only one declaration would be sufficient since all requirements are introduced based on one CDL channel model.

Proposal 11: If requirements with rCDL are introduced, their applicability should be optional and based on single UE declaration to support these requirements.

Release independent for SCM SU-MIMO requirements

One issue is release independent for SCM SU-MIMO requirements, the candidate options are:

- Candidate options: - Proposal 1: Discuss whether to define release independent rules for the new SCM requirements.

We prefer not to introduce release independency for SCM SU-MIMO requirements. Since the requirements are introduced in Rel-20, it is very aggressive to let early UE which is put in market to be tested.

Proposal 12: RAN4 not to introduce release independent for SCM SU-MIMO requirements.

- Conclusion

In this contribution we provide our views on performance requirements for SCM, the proposals and observations are:

Proposal 1: RAN4 to only consider maximum Doppler fD =10Hz

Proposal 2: RAN4 to only consider tabulated desired AS values ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 and not consider additional rCDL channel Models based on different AS.

Proposal 3: RAN4 to postpone the further rCDL channel alignment to 6G SI.

Proposal 4: RAN4 to only consider TDD in R20.

Proposal 5: RAN4 to postpone the update of UE antenna modelling to 6G SI.

Proposal 6: RAN4 to reuse the Rel-10 SI approach captured in TR 38.753.

Observation 1: Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel, which is measured in the deployment, so that the practicality will be destroyed.

Observation 2: Time variant mean angles will also destroy the inherent angular distribution which is measured in the field and also change the doppler shift. Since the doppler parameters estimation is performed in each TRS slot, varying doppler shift within one TRS period will degrade the performance.

Observation 4: Changing BS bearing angle (alpha) can effectively change power spectrum distribution, resulting in different desired PMI.

Observation 5: Legacy rCDL-C1 channel results in bias PMI statistics with optimal PMI index always being (4,0,0). With Time varying BS bearing angle, all PMI are covered with more even distribution.

Proposal 7: RAN4 to consider time varying BS bearing angle (denoted as α) for PMI test to address PMI bias issue with following formula:

α=α0+∆α*k

α0 is the random start value with the uniform distribution, ∆α is the step of phase variation, k is the linear increment.

To ensure the relative even PMI distribution, it’s proposed to limit the α to [-76°, 14°]. So the α could be described as following:

- α=modα0+∆α*k,90°-76°

- Proposal 8: RAN4 to consider omni Base station antenna radiation pattern for PMI test.

Proposal 9: Not to define PMI requirements in 5G if PMI bias issues cannot be reasonably addressed.

Proposal 10: RAN4 to select option c, the test metric is defined based on total throughput with same MCS per CW.

Proposal 11: If requirements with rCDL are introduced, their applicability should be optional and based on single UE declaration to support these requirements.

Proposal 12: RAN4 not to introduce release independent for SCM SU-MIMO requirements.

- Reference

[1] R4-2514823 WF on NR_demod_Ph6_Ph5