3GPP TSG-RAN WG4 Meeting #117	R4-2521382

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.5

Source:	Huawei, HiSilicon

Title:	Discussion on RRM core requirement and testing framework for AIML positioning

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #116 bis [1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

# LCM related requirements

In RAN1#118 the following agreements are reached related to performance monitoring.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

In LCM procedure, the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., triggered by LMF via LPP signaling, the delay of activating/deactivating one function can be defined. Otherwise if these functionality/model changes are autonomously performed by the assisting node (UE or gNB) with AL/ML model, the assisting node would need to monitor the function performance, then the requirements would be related with performance monitoring requirements.

If performance monitoring is mainly executed at the assisting node, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or monitoring delay when certain criterion is met may need to be specified.

Based on RAN1 progress so far, there are many open schemes for performance monitoring. Similar to other use cases, whether and how to define requirements for performance monitoring depends on the selected schemes which will be defined separately for each positioning sub-case. Also, it may depend on what kind of requirements are defined for inference for each positioning sub-case.

Proposal 1: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

# References

- R4-2514631, “WF on AI ML for air interface study”, 3GPP RAN4 #116bis, 2025.