3GPP TSG-RAN WG4 Meeting #117	R4-2521391

Dallas, US, 17th – 21st November 2025

Agenda item:	8.2.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G modulation

Document for:	Discussion

- Introduction

In RAN4#116bis, we started to discuss the 6G system parameter. At that meeting, the approach to the discussion was agreed upon as outlined in the WF [1], and some items are dependent on the progress of RAN1. This contribution describes the views on the modulation that should be considered by RAN4 based on the progress made by RAN1 at the previous meeting.

- Discussion

According to R4-2514650 [1], we agreed the direction of the 6G system parameters as below.

- Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

The study of this item depends on the progress in RAN1. In the previous meeting, RAN1 reached the following agreements regarding modulation.

Agreement For 6GR constellation shaping evaluation for CP-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following: - BLER performance under AWGN channel (at least for performance calibration) - 1st transmission (baseline) and with HARQ re-transmission - BLER performance under fading channel with fixed MCS - 1st transmission (baseline) and with HARQ re-transmission - Throughput performance with link adaptation (adaptive MCS and rank) under fading channel - Needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions) - Transmitter and receiver complexity (e.g., shaping/deshaping, demapper), latency, parallelism implementation, and storage requirements, - Other KPI not excluded, such as PAPR, EVM, MPR/A-MPR - Expected spec impact - FFS detailed assumption of constellation shaping and improved MCS table - System level evaluation can be done after link level evaluation. Agreement For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following - Probabilistic shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss) - Geometric shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE - If multiple constellation shapes are provided for each SE, how to switch between them - GS mapping details, such as bit to constellation point mapping and shape - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission Agreement For link level simulation for modulation evaluation, companies are encouraged to evaluate with the following assumptions and should report the exact scheme evaluated. - channel configuration, including Channel profiles,Tx/RX antenna settings - For MIMO scenario: SU-MIMO or MU-MIMO, follow agenda item 11.2 for MIMO when available. - Precoder assumption - Close loop MIMO (reciprocal beamforming (e.g., SVD, SLR/RZF, etc.), codebook based) - Realistic CSI/SRS/AP-SRS periodicity and delay, and SRS chanEst assumptions, - or genie beamforming - Open loop MIMO - Receiver assumption (for MIMO): LMMSE (baseline) for UL, rML or LMMSE for DL - LLR demapper: Max-log (baseline) or Log-MAP - Channel estimation: Realistic (baseline) or ideal - Other assumptions: Channel coding NR-LDPC (baseline), PxSCH bandwidth, SCS, FD interleaver used or not, 5GNR BICM interleaver usage - Note: For MIMO, SIMO, MISO and SISO are included when possible Agreement For 6GR constellation shaping evaluation for DFT-s-OFDM, and improved MCS table, the proposed scheme will be compared with non-shaping with NR MCS table. In addition to what has been agreed for CP-OFDM in earlier agreement, the evaluation and comparison should further consider at least the following: - PAPR/CM of the resulting waveform - EVM, MPR/A-MPR Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

In RAN1, it has been agreed that modulation schemes defined in NR will be used as the basis for 6G. Currently, 4096QAM for downlink and 1024QAM for uplink are under discussion, but they are still in the study phase. Therefore, RAN4 should first consider evaluating NR-based modulation schemes for 6G. As the 7GHz to 24GHz frequency range is newly considered for 6G, RAN4 should proactively conduct modulation evaluations for these bands. In line with the agreements from RAN4#116bis, this evaluation should be supported by a foundational evaluation framework and include alignment on the transmitter chain model, particularly considering PA model assumptions, to ensure consistency and technical relevance in RF performance assessments.

Observation 1: Based on the current agreements in RAN1, it has been agreed to adopt NR-based modulation schemes for 6G. Accordingly, RAN4 can initiate discussions on evaluation methodologies, including for new frequency ranges.

Proposal 1: From the current agreements in RAN1, RAN4 proposes to initiate feasibility evaluations of NR-based modulation schemes for 6G, including assessments in newly considered frequency bands such as 7GHz to 24GHz.

- Conclusion

In this contribution, we offer our RAN4 views on 6G modulation as follows.

Observation 1: Based on the current agreements in RAN1, it has been agreed to adopt NR-based modulation schemes for 6G. Accordingly, RAN4 can initiate discussions on evaluation methodologies, including for new frequency ranges.

Proposal 1: From the current agreements in RAN1, RAN4 proposes to initiate feasibility evaluations of NR-based modulation schemes for 6G, including assessments in newly considered frequency bands such as 7GHz to 24GHz.

References

[1] 3GPP, R4-2514650, “WF on 6GR system parameter”, Feature lead (Huawei, HiSilicon).