3GPP TSG-RAN WG4 Meeting #117	R4-2521397

Dallas, US, 17th – 21st November 2025

Agenda item:	8.12.2.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G specification enhancement for RRM

Document for:	Discussion

- Introduction

At RAN#116bis, discussions on RAN4 operation efficiency for 6G have begun. One of the topics under discussion is specification enhancement. This contribution proposes improvements to the RAN4 specifications related to RRM.

- Discussion

According to WF [1], the observed issues of 6G RRM specification enhancement are captured as below.

Issue 1-1-3: Issues on RRM Specifications - Observed issues - Issue 1: Redundancy - The duplicated requirement in TS 38.133 leads to confusion and unnecessary extra effort for spec maintenance. - Many instances of redundancy in TS 38.133 for NR, with repeated definitions and tables appearing throughout the specification. - Duplicated clauses exist in TS 38.133. - Duplicated requirements scattered throughout different sections. - Many configuration parameters in test cases are repeated in RRM specs - Issue 2: Cases and scenarios - When defining RRM requirements there are multiple cases and scenarios involved even when they are rarely deployed. - Issue 3: Readability - Measurement requirements categorized by intra/inter-frequency are not efficient in TS 38.133. - Readability and friendliness issues in TS 38.133. - Unclear or non-straightforward relation between requirements and test cases in RRM specs. - Issue 4: Test cases - Test cases exist in the top-level heading in RRM specs

Redundancy

To address redundancy in TS 38.133, the first step is to consolidate duplicated requirements and definitions into a single, unified section. Currently, parameters and tables are repeated across FR1, FR2, RedCap, ATG, and other scenarios, which creates unnecessary complexity. These should be centralized, with only scenario-specific differences highlighted. For example, tables that are currently split between FR1 and FR2 should be merged into one table with separate columns for FR1 and FR2 values. Repeated notes and clarifications, such as those related to DRX handling or measurement gaps, should be moved to an Annex and referenced from the main text. Additionally, void clauses intended for future use should be removed from the main body and placed in an Annex as informative content. This approach will significantly reduce duplication and improve maintainability.

Observation 1: To eliminate redundancy, it is important to first organize and standardize requirements and definitions that are listed repeatedly.

Proposal 1: For sections where the same parameters or tables appear repeatedly across different scenarios such as FR1, FR2, RedCap, and ATG, we will consolidate them into a single common section and adopt a method where only the differences are documented.

Cases and Scenarios

The specification currently includes numerous cases and scenarios that are rarely deployed in real-world networks, which adds unnecessary complexity. To improve clarity, less frequent scenarios should be moved to an Annex, while the main body focuses on common deployment cases. Conditional branching should be simplified by consolidating similar conditions—such as same-band versus different-band or contiguous versus non-contiguous configurations—into unified logic. Furthermore, scenarios should be clearly categorized as “mandatory,” “optional,” or “informative,” with only mandatory requirements retained in the main text. This restructuring will make the specification more practical and easier to navigate.

Observation 2: The definition of RRM requirements includes numerous cases and scenarios that are rarely used in actual operation, leading to increased specification complexity.

Proposal 2: Scenarios that are only used infrequently should be moved to the Annex, allowing the main text to focus on typical cases.

Readability

Improving readability requires a redesign of the document structure. Currently, measurement requirements are split between intra-frequency and inter-frequency sections, which reduces efficiency and makes cross-referencing difficult. These should be reorganized by functional category, with intra/inter-frequency differences noted only where necessary. Complex measurement procedures and conditional logic should be illustrated using flowcharts or diagrams to reduce reliance on lengthy text descriptions. Additionally, the relationship between requirements and test cases should be clarified by explicitly linking each requirement to its corresponding test case ID and providing a comprehensive mapping table in an Annex. This will make the specification more user-friendly and easier to implement.

Observation 3: Each requirement is listed in a straightforward manner, making it difficult to reference.

Proposal 3: Needs to make it easier to understand the specifications by representing measurement procedures and conditional branching with flowcharts and diagrams to reduce text dependency.

- Conclusion

In this contribution, we offer our views on 6G RRM specification enhancement as follows.

Observation 1: To eliminate redundancy, it is important to first organize and standardize requirements and definitions that are listed repeatedly.

Proposal 1: For sections where the same parameters or tables appear repeatedly across different scenarios such as FR1, FR2, RedCap, and ATG, we will consolidate them into a single common section and adopt a method where only the differences are documented.

Observation 2: The definition of RRM requirements includes numerous cases and scenarios that are rarely used in actual operation, leading to increased specification complexity.

Proposal 2: Scenarios that are only used infrequently should be moved to the Annex, allowing the main text to focus on typical cases.

Observation 3: Each requirement is listed in a straightforward manner, making it difficult to reference.

Proposal 3: Needs to make it easier to understand the specifications by representing measurement procedures and conditional branching with flowcharts and diagrams to reduce text dependency.

References

[1] 3GPP, R4-2514648, “Way Forward for [116bis][111] 6G operation efficiency”, Feature lead (CATT).