3GPP TSG-RAN4 Meeting #117	R4-2521398

# Dallas, U.S.A, 17th – 21th November, 2025

Source:	LG Electronics

Title: 	Discussion on the frequency range framework

Agenda Item:	8.5

Document for:	Discussion

Introduction

The key discussion point in 6G spectrum is how to classify and define the 7 GHz to 24 GHz frequency range [1]. And Regulatory input on SAR/PD

Discussion Frequency Range Framework

The definition of the frequency range framework shall take into account the regulatory framework, Mobile Connector, Package Type and Conducted Reference Conditon Arcitecher.

[1] Conducted Reference Conditon Architecher

“Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s)”

The phrase “at the antenna connector … with a single or multiple transmit antenna(s)” defines the reference point for specifying transmitter characteristics at the physical antenna port of the UE. It implies that each transmit chain is evaluated at its corresponding antenna connector, establishing a one-to-one relationship between the transmit chain (Tx path) and the antenna port.

Accordingly:

• “Single transmit antenna” refers to a configuration where one Tx chain is connected to one antenna port.(figure.1a)

• “Single transmit antenna” refers to a configuration where two power amplifiers (PAs) are combined to drive a single antenna, since the combined output shares one conducted reference point at the antenna connector. .(figure.1c)

• “Multiple transmit antennas” refers to a configuration where two or more independent Tx chains are connected to their respective antenna ports (e.g., MIMO transmission). (figure.1b)

However, this phrase does not include architectures where a single Tx chain feeds multiple antennas through a power splitter (figure.1d) for directivity. Such configurations no longer have a clearly defined reference connector per transmit path and therefore fall outside the intended meaning of “specified at the antenna connector.”

Fig 1.

[2] Mobile connector

Generally, When the operating frequency of a mobile connector increases, several challenges arise. First, the connector becomes more susceptible to electromagnetic interference, making it necessary to implement stronger shielding and grounding to meet RF performance requirements. Second, higher frequencies can degrade signal integrity, leading to increased insertion loss, return loss, and crosstalk, which may affect compliance with RF targets for error vector magnitude and data throughput. Finally, the combination of higher data rates and greater power density can create thermal stress and mechanical wear, potentially reducing long-term reliability under 3GPP environmental testing conditions.

From the UE perspective, not only RF performance but also the height of the RF connector and the thickness of the cable are critical factors in determining whether they are suitable for practical use (design).

Generally, the mate height should be less than 1.2 mm, and it is common to use a coax cable with an even smaller diameter of 0.84 mm, with RF performance around 0.5 – 1 dB under 3 GHz and nearly 1 dB around 6 GHz. While the connector company claimed it could be used up to 12 GHz, the mobile UE manufacturer only utilized it for frequencies under 6 GHz.  The recent RF performance (Insertion loss) of mobile connector w/ proper mate height (nearly 1mm) and coaxial cable size is about 0.5–1 dB below 6 GHz, nearly 1 dB around 10 GHz, and approximately 1.3–1.4 dB at 15 GHz. While the connector company claims it can be used up to 15 GHz, it is expected not to be used above 10 GHz because the insertion loss exceeds 1 dB.

(2.5mm  1.0mm)

(Cable Size : 0.46,0.64,0.81,1.13,1.37mm )

Fig.2 Improvement of Mate Height from 2.5mm to 1mm and Mcro coaxial cable size for low profile UE

Fig.3 Reduced Insertion Loss in the Latest Mobile Connector Version Compared to Previous Models

[3] Package Tyape - AIP vs SRP vs SEMI-AIP

An Antenna-in-Package (AIP) is a fully integrated solution that combines the antenna, RFIC, filters, Phase Shifter, PA, LNA, and matching networks onto a single low-loss substrate. In contrast, a Stand-alone RFIC Package (SRP) includes all the components of an AIP except the antenna and phase shifter. In both cases, the key is integrating components on the substrate to ensure high-frequency performance and reliability — for example, AIP designs are often used in mmWave 5G modules where the antenna must be tightly coupled to the RFIC, while SRP packages having mobile connector are essentially AIPs with the antenna removed.(Wifi-chip)

In certain 5G modules, a Semi-AIP configuration exists in which the antenna is not integrated into the same package as in AIP, but is also not fully separated as in SRP with a mobile connector. In this structure, the package and antenna are physically separated, while a breakout connector or special connecting technology is employed to interface them (see Fig. 3).

The frequency range below 24 GHz is expected to primarily employ SRP structures and/or Semi-AIP structures with a substrate-based manufacturing technique, whereas frequencies above 24 GHz are considered more suitable for AIP structures.  Below 24 GHz, the longer wavelength leads to larger antenna sizes, which makes side-edge AIP placement impractical. However, the Semi-AIP structure offers advantages such as integration with the phone design and improved mechanical stability, but it is expensive and has about performance loss.

AIP                                                                       Semi – AIP

Fig.4 AIP vs Semi-AIP

Conclusion Frequency Range Framework

Observation1. “Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s)”

The phrase “at the antenna connector … with a single or multiple transmit antenna(s)” defines the reference point for specifying transmitter characteristics at the physical antenna port of the UE. It implies that each transmit chain is evaluated at its corresponding antenna connector, establishing a one-to-one relationship between the transmit chain (Tx path) and the antenna port.

Accordingly:

• “Single transmit antenna” refers to a configuration where one Tx chain is connected to one antenna port.(figure.1a)

• “Multiple transmit antennas” refers to a configuration where two or more independent Tx chains are connected to their respective antenna ports (e.g., MIMO transmission). (figure.1b)

• “Single transmit antenna” refers to a configuration where two power amplifiers (PAs) are combined to drive a single antenna, since the combined output shares one conducted reference point at the antenna connector. .(figure.1c)

However, this phrase does not include architectures where a single Tx chain feeds multiple antennas through a power splitter (figure.1d) for directivity. Such configurations no longer have a clearly defined reference connector per transmit path and therefore fall outside the intended meaning of “specified at the antenna connector.”

Figure 1

Proposal 1. It is reasonable to consider defining the frequency range between FR1 and FR2-1 as a range where one transmit chain can be connected to multiple radiating elements via a passive power divider in order to compensate for path loss.

Observation2. The recent mobile connector, with a loss of less than a maximum of 1 dB and a mechanical size (mate height and cable size) under 1 mm, is suitable for use in mobile phones. Based on the provided connector performance data, it appears usable up to 10 GHz for smart-phone.

Observation3. For frequencies below 24 GHz, the longer wavelength increases antenna size, making AIP (Anitenna in Package) integration along handset edges impractical and mechanically less robust. Therefore, SRP structures/ Semi-AIP structure are preferred in this frequency range.

Proposal 2. Lowering the FR2-1 frequency range is worth considering. While a reduced frequency increases antenna dimensions in AIP (Antenna in Package) modules, making practical PCB-edge integration more difficult yet still feasible, it becomes viable for commercial use in Semi-AIP (Semi-Antenna in Package) modules due to advantages such as integration with the phone structure and improved mechanical stability. (ex). FR2-1: 24.125  15GHz

Proposal 3.  At present, it is reasonable to consider extending the FR1 frequency range upward to around 10 GHz, and it can be used up to 15 GHz as well, since the mate height and cable dimensions of mobile connectors are within 1 mm, making them suitable for mounting in high-end mobile phones. (ex). FR1 : 7.125  15GHz

Proposal 4.  Assuming there is a path loss issue from around 7 GHz and above, FR1 can be divided into FR1-1 and FR1-2. FR1-1 would consider the basic structure with an antenna directivity of 0 dBi, while FR1-2 could consider antennas with directivity. The reason is that the path loss in the FR1-2 band is greater than that in the FR1-1 band.

(FR1  Devide region FR1-1 & FR1-2)

Proposal 5. The FR1‑2 region appears to be almost impractical to use for beamforming/beamsweeping with an array configuration, due to reliability issues arising from array size and mechanical structural constraints. Furthermore, when using the existing basic structure with an antenna directivity of 0 dBi, there seems to be no suitable method to compensate for the increased path loss

- 5. Summary for Frequency Range Framework

Proposal 1. It is reasonable to consider defining the frequency range between FR1 and FR2-1 as a range where one transmit chain can be connected to multiple radiating elements via a passive power divider in order to compensate for path loss.

Proposal 2. Lowering the FR2-1 frequency range is worth considering. While a reduced frequency increases antenna dimensions in AIP (Antenna in Package) modules, making practical PCB-edge integration more difficult yet still feasible, it becomes viable for commercial use in Semi-AIP (Semi-Antenna in Package) modules due to advantages such as integration with the phone structure and improved mechanical stability.

(ex). FR2-1: 24.125  15GHz

Proposal 3.  At present, it is reasonable to consider extending the FR1 frequency range upward to around 10 GHz, and it can be used up to 15 GHz as well, since the mate height and cable dimensions of mobile connectors are within 1 mm, making them suitable for mounting in high-end mobile phones.

(ex). FR1 : 7.125  15GHz

Proposal 4.  Assuming there is a path loss issue from around 7 GHz and above, FR1 can be divided into FR1-1 and FR1-2. FR1-1 would consider the basic structure with an antenna directivity of 0 dBi, while FR1-2 could consider antennas with directivity. The reason is that the path loss in the FR1-2 band is greater than that in the FR1-1 band.

(FR1  Devide region FR1-1 & FR1-2)

Proposal 5. The FR1‑2 region appears to be almost impractical to use for beamforming/beamsweeping with an array configuration, due to reliability issues arising from array size and mechanical structural constraints. Furthermore, when using the existing basic structure with an antenna directivity of 0 dBi, there seems to be no suitable method to compensate for the increased path loss.

- Other Path loss compensation technology: F.F.S

- Proposed

Figure1.(a)

Figure 1.(b)

- Around 7 GHz: closer to FR1 characteristics;

- Around 15 GHz: closer to FR2 characteristics

; eq. 4 element Array (AIP / Semi-AIP module) for beamforming/sweeping capability.

- From around 7 to 15 GHz : closer to between FR1& FR2 characteristics

; directivity antenna for pathloss compensation

Advantage:

- Minimum 3GPP specification with compensation for the gray area, addressing the drawback of path loss.

- Clarification of the definition of the gray area using criteria

Disadvantage : Impact on standards when using antennas with 2–3 dBi directivity instead of typical 0 dBi antennas.

Figure 2.

- Around 7 GHz: closer to FR1 characteristics;

- Around 15 GHz: closer to FR2 characteristics; eq. 4 element w/ 1- Array Panel for beamforming capability.

- From around 7 to 15 GHz : F.F.S

References

[1] R4-2514626 WF on 6G Spectrum, 3GPP TSG-RAN4 Meeting #116 bis