3GPP TSG-RAN4 Meeting #117	R4-2521406

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0520 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.141-1 for Rel-19 BS MMSE-IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Need to align the Notes in Table 8.2.16.4.2-2 with 38.104 and 38.141-2. - Need to fix the number of Rx for each number of interferer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made in Table 8.2.16.4.2-2: - Update the Notes in Table 8.2.16.4.2-2 to align with 38.104 and 38.141-2. - Remove 4/8 Rx for 1 interferer case in the Notes - Remove 2Rx for 2 interferers case in the Notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong number of interferers for 2Rx or 4 Rx or 8Rx is set for the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding draftCR R4-2514284 has been endorsed in RAN4#116bis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

<Start of Change#1>

### 8.2.16	Enhanced performance requirements for PUSCH with synchronous interference

#### 8.2.16.1	Definition and applicability

The enhanced performance requirement of PUSCH with synchronous interference is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.1.11.

#### 8.2.16.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] clause 8.2.X.

#### 8.2.16.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with enhanced performance requirements.

#### 8.2.16.4	Method of test

##### 8.2.16.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

##### 8.2.16.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.16.4.2-1.

Table 8.2.16.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15kHz | 5 | -86.5 dBm / 4.5MHz |
|  | 20 | -80.2 dBm / 19.08MHz |
| 30kHz | 10 | -83.6 dBm / 8.64MHz |
|  | 100 | -73.1 dBm / 98.28MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.16.4.2-2.

Table 8.2.16.4.2-2: Test parameters for testing PUSCH with enhanced performance requirement

| Parameter |  |  | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interfer 1 (Note 1) | Interfer 2 (Note 1) |
| Transform precoding |  |  | Disabled |  |  |
| Default TDD UL-DL pattern (Note 2) |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |  |
| INR (Note3) |  | Set 1 | N/A | 5.35 for 2Rx tests 7.28 for 4/8 RX tests | N/A for 2Rx tests -2.52 for 4/8 RX tests |
|  |  | Set 2 |  | 9.82 for 2Rx tests 12.38 for 4/8 RX tests | N/A for 2Rx tests -0.97 for 4/8 RX tests |
| Cell ID |  |  | 0 | 1 | 2 |
| Interference model |  |  | N/A | As specified in clause B.7.2 | As specified in clause B.7.2 |
| HARQ | Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
|  | RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DM-RS | DM-RS configuration type |  | 1 |  |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |  |
|  | Additional DM-RS position |  | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |  |
|  | DM-RS port(s) |  | {0} |  |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type |  | A, B |  |  |
|  | Start symbol |  | 0 |  |  |
|  | Allocation length |  | 14 |  |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |  |
|  | Frequency hopping |  | Disabled |  |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |  |
| NOTE 1: One explicit interferer, i.e., interferer 1, is modelled for tests with 2RX antennas. Two explicit interferers are modelled for tests with 4/8 RX antennas. NOTE 2: The same requirements are applicable to FDD and TDD with different UL-DL pattern. NOTE 3: The respective received energy of each interferer relative to noise is defined by its associated INR value as specified in Annex B.7.1. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. NOTE 4 All cells are time-synchronous. |  |  |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in table 8.2.16.5-1 to table 8.2.16.5-8 is achieved at the BS input.

6)	For each of the reference channels in table 8.2.16.5-1 to table 8.2.16.5-8 applicable for the base station, measure the throughput.

#### 8.2.16.5	Test Requirement

The throughput measured according to clause 8.2.16.4.2 shall not be below the limits for the SNR levels specified in table 8.2.16.5-1 to table 8.2.16.5-8.

Table 8.2.16.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 14.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 15.3 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 10.7 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 7.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 6.7 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 14.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 15.3 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 11.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 11.7 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 7.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 7.4 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-3: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 15.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 15.4 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 11.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 11.0 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 7.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 7.1 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-4: Test requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-14 | pos1 | 15.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-14 | pos1 | 15.3 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-14 | pos1 | 11.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-14 | pos1 | 11.4 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-14 | pos1 | 7.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-14 | pos1 | 7.1 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-5: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 14.6 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 15.2 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 11.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 10.8 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-8 | pos1 | 7.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-8 | pos1 | 6.7 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-6: Test requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 14.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 15.4 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 11.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 11.5 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-10 | pos1 | 7.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-10 | pos1 | 6.8 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-7: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 14.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 15.3 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 11.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 11.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 7.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 7.3 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

Table 8.2.16.5-8: Test requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) |  | INR set | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 15.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 15.0 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 11.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 11.4 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* | 70 % | G-FR1-A4-11 | pos1 | 7.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 | 70 % | G-FR1-A4-11 | pos1 | 7.2 |
| Note*: Not applicable for Local Area BS Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent |  |  |  |  |  |  |  |  |  |

==============End of change==============