3GPP TSG-RAN WG4 Meeting #117	R4-2521414

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	On Demodulation and CSI Reporting Requirements for CSI Prediction

Agenda item:	6.11.8

Document for:	Discussion

Introduction

The open issues for discussion in RAN4#116bis was documented in the topic summary for NR AIML air part1 in [1]. The ones that relate to demodulation and CSI reporting requirements are summarized below:

### Sub-topic 1-6 Simulation results and next steps Several companies submitted simulation results and proposals for next steps (refinement of parameters, etc) Issue 1-6: Simulation results and next stepts - Proposals - Discuss the simulation results and next steps - Simulation results - Refinement of parameters: - Introduce realistic channel estimation - Further discuss CSI-RS configuration, codebook configuration, Dopller, SNR, etc - Recommended WF - To be discussed Discussion on refinement of simulation parameters to be done mainly offline ### Sub-topic 1-7 Doppler and MCS choiceGeneralization Issue 1-7: Doppler and MCS - Proposals - Option 1: Introduce tests for 20Hz Doppler with MCS17/19 with 32Tx ports - Option 2: Introduce tests for 50Hz Doppler with MCS17/19 with 32Tx ports - Option 3: Others - Recommended WF - Option 1

During RAN4#116bis discussions, the following agreements related to the above mentioned issues have been made and documented in the way forward of Rel-19 Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface in [2] and shown below:

Issue 1-7: Doppler and MCS Agreement: Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD Introduce test for TDD with the details FFS

This contribution provides our views on few issues in demodulation and/or CSI reporting requirements for CSI prediction for discussions in RAN4#117.

Discussion

The agreements for the Doppler and MCS issue made in RAN4#116bis were to introduce tests for 20Hz Doppler with MCS 17 or 19 with 16Tx ports and 2Rx in FDD and to introduce test for TDD with the details to be kept as FFS.

In relation to those agreements, the following is stated in the WF on simulation assumption in [3] as below:

- For the simulation assumption, as agreed in RAN4 #116 bis meeting, “Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD, Introduce test for TDD with the details FFS”, following revising will be captured in the updated simulation. - Remove the 50Hz doppler and remove the mixed Doppler assumption - keep MCS 17 and MCS 19, remove MCS 13 - Use 2RX for FDD - Use 4RX for TDD (update is allowed if any new agreement) - Remove the assumptions on 32Tx ports

Furthermore, [3] also lists one open issue related to aperiodic CSI-RS configuration, namely,:

FFS test setup for aperiodic CSI-RS configuration

In RAN4#116bis several companies provided simulation results for Step 3, which were documented in [4]. One of the issues observed from the simulation results collection in [4] was the unaligned results, especially, for the throughput in Step 3. Nonetheless, if one removes the outliers, one may observe that, in the case of AI/ML with mixed dataset, the span between the remaining SNR and throughput results of MCS 19 is smaller compared to MCS 17. This shows that the SNR and throughput results of MCS19 are more aligned.

The remaining time for this WI is very limited; hence, RAN4 may need to make prioritization on the use cases.

From Step 3 SNR and throughput simulation results collection in RAN4#116bis, in the case of AI/ML with mixed dataset, after outlier removal, the span of MCS 19 is smaller than MCS 17.

Considering the limited time left for this WI, RAN4 to prioritize FDD and TDD periodic CSI-RS configuration as agreed in RAN4#116bis and deprioritize the aperiodic one.

Concerning the MCS, RAN4 can prioritize MCS 19 over MCS 17.

.

Conclusion

In this contribution, we provide our views on few issues in Rel-19 NR AIML air part1. The following Observations and Proposals were made:

Observation 1: The remaining time for this WI is very limited; hence, RAN4 may need to make prioritization on the use cases.

Observation 2: From Step 3 SNR and throughput simulation results collection in RAN4#116bis, in the case of AI/ML with mixed dataset, after outlier removal, the span of MCS 19 is smaller than MCS 17.

Proposal 1: Considering the limited time left for this WI, RAN4 to prioritize FDD and TDD periodic CSI-RS configuration as agreed in RAN4#116bis and deprioritize the aperiodic one.

Proposal 2: Concerning the MCS, RAN4 can prioritize MCS 19 over MCS 17.

- References

- R4-2514519, Topic summary for [116bis][112] R19 AI for air interface, 3GPP RAN4#116bis, Qualcomm, Prague, Czech Republic, October 13-17, 2025

- R4-2514631, WF on [116bis][112] R19 AI for air interface, 3GPP RAN4#116bis, Qualcomm Incorporated, Prague, Czech Republic, October 13-17, 2025

- R4-2514634, WF on updated simulation assumption for AI based CSI prediction, 3GPP RAN4#116bis, Oppo, et. al, Prague, Czech Republic, October 13-17, 2025

- R4-2514635, Summary on the CSI prediction simulation results, 3GPP RAN4#116bis, Oppo, Prague, Czech Republic, October 13-17, 2025