3GPP TSG-RAN WG4 Meeting #117	R4-2521415

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	On the Evolution of NR Duplex Operation - Sub-Band Full Duplex

Agenda item:	6.13.4

Document for:	Discussion

Introduction

The way forward from Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD) demodulation performance discussions during RAN4#116bis is documented in [1]. The agreements made during RAN4#116bis are summarized below [1]:

Issue 1-1-1 how to model interference modeling for performance evaluation Agreement: - If RAN4 to define Rel-19 SBFD PUSCH requirement with UL muting only considering inter-site gNB-gNB interference, self-interference and intra-site inter sector interference are not taken into account. - For inter-site gNB-gNB interference modelling - Considering the RF requirement, the following INR values derived based on link budget analysis can be considered for evaluation. Based on evaluation results, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting Table 2-1 Revised INR values for FR1 Conditions; FR1. Interf1 > Interf2; INR1 <= 9.2dB, INR2 <= -0.8dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1]} dB. Interf1 = Interf2; INR1 = INR2 <=3.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Not pursue to derive INR based system level simulation for inter-site gNB-gNB interference modelling when defining PUSCH requirement with UL muting for SBFD in Rel-19. Issue 1-1-2 whether to introduce the PUSCH requirement with UL muting Agreement: - FFS on whether RAN4 introduce the PUSCH requirement with UL symbols muting

In addition to the agreements above, several open issues concerning the test setup for demodulation requirements (if introduced) as listed below are captured in [1]:

Issue 1-2-1 Frequency Range Issue 1-2-2 Waveform for wanted signal Issue 1-2-3 sub-band pattern Issue 1-2-4 Channel Bandwidth for wanted signal Issue 1-2-5 TDD pattern Issue 1-2-6 Sub-band configuration in SBFD slot Issue 1-2-7 Number of symbol muting and symbol configuration Issue 1-2-9 PUSCH mapping type for wanted signal and interferers Issue 1-2-10 Number of DMRS symbols for wanted signal Issue 1-2-11 Channel model Issue 1-2-12 MCS Issue 1-2-13 Antenna Configuration Issue 1-2-14 Other Assumption

It is also an important reminder that, in RAN4#116, RAN4 has agreed to not define UE demodulation requirements for Rel-19 SBFD [2], as shown below

Topic 1 UE demodulation requirement ### Sub-topic 1-1 General Scope Issue 1-1-1 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Agreement - Not to introduce UE demodulation requirements with SBFD carrier (with DL sub-band and UL sub-band)

The discussions in RAN4#117 will mainly focusing on whether RAN4 will introduce SBFD BS PUSCH requirement with UL muting. This contribution provides our views on the issues for discussions during RAN4#117 meeting.

Discussion

We will start by returning to the Work Item Description (WID) in [3] as a reminder of the description of the work for Rel-19 SBFD. Afterwards, our views on Rel-19 SBFD BS PUSCH demodulation requirements will follow.

Related information from the Work Item Description

Some important information extracted from the justification of the WI:

According to the conclusion in the TR 38.858, it is worth specifying SBFD operation at the gNB side within a TDD carrier, gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation, and RF requirements for SBFD operation at gNB in Rel-19.

As shown above two of the main components for SBFD operations, especially in relation to the demodulation performance are gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation. This emphasizes the importance of CLI handling for SBFD operations both at the BS and UE.

The justification of the WI in the WID emphasized gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation. It shows the importance of CLI handling for SBFD operations both at the BS and UE when defining the corresponding SBFD demodulation requirements.

PUSCH Requirements

In WF [2], it is stated that MMSE-IRC as in Rel-19 performance enhanced (that is Rel-19 Demodulation Phase 5 BS MMSE-IRC) WI is to be considered for Rel-19 SBFD. Consequently, a proper SBFD interference model should be used, since MMSE-IRC in Rel-19 performance enhanced WI is also using a proper interference model.

Compared to the legacy TDD operation, the PUSCH demodulation in an SBFD-capable gNB is affected by its own self-interference as well as the gNB-to-gNB CLI (cf. CLI types 1, 2, 3 in Figure 1 in the Appendix). Concerning the gNB self-interference, on how to accommodate such an interference into the link-level simulation is not yet clear. As the transmit antennas and the receive antennas are close to each other, propagation conditions as in [6] may not be used as the propagation condition of the self-interference signal. Nonetheless, referring to [5] self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

However, in RAN4#116bis, it was agreed that if RAN4 define requirements, it would not consider self-interference and intra-sector interference [1].

Even though in TR 38.858, the self-interference can be modelled as AWGN, in RAN4#116bis, it was agreed not to consider self-interference and intra-sector gNB-to-gNB interference.

Concerning the inter-site gNB-to-gNB interference, as this is appearing in SBFD multi-cell scenarios, RAN4 needs to properly model it. It can be modelled following the common procedure in RAN4, which was also done in LTE when modelling the interference in UL inter-cell interference scenario (cf. TR 36.884 [8]). In TR 36.884 [8], the UL inter-cell interference model of LTE is obtained from system-level simulations. Such extensive system-level simulations are needed to quantify the so-called Dominant Interference Proportion (DIP), which would then be used in the link-level simulation to define the corresponding demodulation requirements. Different to TR 36.884 for LTE, in SBFD case, the system-level simulation needs to consider the other gNBs from other cells as sources of interference instead of the UEs from other cells as in the LTE case. Moreover, different allocations (e.g., DUD vs DU/UD) in SBFD symbols/slots and different SBFD BW sizes among different cells should be considered.

Even for the work of Rel-19 NR Demodulation Phase 5 BS MMSE-IRC, it was understood that a proper DIP from system level is needed. Nonetheless, since the DIP from LTE in TR 36.884 [8] can still be used (as the source of interference in TR 36.884 [8] and in Rel-19 BS MMSE-IRC is the same), Rel-19 BS MMSE-IRC was reusing LTE’s DIP for defining the requirements. This is not the case for Rel-19 SBFD, which source of interference is different than in LTE case, namely, the other gNB signals instead of other UEs in other cells.

It is a common procedure in 3GPP RAN4 both in LTE and 5GNR when defining BS demodulation requirements in a multi-cell scenario with inter-cell interference, to properly model the interference with realistic deployment scenario using system-level simulation.

As documented in [1], however, for evaluation purpose of Rel-19 SBFD, the INR values derived from link budget analysis are used. Furthermore, most of the companies were reluctant to follow system-level simulation approach as in LTE and 5GNR legacy to properly model the interference under a realistic deployment scenario. While such INR values derived from a simplified link-budget model could be fine for evaluation purposes, nonetheless, for defining minimum performance requirements, INR values from a realistic deployment scenario are needed to ensure that the minimum performance requirements will somehow reflect the minimum performance of the DUT (device under test) when it is deployed in the field.

The INR derived from link-budget is used for evaluation. On the other hand, system-level simulations for deriving INR under realistic deployment scenario were not agreed by companies in RAN4#116bis.

There is no proper interference model from realistic deployment scenario derived from system-level simulation is available to specify minimum performance requirements for Rel-19 SBFD BS.

We further note that there is one contribution from multiple companies in RAN#109 which support Rel-20 SBFD. They proposed to have SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenarios for Rel-20 SBFD. This shows two important issues which not yet covered in Rel-19 SBFD, namely, SBFD BS to SBFD BS adjacent channel coexistence and realistic SBFD deployment scenario. RAN Plenary has approved a new Work Item (WI) for Rel-20 in which SBFD is a part of it, namely, Rel-20 NR base station (BS) RF requirement evolution for FR1 and testing phase 2 WI [10]. Hence, for Rel-19 SBFD, RAN4 needs also to ensure that for demodulation performance requirements the interference modeling must consider a realistic deployment scenario.

RAN Plenary has approved a WI for Rel-20 SBFD to consider SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenario.

In RAN4#115 offline discussion [12], while it is known that UE-to-UE CLI is indeed a source of interference in SBFD, it was not agreed to consider it due to the difficulties of modeling the UE-to-UE interference. Hence, no UE demodulation requirements are defined.  We note here that BS demodulation has even more complex interference to consider, namely, the self-interference, intra-sector gNB and inter-site gNB interference.

For SBFD UE demodulation, as UE-to-UE CLI was considered difficult to model, it was agreed not to define SBFD UE demodulation requirements. However, the gNB-to-gNB CLI is even more complex than UE-to-UE CLI. Hence, if RAN4 do not define UE requirements due to the lack of proper UE-to-UE CLI, it is also fair enough to not define BS requirements due to the lack of proper model of gNB-to-gNB CLI.

RAN4 shall follow the common procedure as in LTE and 5GNR legacy to properly model the interference by considering a realistic deployment scenario when defining the minimum performance requirements for BS with multi-cell interference.

If there is no proper gNB-to-gNB interference model from a realistic deployment scenario, RAN4 shall not define BS demodulation minimum performance requirements.

In RAN4#116bis, several companies have provided simulation results which were documented in [13]. They were using INR derived from link-budget method to model the inter-site gNB interference, and not from system level simulations under realistic deployment scenario. While this is fine for any evaluation on how UL muting is functioning, it is not an appropriate way for defining minimum performance requirements which need proper model for interference considering realistic deployment scenario.

Analyzing the gain of UL muting from the simulation results in [13], only relatively small gain can be observed compared to without UL muting. It is worth mentioning that in RAN4, there is normally imperfection added on top of the ideal simulation results. If the gain is too small, the requirements itself cannot differentiate UL muting versus without UL muting. Hence, passing the requirements itself could not guarantee the proper implementation of UL muting.

From the simulation results collection in RAN4#116bis, it can be observed that the gain of UL muting is small.

Impairment margins around 2-3 dB are normally added on top of ideal simulation results. The final requirements are defined based on the impairment results.

Passing the requirements could not guarantee that UL muting is correctly implemented if the gain of UL muting is considerably small.

If the gain of UL muting is relatively small, which makes it difficult to ensure the correct implementation even when passing the test, RAN4 shall avoid specifying BS requirements with UL muting.

Conclusion

In this contribution, we provide our views on demodulation performance requirements for Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD). The following Observations and Proposals were made:

Observation 1: The justification of the WI in the WID emphasized gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation. It shows the importance of CLI handling for SBFD operations both at the BS and UE when defining the corresponding SBFD demodulation requirements.

Observation 2: Even though in TR 38.858, the self-interference can be modelled as AWGN, in RAN4#116bis, it was agreed not to consider self-interference and intra-sector gNB-to-gNB interference.

Observation 3: It is a common procedure in 3GPP RAN4 both in LTE and 5GNR when defining BS demodulation requirements in a multi-cell scenario with inter-cell interference, to properly model the interference with realistic deployment scenario using system-level simulation.

Observation 4: The INR derived from link-budget is used for evaluation. On the other hand, system-level simulations for deriving INR under realistic deployment scenario were not agreed by companies in RAN4#116bis.

Observation 5: There is no proper interference model from realistic deployment scenario derived from system-level simulation is available to specify minimum performance requirements for Rel-19 SBFD BS.

Observation 6: RAN Plenary has approved a WI for Rel-20 SBFD to consider SBFD BS to SBFD BS adjacent channel coexistence focusing on realistic deployment scenario.

Observation 7: For SBFD UE demodulation, as UE-to-UE CLI was considered difficult to model, it was agreed not to define SBFD UE demodulation requirements. However, the gNB-to-gNB CLI is even more complex than UE-to-UE CLI. Hence, if RAN4 do not define UE requirements due to the lack of proper UE-to-UE CLI, it is also fair enough to not define BS requirements due to the lack of proper model of gNB-to-gNB CLI.

Proposal 1: RAN4 shall follow the common procedure as in LTE and 5GNR legacy to properly model the interference by considering a realistic deployment scenario when defining the minimum performance requirements for BS with multi-cell interference.

Proposal 2: If there is no proper gNB-to-gNB interference model from a realistic deployment scenario, RAN4 shall not define BS demodulation minimum performance requirements.

Observation 8: From the simulation results collection in RAN4#116bis, it can be observed that the gain of UL muting is small.

Observation 9: Impairment margins around 2-3 dB are normally added on top of ideal simulation results. The final requirements are defined based on the impairment results.

Observation 10: Passing the requirements could not guarantee that UL muting is correctly implemented if the gain of UL muting is considerably small.

Proposal 3: If the gain of UL muting is relatively small, which makes it difficult to ensure the correct implementation even when passing the test, RAN4 shall avoid specifying BS requirements with UL muting.

- Appendix

Interference modeling

Due to different transmitter and receiver imperfections, SBFD faces challenges from cross-link interference (CLI) between UEs and CLI between gNBs. Figure 1 illustrates the CLI types studied in RAN1 and RAN4, namely:

- gNB self-interference (1),

- gNB-to-gNB inter-sector (or intra-site) interference (2),

- gNB-to-gNB inter-site interference (3),

- UE-to-UE intra-cell interference (4), and

- UE-to-UE inter-cell interference (5).

Figure 1: CLI types present in SBFD

Concerning interference signals at the gNB, the modelling of CLI assumed during the Rel-18 Duplex Evolution SI is briefly described in the following, with primary focus on FR1 (more details (and on FR2-1) can be found in TR 38.858 [5]).

- gNB Self-interference modeling

The feasibility of SBFD was assessed by breaking down the self-interference into different components, such as transmit power, frequency isolation capability, spatial isolation, TX beam nulling and RF interference cancellation. The assessment was based on a framework for calculating the residual self-interference calculation (RSIC) under the assumption that the UL sensitivity degradation at the gNB receiver due to the self-interference is 1 dB. Candidate values of RSI were provided by RAN4 to RAN1, so that the desense target was also considered in the performance evaluation of SBFD.

- gNB Inter-sector interference

The objective of gNB inter-sector interference analysis was to identify the inter-sector spatial isolation values that would result in additional desensitization at the gNB receiver. RAN4 considered different factors in both the transmitter characteristics (e.g., tx power and beam nulling) and the BS deployment (e.g., antenna isolation and absorbent materials between the different sectors). gNB inter-sector spatial isolation values between 75 dB to 103 dB for FR1 were discussed, where the latter would also result in a negligible UL received desensitization (considering the effects of ACLR/ACS on top of the spatial isolation) even for high-power FR1 macro base station deployments.

- Inter-site gNB-to-gNB interference

The co-channel inter-sub-band inter-site interference analysis considered the unwanted emission impact on the receiver selectivity. The same transmitter leakage and receiver impairment model as for investigating the gNB self-interference was used. However, the antenna isolation was replaced with the inter-site isolation which can be derived using 3GPP propagation models (based on TR 38.901 [6]). The Tx leakage baseline studied was the gNB adjacent channel leakage ration (ACLR), and the receiver impairment was the gNB adjacent channel selectivity (ACS).

- References

- R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, Samsung, 3GPP TSG RAN4#116, Prague, Czech Republic, October 13-17, 2025

- R4-2512615, Way Forward for [116][328] NR_duplex_evo_demod, Samsung, 3GPP TSG RAN4#116, Bengaluru, India, August 23-29, 2025

- RP-251874, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, 3GPP TSG RAN4#114, Athens, GR, February 17 – 21, 2025

- 3GPP TR 38.858, Study on Evolution of NR Duplex Operation, V18.2.0, 12-2024

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0, 03-2024

- R4-2507626, On the Evolution of NR Duplex Operation - Sub-Band Full Duplex, Nokia, 3GPP TSG RAN4#115, Malta, May 19 – 23, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, V13.1.0, 09-2016

- 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0, 03-2025

- RP-252905, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Huawei, 3GPP TSG RAN#109, Beijing, China, September 15-18, 2025

- RP-252691, Proposal for SBFD-SBFD adjacent channel co- existence scenario, China Unicom, et al., 3GPP TSG RAN#109, Beijing, China, September 15-18, 2025

- R4-2508721, Offline meeting minutes for [115][330] NR_duplex_evo_demod, Samsung, 3GPP TSG RAN4#115, Malta, May 19-23, 2025

- R4-2515039, Simulation results summary for SBFD demodulation, Samsung, 3GPP TSG RAN4#116, Prague, Czech Republic, October 13-17, 2025