# 3GPP TSG-RAN WG4 Meeting #117	R4-2521423

# Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on UE RF requirements of HD-FDD for Ku band with FR1 numerology

Source:	Samsung

Agenda item:	7.6.4.1

Document for:	Approval

Introduction

In last RAN #109 meeting, the meeting approved the WID [1] for further enhancement of both NR-NTN and IoT-NTN. In this agenda, it includes three major aspects: HPUE for NTN, HD-FDD for NTN and DL intra-band intra-SAN CA. And in this paper, we will focus on the NR-NTN objectives as follows:

HD-FDD for NTN Core Part

- Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 2:  Ku band and Ka band with FR2 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

- Note: Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

Discussion

In last #116bis RAN4 meeting, the WF [2] had captured following assumptions for HD-FDD discussion, which we believe are important and reasonable.

- Issue 1-1-1: Antenna type assumption for HD-FDD VSAT - WF - The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with Tx/Rx switching capability. - Other implementations are not precluded.

The single physical aperture is supported by the justification from the WID[2], and also aligned with our previous discussion paper[3]. The target of this WID, as explained in the WID, is to introduce a lower cost, and space-limited VSAT type that fits into the automotive.  From this perspective, we believe we should anyhow focus or prioritize the spec work on single physical aperture device.

Observation 1: According to WID, we believe the single physical aperture device should be focused or prioritized in this WI.

With the same understanding, in our last paper, our initial view is that apart from the potential gain of less insertion loss for EIRP and min EIS requirements, the Ku-band VSAT requirements should be able to be re-used to the HD-FDD Ku-band VSAT.

Observation 2: In general, our observation is that no changes may be brought to RF requirements by simply having the HD-FDD VSAT compared to FD-FDD. As a result, maybe a general description on existing Ku-band VSAT requirements can be introduced to declare the requirements can also apply to HD-FDD VSAT.

- Issue 1-1-5: EIRP requirements for HD-FDD - WF - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point - FFS on whether to modify. - Issue 1-1-6: EIS requirements for HD-FDD - WF - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point - FFS on whether to modify.

Proposal 1: To re-use existing FDD VSAT requirements for HD-FDD Ku-band under FR1 numerology.

Besides the HD-FDD, the impacts from introducing the ultra small antenna is somehow confusing us. In our last paper [3], we suggest to further clarify the antenna characteristics of this targeted “ultra small antenna”, however it was lack of inputs from the proponents.

In our initial review, the change of antenna size assumption of VSAT would generally change the antenna gain, output power in Tx parts and also the referense sensitivity in Rx parts. However, with lack of the specific assumption of this “Ultra small” antenna, the analysis work would be hard to carry out.

Observation 3: It should be clarified first about the detailed “typical assumption” of this “ultra small antenna”, especially on its element gain, element number, Tx power per element and Rx.

The WF[2] also requests to further clarify the assumption for this antenna:

- Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO. - Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna - WF - Consider the existing Ku-band ACLR and ACS as the starting point - FFS on whether to modify. - Encourage companies to provide justifications for whether to re-use the existing specifications.

Another reference we can think of, for the sake of making progress, is that we should focus on the 20x20 which is explicitly mentioned by the WID[2] to progress some of the work. And then, we could refer to one of the contribution in Rel-19 Ku-band discussion as a reference[4], in which there was a proposed antenna configuration for 20x20.

R4-2418031 [4] Table 1 Essential receiver characterisitcs of 20cm x 20cm, 40cm x 40cm, and 60cm x 60cm antennas at boresight operating at 11.8 GHz. 20cm x 20cm; 40cm x 40cm; 60cm x 60cm. Antenna Gain (dBi); 22.3; 28.3; 32.1. Antenna Temp (K); 25; 25; 25. Noise Figure (dB); 3; 3; 3. Noise Figure Temp (K); 288.6; 288.6; 288.6. System Temp (k); 313.6; 313.6; 313.6. G/T (dB/K); -2.7; 3.3; 7.1.

Proposal 2: To consider the following 20x20cm antenna configurations as the starting point for analysis.

|  | 20cm x 20cm |
| --- | --- |
| Antenna Gain (dBi) | 22.3 |
| Antenna Temp (K) | 25 |
| Noise Figure (dB) | 3 |
| Noise Figure Temp (K) | 288.6 |
| System Temp (k) | 313.6 |
| G/T (dB/K) | -2.7 |

Regarding the ACLR and ACS, our initial observation is that given there is clear rule of off-axis eirp limit for NTN VSAT, the leakage in other directions would have minor difference to the actual antenna configurations. It means, even the NTN VSAT has less antenna elements which causes worse side lobe from the Tx beam, the VSAT will have to reduce its power to meet the off-axis e.i.r.p. rule, so in general the interference leakage in spatial domain should be somehow equivalent or even less to the other victim stations.

Observation 4: The off-axis e.i.r.p. limit will request the NTN VSAT to reduce its Tx power to comply. Smaller antenna may cause worse side lobe, but the interference leakage in un-wanted direction would be even further compressed by less Tx power to meet off-axis e.i.r.p limit.

Proposal 3: To re-use the ACLR and ACS for ultra small antenna VSAT in Ku-band, if off-axis e.i.r.p. limits are the same.

Conclusion

In this contribution, we provide following discussion points on HD-FDD VSAT and ultra-small antenna VSAT:

Observation 1: According to WID, we believe the single physical aperture device should be focused or prioritized in this WI.

Observation 2: In general, our observation is that no changes may be brought to RF requirements by simply having the HD-FDD VSAT compared to FD-FDD. As a result, maybe a general description on existing Ku-band VSAT requirements can be introduced to declare the requirements can also apply to HD-FDD VSAT.

Proposal 1: To re-use existing FDD VSAT requirements for HD-FDD Ku-band under FR1 numerology.

Observation 3: It should be clarified first about the detailed “typical assumption” of this “ultra small antenna”, especially on its element gain, element number, Tx power per element and Rx.

Proposal 2: To consider the following 20x20cm antenna configurations as the starting point for analysis.

|  | 20cm x 20cm |
| --- | --- |
| Antenna Gain (dBi) | 22.3 |
| Antenna Temp (K) | 25 |
| Noise Figure (dB) | 3 |
| Noise Figure Temp (K) | 288.6 |
| System Temp (k) | 313.6 |
| G/T (dB/K) | -2.7 |

Observation 4: The off-axis e.i.r.p. limit will request the NTN VSAT to reduce its Tx power to comply. Smaller antenna may cause worse side lobe, but the interference leakage in un-wanted direction would be even further compressed by less Tx power to meet off-axis e.i.r.p limit.

Proposal 3: To re-use the ACLR and ACS for ultra small antenna VSAT in Ku-band, if off-axis e.i.r.p. limits are the same.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2] R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek

[3] R4-2513356, Discussion on HD-FDD VSAT RF requirements, Samsung

[4] R4-2418031, Charactersation of Ku Band Mobile VSAT Antennas for GSO and NGSO Applications, Intelsat, Fraunhofer, BMW.