3GPP TSG-RAN4 Meeting #117	R4-2521428

Dallas, USA, 17 November – 21 November 2025

Source:			Ericsson

Title:			6 MHz channel bandwidth – BS RF impacts

Agenda item:		7.1.5.1

Document for:		Approval

# Introduction

In last RAN#109, a new WI ([1]) has been approved to enhance UE RF requirements for the deployment in NR FR1. One of the objectives of this WI is to specify 6 MHz channel bandwidth for NR bands n5, n8, n12, n85 and n29.

Based on the agreed Way Forward ([3]) from last RAN4#116bis meeting, considering 30 RBs (15kHz SCS) as the maximum transmission bandwidth for 6 MHz channel bandwidth, leading then to a minimum guard band of 292.5 kHz for this channel bandwidth, this contribution is discussing the remaining open issues related to BS RF requirements.

# Discussion

## Further detailed analysis

#### ACS

Proposal 1: Specify the ACS interferer frequency offset with ±2.5125 MHz value.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |
| --- | --- |
| 6 | ±2.5125 |

#### Narrow band blocking

Proposal 2: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±360 (+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |

#### Intermodulation

Proposal 3: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 6 | ±7.475 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

#### Narrowband Intermodulation

Proposal 4: Specify the narrowband intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±425 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

#### In-channel selectivity

Instead of specifying a new FRC to specify ICS for 6 MHz channel bandwidth, we would propose to specify the ICS requirement based on the requirement for 7 MHz channel bandwidth.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 6, 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |

Proposal 5: Specify the In-channel selectivity requirements for 6 MHz channel BW based on the requirements for 7 MHz channel BW.

# Conclusion

In this contribution, we discussed the remaining open issues related to BS RF when introducing 6 MHz channel bandwidth. We made the following observations and proposals:

Proposal 1: Specify the ACS interferer frequency offset with ±2.5125 MHz value.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) |
| --- | --- |
| 6 | ±2.5125 |

Proposal 2: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±360 (+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |

Proposal 3: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 6 | ±7.475 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Proposal 4: Specify the narrowband intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±425 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

Proposal 5: Specify the In-channel selectivity requirements for 6 MHz channel BW based on the requirements for 7 MHz channel BW.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 6, 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |

# References

- RP-252901, UE RF enhancements for NR FR1, Phase 5,

- R4-2513834, 6 MHz channel bandwidth – system parameters and UE RF impacts, Ericsson

- R4-2515084, Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm