3GPP TSG-RAN4 Meeting #117	R4-2521431

Dallas, USA, 17 November – 21 November 2025

Source:			Ericsson

Title:			US 4.9 GHz band – UE RF aspects

Agenda item:		5.11.4

Document for:		Approval

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US.

Based on the Way Forward ([3]) agreed in last RAN4#116bis meeting, this contribution is discussing the remaining UE RF open issues related to the introduction of the US 4.9GHz band.

# Discussion

## Background

In last RAN4#116bis meeting, RAN4 agreed ([3]) on the following UE RF aspects:

- Maximum output power limit per channel bandwidth

- Peak PSD of 21 dBm/Mhz.

- Adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth.

- Specify a new NS for the US 4.9GHz band with FCC part 90 including:

  - The Mask M requirement (note this covers the 5% channel bandwidth edge (inside), the SEM domain and the spurious domain).

  - Maximum output power limitations mentioned above.

Companies were encouraged to provide the A-MPR simulation assumptions.

## A-MPR simulation assumptions

For the A-MPR simulations, we propose to reuse the following usual assumptions:

- Power Class 3 (if other UE power classes are not included in the WID revision)

- Calibration: 1dB MPR with DFT-s-OFDM QPSK 20MHz 15kHz SCS, L_CRB=100 and Start_RB=0

- Carrier Leakage: -28 dBc

- I/Q image: -28 dBc

- CIM3: -60 dBc

- Frequency range: 4940 – 4990 MHz

- SCS: 30 kHz

- Waveform: DFT-s-OFDM and CP-OFDM

- Modulation: QPSK, 16QAM, 64QAM, 256QAM

And consider the following FCC requirements:

- FCC emission mask M (§90.210)

- Peak PSD limit of 21dBm / MHz

For the peak PSD limit, we propose to consider 1 dB tolerance, as it was suggested in last RAN4#116bis meeting ([4]).

Proposal 1: Consider the above A-MPR assumptions to evaluate FCC mask M (§90.210) and the PSD limit of 21dBm/MHz (assuming 1dB tolerance).

## High Power UEs

Even if UE power classes 2, 1.5 and 1 are not in the scope of the WID, RAN4 had some discussion on these UE power classes in RAN4#116bis. It was indeed assumed that those UE power classes could be added later in the scope of the usual basket WI, once the 4.9 GHz band has been specified. But, from the past RAN4 discussion, this might be not that obvious.

FCC indeed specified a maximum output power per channel bandwidth (§90.1215), as shown in Table 1

Table 1: Maximum output power per channel bandwidth - §90.1215(a)(1)

Comparing with the UE maximum output power for each power class, all UE power classes might not be able to support all channel bandwidths considered for the 4.9GHz band, as shown in Table 2.

Table 2: UE power class and problematic channel bandwidth(s)

| Power Class | Maximum output power (dBm) | Channel bandwidths which might not be supported (MHz) |
| --- | --- | --- |
| PC2 | 26 + 2dB tolerance | 1, 5 |
| PC1.5 | 29+ 2dB tolerance | 1, 5, 10 |
| PC1 | 31+ 2dB tolerance | 1, 5, 10, 15 |
| Note: the channel bandwidths in red are supposed to be supported in the new US 4.9 GHz band. |  |  |

Observation 1: Considering the maximum output power per channel bandwidth (§90.1215(a)(1), some requested channel bandwidths can not be supported by all UE power classes at maximum output power.

It should be noted that the above observation is made by only looking at the maximum output power per channel bandwidth as specified by FCC (§90.1215).

When doing A-MPR simulations, considering the FCC M-mask and the 21dBm/MHz PSD limit, we should expect that additional channel bandwidth(s) might not be supported at UE power class  maximum output power. Due to the short time between RAN4 meetings, we were not able to run A-MPR simulation to confirm this, but this was already observed in [4].

Observation 2: When doing A-MPR simulations considering the FCC M-mask and the 21dBm/MHz PSD limit additional requested channel bandwidths might not be supported by all UE power classes at maximum output power.

In last RAN4#116bis meeting, RAN4 agreed ([3]) that “The zero dB reference for FCC Emission mask M is calculated based on UE power class maximum output power and the configured UE channel bandwidth”.

With the observations made previously, this statement would not stand anymore for certain UE power classes and certain channel bandwidths, the zero dB reference could not be anymore at UE power class maximum power.

To address this issue, a list of channel bandwidths per UE power class should not be allowed in the 4.9 GHz band. This list of “forbidden” channel bandwidths could be defined based on A-MPR simulation results, looking at the channel bandwidths/UE power class for which there is no region with 0 dB A-MPR.

Proposal 2: The list of channel bandwidth(s) disallowed for a certain UE power class shall be built based on A-MPR simulation results, considering the channel bandwidth for which there is no region without any A-MPR.

As this situation never happened before, RAN4 should then agree on a mechanism to disallow certain channel bandwidth per UE power class. And this should preferably be discussed then in the scope of the 4.9 GHz band WI as such update might not fit with the scope of the High-Power UE basket WI. But to do so, the 4.9 GHz WI should be updated adding at least one additional power class (PC2, PC1.5 or PC1) to the scope.

Proposal 3: Update the 4.9GHz WI adding at least one UE higher power class (PC2, PC1.5 or PC1) so that RAN4 could define a mechanism to disallow some channel bandwidth(s) depending on UE power class.

# Conclusion

In this contribution, we discussed the remaining UE RF open issues when introducing the new US 4.9 GHz band and made the following observations and proposals related to the UE RF impacts:

Proposal 1: Consider the above A-MPR assumptions to evaluate FCC mask M (§90.210) and the PSD limit of 21dBm/MHz (assuming 1dB tolerance).

Observation 1: Considering the maximum output power per channel bandwidth (§90.1215(a)(1), some requested channel bandwidths can not be supported by all UE power classes at maximum output power.

Observation 2: When doing A-MPR simulations considering the FCC M-mask and the 21dBm/MHz PSD limit additional requested channel bandwidths might not be supported by all UE power classes at maximum output power.

Proposal 2: The list of channel bandwidth(s) disallowed for a certain UE power class shall be built based on A-MPR simulation results, considering the channel bandwidth for which there is no region without any A-MPR.

Proposal 3: Update the 4.9GHz WI adding at least one UE higher power class (PC2, PC1.5 or PC1) so that RAN4 could define a mechanism to disallow some channel bandwidth(s) depending on UE power class.

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Eight Report and Order, WP Docket No. 07-100, 22 October 2024.

- R4-2515103, Way Forward for [116bis][322] NR_TDD_4.9GHz_band, Fujitsu

- R4-2514507, On US 4.9 GHz UE RF requirements, Qualcomm Incorporated