3GPP TSG-RAN4 Meeting #117	R4-2521435

Dallas, US, November 17th – 21st 2025

Source:			Ericsson

Title:			BS RF considerations for 6G

Agenda item:		8.4.1

Document for:		Discussion

# Introduction

RAN4 started the discussion related to 6G BS RF aspects and 6G co-existence evaluation aspects in last RAN4#116bis meeting and a Way Forward ([2]) with initial agreements was agreed.

In this contribution, we discuss the BS RF aspects to be considered within the scope of RAN4 6G SI, based on this agreed Way Forward [2]).

# Discussion

## 2.1	6G BS RF requirements

### 2.1.1	Requirements to be re-assessed

In the Way Forward agreed in last RAN4#116bis meeting ([3]), companies were encouraged to share their view on 5G BS RF requirements, indicating if they should be:

- Reused for 6G as specified for 5G.

- Re-evaluated based on the 6G system parameters agreements, but no further study would be needed, the re-evaluation could be done based on TR 38.817-02 for example.

- Re-studied for 6G, considering or not a new methodology to define them. For those requirements, companies should also provide the rationale motivating their proposal.

Following this WF, the table in Annex A shares our view on 5G BS RF requirements and what should be study in the scope of the 6G SI.

Proposal 1: Agree with table in Annex A and the list of RF requirements to be studied in the scope of the 6G SI.

### 2.1.2	BS output power

With 5G and AAS BS, RAN4 introduced many instantiations of the BS maximum output power (15 symbols have been defined in total, i.e. maximum carrier output power measured per antenna connector, maximum carrier output power per TAB connector TX min cell group, maximum carrier output power per TAB connector, Maximum carrier TRP output power measured at the RIB, ...). While each definition has its own rationale and justified usage when specifying requirements, it would be worth, in the scope of this 6G study, to further study if it would be possible to simplify how power is handled in RAN4 specification, eventually considering PSD based requirements instead when relevant.

Proposal 2: Re-visit how BS output power is handled in relevant TS 38.104 requirements and consider introducing PSD based requirements instead.

### 2.1.3	In-band and out-of-band blocking

Also, other requirements were derived for UTRA have been re-used for 4G and 5G. For 6G it be worth to re-evaluate in-band blocking requirement based on for 6G relevant deployment assumptions. For bands in the upper region of FR1 it is expected that AAS BS with large antenna arrays will dominate. For BS with large array, a single receiver branch will see a fraction of the interferer signal seen by a non-AAS with a single column passive antenna. Therefore, RAN4 need to re-evaluate receiver in-band blocking requirement levels for BS operating in the frequency region where large array antennas are expected to be used.

For BS-type 1-O, out-of-band blocking requirement is defined as a field strength level applicable for all bands. Considering AAS BS with large antenna arrays, depending on frequency and array geometry the relation between in-band blocking and out-of-band blocking requirement levels may need further considerations.

Proposal 3: When re-evaluate some requirements (e.g. in-band blocking, out-of-band blocking) assuming today’s actual deployments instead of assumptions and methodology from 1999 and UTRA requirement derivation.

## 2.2	BS RF timing requirements

### 2.2.1	Transmitter transient period

In Ericsson’s RRM contribution for 6G SI [1] we pointed out that the RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period

It also needs to accommodate sufficient time for TX<=>RX switching. This means that , Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

In existing NR specification (TS 38.133, Table 7.1.2-2) there are several coexistence cases listed, since the RRM requirement NTA offset affects UL timing. There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 1: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Furthermore, in the same RRM contribution from Ericsson [1] we also observe that a large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

As highlighted above, we consider robustness, deployment flexibility, and the ability to leverage existing infrastructure to be more important than marginal TDD overhead reductions. Therefore, we propose to maintain the TDD Cell Phase Synchronization requirement at the same level as in NR NR, i.e., 3 μs. Similarly, we believe that synchronization enhancements are better utilized to improve overall system robustness rather than to achieve the aforementioned minor TDD overhead reductions.

Proposal 4: Keep TDD Cell Phase Synchronization requirement the same as in NR.

Keeping the same as in NR due to various configuration and inter RAT co-existence dependencies which in NR FR1 and FR2 and proposing that TDD Cell Phase Synchronization requirement the same as in NR leads to the conclusion that:

Proposal 5: Transmitter transient period can be the same in NR for FR1 and FR2 and no impact is expected in BS RF requirement analysis.

### 2.2.2	TAE

In Ericsson’s BS RF and co-existence considerations for 6G contribution, [2] we consider whether the synchronization requirements of a feature or function really need to be specified in the standard. Often, a sync requirement cannot be adequately captured by a single figure, as there generally are complex and varying input parameters associated with it. In the worst-case scenario, specifying a value based on the most challenging dimensioning can lead to increased costs and complexity, which will not be necessary in most scenarios. If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Let us discuss an important example. In Figure 1, we know that:

RTD= TAE+ ∆RF_Propagation+Channel_dispersion

If the RAN4 requirement is specified as Maximum Receive Time difference, MRTD then it is possible to trade TAE and ∆RF_Propagation, within a total budget.

Figure 1: Receive time difference example

Proposal 6: If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system., like defining MRTD in RRM and avoid stating TAE between ARP.

Moreover, TAE is strongly connected to which preconditions that are assumed for a feature (deployment, radio channel characteristics) as well as UE RF and baseband assumptions (Common LNA or not, common FFT or not).

Proposal 7: TAE would need to be studied in the 6G SI Scope.

# Conclusion

In this contribution, we discussed the 6G BS RF aspects to be considered in the in the scope of the 6G SI. We made the following observations and proposals:

Proposal 1: Agree with table in Annex A and the list of RF requirements to be studied in the scope of the 6G SI.

Proposal 2: Re-visit how BS output power is handled in relevant TS 38.104 requirements and consider introducing PSD based requirements instead.

Proposal 3: When re-evaluate some requirements (e.g. in-band blocking, out-of-band blocking) assuming today’s actual deployments instead of assumptions and methodology from 1999 and UTRA requirement derivation.

Observation 1: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Proposal 4: Keep TDD Cell Phase Synchronization requirement the same as in NR.

Proposal 5: Transmitter transient period can be the same in NR for FR1 and FR2 and no impact is expected in BS RF requirement analysis.

Proposal 6: If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system., like defining MRTD in RRM and avoid stating TAE between ARP.

Proposal 7: TAE would need to be studied in the 6G SI Scope.

# References

- R4-251333, Discussion on 6G RRM requirements, Ericsson

- R4-2513339, BS RF and co-existence considerations for 6G, Ericsson

- R4-2514638, WF on 6G BS RF and coexistence, Feature Lead (Ericsson)

# Annex A – 5G BS RF requirements analysis

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power |  |  | X , see rationale in 2.1.2 |
| Output power dynamics |  |  |  |
| RE power control dynamic range | X |  |  |
| Total power dynamic range | X |  |  |
| Transmit ON/OFF Power |  |  |  |
| Transmitter OFF power |  |  | On hold, pending on Rel-19/20 conclusions. |
| Transmitter transient period | X |  |  |
| Transmitted signal quality |  |  |  |
| Frequency error | X |  |  |
| Modulation quality | X |  |  |
| Time alignment error |  |  | X see rationale in 2.2.2 |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X |  |  |
| ACLR | X + Reuse Rel-19 IMT |  |  |
| Operating Band Unwanted Emissions | X |  |  |
| Transmitter spurious emission |  |  |  |
| General requirement | X |  |  |
| Protection of BS receiver of own or different BS | X |  |  |
| Additional spurious (coex) |  |  | On hold, pending on Rel-19/20 conclusions. |
| Co-location |  |  | On hold, pending on Rel-19/20 conclusions. |
| Transmitter intermodulation |  |  | On hold, pending on Rel-19/20 conclusions. |
| Receiver sensitivity level |  | X |  |
| Dynamic range |  | X |  |
| In-band selectivity and blocking |  |  |  |
| ACS | X + Reuse Rel-19 IMT |  |  |
| In-band blocking |  |  | X see rationale in 2.1.3 |
| Out of band blocking |  |  | X see rationale in 2.1.3 |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  | X |  |
| In channel selectivity |  | X |  |
| Radiated requirements |  |  |  |
| Radiated transmit power |  |  | X see rationale in 2.1.2 |
| OTA base station output power |  |  | X see rationale in 2.1.2 |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range | X |  |  |
| OTA Total power dynamic range | X |  |  |
| OTA transmit ON/OFF |  |  |  |
| OTA transmitter OFF power |  |  | On hold, pending on Rel-19/20 conclusions. |
| OTA transmitter transient period | X |  |  |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error | X |  |  |
| OTA Modulation quality | X |  |  |
| OTA Time alignment error |  |  | X see rationale in 2.2.2 |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X |  |  |
| OTA ACLR | X + Reuse Rel-19 IMT |  |  |
| OTA Operating Band Unwanted Emissions | X |  |  |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X |  |  |
| OTA Protection of BS receiver of own or different BS | X |  |  |
| OTA Additional spurious (coex) |  |  | On hold, pending on Rel-19/20 conclusions. |
| OTA Co-location |  |  | On hold, pending on Rel-19/20 conclusions. |
| OTA transmitter intermodulation |  |  | On hold, pending on Rel-19/20 conclusions. |
| OTA sensitivity |  | X |  |
| OTA receiver sensitivity level |  | X |  |
| OTA dynamic range |  | X |  |
| OTA in-band selectivity and blocking |  |  |  |
| OTA ACS | X + Reuse Rel-19 IMT |  |  |
| OTA In-band blocking |  |  | X see rationale in 2.1.3 |
| OTA out of band blocking |  |  | X see rationale in 2.1.3 |
| OTA receiver spurious emissions | X |  |  |
| OTA receiver intermodulation |  | X |  |
| OTA in channel selectivity |  | X |  |