3GPP TSG-RAN4 Meeting #117	R4-2521436

Dallas, US, November 17th – 21st 2025

Source:			Ericsson

Title:			6G NTN SAN and coexistence aspects

Agenda item:		8.4.2

Document for:		Approval

# Introduction

RAN4 started the discussion related to 6G BS RF aspects and 6G co-existence evaluation aspects in last RAN4#116bis meeting and a Way Forward ([2]) with initial agreements  was agreed.

In this contribution, we discuss the NTN SAN and coexistence aspects to be considered within the scope of RAN4 6G SI.

# Discussion

## NTN-TN coexistence L-/S- band – isolation distance

When RAN4 studied the S-band, an isolation distance of 2*ISD (1500m) was considered in scenario 1 (TN DL aggressor, NTN DL victim) to facilitate the coexistence and derive NTN UE ACS value.

The value of this isolation distance was a very rough estimation, decided while doing the coexistence study, under the time pressure to finalize this WI on time. And, once RAN4 agreed on the final NTN UE ACS value, RAN4 didn’t have time to check what would be the minimum isolation distance required for the agreed ACS value.

In the scope of the 6G SI, to get a more accurate evaluation of NTN-TN coexistence in the L-/S-bands, it would then make sense to evaluate what would be this minimum isolation distance.

Proposal 1: Evaluate the minimum isolation distance for NTN and TN to coexist in the L-/S- band for the agreed NTN UE ACS value (scenario 1 of the coexistence study, TR 38.863).

## MEO satellite

So far, RAN4 only considered the following type of satellite: GEO, LEO at 1200km and LEO at 600 km. All requirements (SAN RF and UE RF) have been specified considering those types of satellite only.

In the scope of the IRIS2, as shared by ESA (XX), a mixed constellation of LEO and MEO satellites will be deployed, suing the Ku- and Ka-bands. It seems then obvious that RAN4 should early study the MEO type of satellite, doing a coexistence study and evaluating the SAN and VSAT RF requirements impacts.

Proposal 2: Study the MEO type of satellite, doing a coexistence study and evaluating SAN/VSAT RF requirements impacts.

# Conclusion

In this contribution, we discussed the 6G NTN aspects to be considered in the in the scope of the 6G SI. We made the following proposals:

Proposal 1: Evaluate the minimum isolation distance for NTN and TN to coexist in the L-/S- band for the agreed NTN UE ACS value (scenario 1 of the coexistence study, TR 38.863).

Proposal 2: Study the MEO type of satellite, doing a coexistence study and evaluating SAN/VSAT RF requirements impacts.

# References

- R4-2513339, BS RF and co-existence considerations for 6G, Ericsson

- R4-2514638, WF on 6G BS RF and coexistence, Feature Lead (Ericsson)