3GPP TSG RAN WG4 Meeting #117	R4-2521440

Dallas, USA, Nov. 17-21, 2025

Agenda item:			8.8.2

Source:	vivo

Title:	Initial discussion on 6G AI for RAN4 driven cases

Document for:	Discussion

- Introduction

The RAN WG study on 6G radio has been approved by RAN#108 as SID in [1]. The AI-relevant objective of RAN4 is copied below for reference.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

Also, as indicated in [2], multiple topics are provided for initial 6G RAN4 discussion from Q4 2025 to Q1 2026. AI, which has been previously explored in AI/ML PHY and AI/ML mobility study, is one of the topics for initial 6G discussion.

At the last meeting, RAN4 conducted preliminary discussions on the General Scope and Timeline, AI/ML Framework, and RAN4-driven use cases. The progress of these discussions was captured in WF [3]. In accordance with the agreed conclusions on the RAN4 timeline for the Rel-20 6G AI/ML study, this meeting will further clarify the study scope for the RAN4 AI/ML Framework, select the top-prioritized RAN4-led AI/ML use cases, and finalize a detailed evaluation plan for Stage-2.

In this contribution, we will provide further detailed analysis of both the use cases summarized at the last meeting and those proposed by us.

- Discussion

  - AI/ML use case for RF issues

Coverage enhancement and energy efficiency are key objectives for 6G systems. The introduction of uplink wide bandwidths up to 200/400 MHz, however, poses significant challenges for PA design, as severe nonlinearity may lead to a reduction in maximum transmit power and power efficiency. AI/ML-based PA nonlinearity compensation is regarded as a promising use case to address these challenges. This section provides an analysis of potential AI/ML solutions, including both DPD and DPoD.

    - PA nonlinearity compensation based on AI DPD

      - Label acquisition

Figure 1 presents three candidate architectures for implementing DPD with AI assistance. Key information is summarized in table1.

In Direction 1, the strategy is to train an AI/ML-based DPD model directly and deploy it for baseband signal pre-distortion. A key challenge is ensuring the model’s generalizability under diverse PA responses affected by factors like Vcc, temperature, and impedance. To address this without excessively increasing model complexity, these critical parameters can be fed as inputs to the AI model during training and inference.

Direction 2 aims to generate LUTs dynamically, triggered by the UE monitoring LUT performance degradation. The AI model in this scheme is supplied with time-domain samples captured by the FBRx. The resulting DPD solution maintains a computational complexity identical to that of a conventional LUT.

Direction 3 utilizes AI to construct a precise behavioral model of the PA, which subsequently facilitates LUT generation and updating. A significant potential advantage of this direction is that the AI’s predictive capability may reduce the dependency on a wide-bandwidth FBRx. Furthermore, the operational complexity of the DPD remains on par with a LUT-based approach.

In direction 1 and 2, the training label is the undistorted time-domain signal, generated locally on the UE which seems to be UE implementation. The core challenge is that training a single, generalizable AI model requires the PA output signal (collected via FBRx) to encompass a wide range of operating conditions. Consequently, this demand for diverse dataset affects key uplink transmission parameters like power, PAPR, and bandwidth. In Direction 3, the training label is the PA output signal, which is acquired via the FBRx path.

Direction 1: AI-based DPD

Direction 2: AI-based LUT DPD update

Direction 3: AI based PA modelling to generate and update LUT

Figure 1 Three candidate DPD architectures with AI assistance

Table 1 AI/ML-assisted dynamic DPD solutions

|  |  | Direction 1 | Direction 2 | Direction 3 |
| --- | --- | --- | --- | --- |
| AI model input | Input in training | Time domain samples after PA distortion and PA config e.g. Vcc etc. |  |  |
|  | Input in inference | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples after PA distortion and PA config e.g. Vcc etc. | Time domain samples before PA distortion and PA config e.g. Vcc etc. |
| AI model output | Label in training (if applicable) | Time domain samples w/o distortion |  |  |
|  | Output in inference | Time domain samples after pre-distortion | LUT coefficients | Time domain samples after PA distortion |
| Assumption on training | Training type | Offline training；Online finetune |  |  |
|  | Label construction (if applicable) | Time domain samples w/o distortion |  |  |
| model location for inference |  | UE-sided |  |  |
| Collaboration/interaction between UE and NW |  | As for UE side model |  |  |
| Evaluation methodology |  | link level simulation |  |  |
| Evaluation assumption |  | Model PA model; Parameter. yGMPn= K∈Kal∈Laaklxn-lxn-l2k+K∈Kbl∈Lbm∈Mbklmxn-lxn-m2k; akl;k∈[0,1,…,7]: l=2: [0.0145707+0.00223568i; 0.0166021+0.0884597i; -0.170987-0.889998i; 0.398012+4.25717i; -0.922915-11.5296i; 1.51648+16.8822i; -1.31708-12.4992i; 0.443603+3.66282i] l=1: [-0.0730384-0.0608598i; 0.316437-0.130488i; -2.64289+1.95766i; 13.9617-8.92706i; -35.9884+25.271i; 49.5323-38.9777i; -34.8388+30.1032i; 9.83576-9.12289i] l=0: [-0.369392-0.616894i; 0.582141-1.54129i; -4.2332+13.9746i; 15.4346-56.4738i; -34.026+106.817i; 42.3779-83.2642i; -26.6004+5.86237i; 6.4982+15.2082i] l=-1: [-0.109009-0.0382752i; 1.34619-0.303139i; -7.57533+2.07457i; 30.8214-7.83883i; -71.9119+13.7515i; 94.7172-10.8742i; -65.1891+3.05573i; 18.1882+0.14561i] l=-2: [0.0913878+0.029207i; -0.205695-0.0047561i; 0.436792+0.098933i; -0.0447736+0.802472i; -1.91069-3.64271i; 3.53201+6.10853i; -2.64467-4.81807i; 0.741402+1.46945i] bklm;k∈[0,1,…,7]: l=1, m=0: [-0.0732748-0.0617029i; 1.04861+0.216692i; -7.53774-2.85579i; 29.348+11.8762i; -68.0727-19.8783i; 92.0079+4.93057i; -66.4247+17.5978i; 19.6384-12.2782i] l=-1, m=0: [-0.108885-0.0392921i; -0.65351-0.122316i; 2.7747+3.26333i; -6.41902-23.391i; 9.68476+79.745i; -10.5191-141.613i; 6.89414+125.231i; -1.92908-43.441i]. |
| Evaluation KPI |  | BLER, EVM, MPR |  |  |
| Evaluation benchmark |  | No DPD |  |  |
| preliminary evaluation results |  | See 2.1.1.2 |  |  |
| High level potential specification impact |  | See 2.1.1.3 |  |  |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Inference complexity |  |  |
|  |  | Depend on AI model complexity | similar as LUT-based DPD | similar as LUT-based DPD |

      - Key advantages over non-AI solution

AI/ML-based DPD solution delivers significant performance gains than non-AI solution. Table 2 shows that for a given PA, AI approach provides a 3 dB greater ACLR reduction compared to traditional non-AI DPD Directions. In addition to offering superior performance in compensating for PA nonlinearity, AI/ML also holds significant potential for jointly mitigating other hardware impairments, such as IQ imbalance.

Table 2 Comparison between AI-based DPD and non-AI DPD, one PA config

| Schemes | NMSE | ACLR left | ACLR right |
| --- | --- | --- | --- |
| Wo DPD | 4.69dB | -37.31dB | -35.55dB |
| LUT DPD | -37.88dB | -42.72dB | -45.03dB |
| AI-based DPD | -44.57dB | -46.55dB | -47.78dB |

As mentioned earlier, AI can accurately model PA behaviour under various conditions by using these variations as inputs. This means that, assuming equivalent complexity and memory capabilities, the AI model can cover more scenarios with high precision. As shown in Table 3, the AI DPD model, trained on a dataset encompassing 13 PA configurations, achieves an NMSE of -31.65 dB between the PA output and the ideal signal using only 587 coefficients. In contrast, while a LUT-based DPD can achieve comparable performance by storing 13 coefficient sets for these conditions, its total number of coefficients is more than double that of the AI-based solution.

Table 3 Comparison between AI-based DPD and non-AI DPD, 13 PA configs

|  | NMSE | Num. of Coefficients | Flops |
| --- | --- | --- | --- |
| LUT-based DPD | -31 dB | 1456 | 1568 |
| AI DPD | -31.65 dB | 587 | 1172 |

AI/ML offers a distinct advantage in predicting PA nonlinearities like third and fifth-order IMD. While the conventional approach demands an FBRx bandwidth roughly five times the baseband width, which is a major challenge for cost-sensitive UEs. The AI-based approach, as shown in Figure 2, accurately fits the PA model with just 1.5 times the bandwidth, limiting ACLR error to 0.2 dB.

Figure 2 ACLR measured by different PA model

Observation 1: AI demonstrates significant potential for accurate DPD modelling while maintaining lower computational complexity and hardware cost.

      - Spec impact analysis

The following three aspects must be taken into account when acquiring the dataset for model training and monitoring:

- Antenna effect: The performance of the DPD is affected by the specific antenna characteristics of the UE. Therefore, during dataset acquisition, the signal must be transmitted over the air to incorporate antenna impedance effects. This approach, however, results in radiated power, which may cause interference to other UEs if the calibration procedure is not properly regulated.

- Calibration occasion: DPD calibration can be initiated by the UE at any time based on its internal performance monitoring. Without a well-defined data collection procedure, such UE-triggered calibration may conflict with network-scheduled signaling, leading to potential scheduling collisions.

- Calibration cost: The need to calibrate the DPD for each individual PA introduces significant hardware cost. Moreover, when one FBRx is shared across multiple antenna ports, the network may need to schedule single port transmission specifically for DPD calibration.

Two options can be considered for data collection

- Option1: gap-based data collection

- Option2: SRS and/or PUSCH based on data collection

Option 1 requires the network to define calibration gaps for UEs. Within these gaps, UEs are permitted to transmit unspecified signals for data collection, even if the transmissions temporarily violate RF requirements like IBE or EVM. Although a common gap can be pre-configured across all UEs, this approach consequently imposes network scheduling constraints and results in uplink throughput deterioration.

In Option 2, the UE can collect data directly from its formal uplink transmissions including PUSCH and SRS, thereby imposing no additional scheduling restrictions on the network. However, the characteristics of these transmission signals, specifically, the output power and PAPR, significantly impact the performance of the resulting model.

As illustrated in Figure 3, the ACLR performance of a PA model is highly dependent on the power level of the data used for training. A model fitted with data collected at 23 dBm achieves near-perfect ACLR, whereas using data from 20 dBm outputs introduces an ACLR error of up to 2 dB. Similarly, Figure 4 demonstrates the sensitivity to PAPR. A dataset with a PAPR below 9.5 dB yields an excellently fitted model. In contrast, when the PAPR ceiling is reduced to just 8 dB, a difference of only 1.5 dB, the model exhibits a substantial ACLR error of up to 5 dB.

Figure 3 ACLR of PA Models Fitted Under Different Power Assumptions

Figure 4 ACLR of PA models Fitted Under Different Power Assumptions

Observation 2: The output power and PAPR of UL signals for data collection significantly impact the performance of the resulting model.

To summarize, the following potential specification impacts need to be considered to support data collection based on SRS and/or PU

- Scheduling preference due to DPD calibration including transmission power, bandwidth and number of transmission ports/layers

- Potential requirement for SRS design (e.g. large PAPR signal and port multiplexing)

- Requirement (e.g. maximum power, IBE) for calibration signal

    - PA nonlinearity compensation based on AI DPoD

      - Label acquisition

Figure 5 illustrates three potential AI/ML-based DPoD solutions designed to compensate for PA nonlinearity at the network side.

- Direction 1 follows a structure similar to traditional DPoD, aiming to train a DPoD model using the DMRS sequence as the reference label which is already known to the network.

- Direction 2 seeks to mitigate PA nonlinearity by training an AI-based de-mapper. The label in this approach is the transmitted bit stream, which must be explicitly signaled to the network.

- Direction 3 employs a standalone DPoD model to correct PA nonlinearity, using the successively decoded signal as the label.

In Direction 1, performing online model training for each UL transmission occasion enables effective compensation of PA nonlinearity under varying conditions. However, it is important to note that conducting such online training for every uplink transmission consumes significant computational resources and increases system complexity, particularly when a single TRP serves numerous UEs. In contrast, Directions 2 and 3 aim to train a generalized model applicable to all UEs, each with distinct PA nonlinearity characteristics. This approach must also accommodate the nonlinearity variations of a single UE under different operating conditions, which poses a significant challenge for collecting a sufficiently diverse dataset from numerous UEs to train this large model effectively.

Direction 1: AI/ML-based DPoD architecture based on DMRS

Direction 2: AI/ML-based De-mapper architecture based on UL data

Direction 3: AI/ML-based DPoD architecture based on successively decoded UL data

Figure 5 Three candidate AI/ML DPoD solutions

      - Spec impact analysis

In Direction 1, the label is the DMRS sequence, which is already known at the network side, thus avoiding any additional signalling overhead. However, the performance of the trained model is influenced by the characteristics of the transmitted signal, particularly its PAPR. For example, in CP-OFDM, the DMRS sequence exhibits a PAPR similar to that of data symbols. However, in DFT-s-OFDM, the DMRS PAPR is typically similar to QPSK which is significantly lower than that of 256QAM data. This discrepancy means that if the PA operates at a lower power region during DMRS transmission, the model trained on DMRS will not accurately capture the nonlinearity experienced by high-PAPR 256QAM data symbols, leading to performance degradation during inference. To address this, the DMRS sequence should be enhanced to emulate the 256QAM PAPR level. Additionally, the EVM requirement for the transmitted signal could be relaxed, allowing the UE to increase transmit power for 256QAM and better characterize the PA’s nonlinear region. Signalling/ procedure related to LCM for NW-sided model is also needed.

Despite the shared use of EVM relaxation and LCM procedures, the data collection mechanisms differ between Direction 2 and Direction 3. Direction 2 requires that the transmitted bit information be explicitly signaled to the network. In contrast, Direction 3 relies on the network scheduling uplink transmissions that use a low code rate to ensure successful decoding, thereby enabling the collection of usable data. Transmission power of Direction 2 and 3 should be specified to align data collection occasion.

    - Summary

Table 4 summarizes performance and model complexity of AI/ML-based DPD and DPOD. AI/ML-based DPD achieves 1 dB net gain for QPSK/16QAM and 64 QAM while about 3dB for 256QAM. AI DPoD has about 3dB gain at 256 QAM. AI/ML-based DPoD introduces computational complexity similar to DPD. Under typical operating assumptions (30,720 samples/slot, 500 inferences/sec), this results in a workload of 0.62 Mops. At a power efficiency of 10 GOPS/mW, the corresponding power consumption is approximately 2 mW.

Table 4 Performance of candidate AI/ML-based DPD and DPoD to compensate PA non-linearity

|  | AI DPD | AI DPoD |
| --- | --- | --- |
| Net gain | 1dB for QPSK, 16QAM, 64QAM 3 dB for 256 QAM | No gain for low modulation order 3 dB for 256QAM |
| Complexity | 0.62 Mops | 0.62 Mops |
| Power consumption | 2 mW | N.A. (NW assumption) |

Observation 3: Both AI/ML-based DPD and DPoD demonstrate promising potential in compensating for PA non-linearity with similar complexity.

In study phase, it is better to keep both AI DPD and DPoD in the scope as the candidate solution for PA non-linearity compensation. After comprehensive study and analysis, we can deeply understand the Pros & Cons for each solution, then it will be a better time to make final decision.

Proposal 1: RAN4 study AI based solution for PA non-linearity compensation in 6G, including both AI DPD and AI DPoD.

In [3], the evaluation procedure for AI DPoD was agreed, as shown in below:

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

However, both DPD and DPoD solve same problem, i.e., PA non-linearity compensation, and most of steps above are also applicable for AI DPD, so we prefer update the procedure and capture both DPD and DPoD.

Proposal 2: Update the evaluation procedure to capture both AI DPD and AI DPoD, as listed below:

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - For both AI DPD and AI DPoD

        - FFS suitable AI/ML model, type of training.

        - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD and DPD performance and ensuring comparability across different implementations

        - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models), measurement-based datasets (from multiple UEs).

      - For AI DPD

        - All modulation orders are expected to be evaluated

        - FFS on detailed baseline assumption for UE requirements, including ACLR, SEM, IBE, EVM, etc.

      - For AI DPoD

        - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

        - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For AI DPoD, under a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. For AI DPD, under a given baseline assumption of UE emission requirements, conduct TX MPR and power boosting simulation.

    - The potential MPR reduction and power boosting enabled by AI-DPoD /AI DPD are evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The potential FBR bandwidth reduction of AI DPD can also be considered.

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

  - AI/ML use case for RRM issues

Regarding AI/ML use cases for RRM, various companies proposed numerous use cases at the last meeting, which were summarized in WF [3]. Next, we will conduct further elaboration and analysis on the use cases.

    - Dynamic Adaptation of Measurement Procedure

At the last meeting, we proposed the use case of "AI-based Optimization for Neighbor Cell Measurement Initiation Mechanism", which was numbered as the 11th use case in the summary. Since this case also uses AI prediction to adjust and optimize the measurement procedure (i.e., the initiation of measurement), it can be categorized under "Dynamic Adaptation of Measurement Procedure" for discussion. Next, we will elaborate on this case in detail.

In current NR systems, the s-measure mechanism acts as the key trigger for neighbour cell measurements in connected mode. It works by having the UE compare real-time measurements of the serving cell's RSRP against a fixed configured measurement threshold. UE will start measurements when RSRP drops below the threshold and stopping them when it rises above. While widely used, this reliance on real-time signals alone makes it less adaptable to the varied, dynamic scenarios of emerging 6G networks.

The traditional s-measure mechanism relies solely on real-time signal quality to trigger measurements. This leads to several key inefficiencies in practical use:

- For fast-moving UEs or those at cell edges, serving cell signals can degrade quickly. The traditional mechanism only starts measurements after RSRP of serving cell falls below the threshold, leaving insufficient time to find a suitable target cell.

- Temporary interference (like signal blockages from moving objects) can cause RSRP to dip briefly below the threshold. The traditional mechanism responds with continuous measurements. It may cause frequent, unnecessary measurement ("ping-pong measurements").

To address these limitations, an enhanced measurement approach combines the traditional s-measure mechanism (idle/inactive cell reselection measurement is similar and can also be considered) with AI prediction. The core idea is to use AI models to forecast the future signal quality trend of the serving cell (such as predicting RSRP changes over a period) based on predicted future RSRPs, then dynamically adjust measurement strategies including when to start/stop measurements, which is based on both real-time signals and predicted trends.

During training, it uses historical serving cell signal data (like RSRP) a as inputs; During inference, it takes real-time serving cell signal data as the core input, and can optionally incorporate network-configured thresholds. The model outputs can be indirect results, such as predicted values of the serving cell's signal quality over a specific time window. Based on the evaluation of the predicted RSRP, a judgment on the future signal trend is formed, and then it is further determined whether to start or stop the neighbour cell measurement. On the other hand, the model outputs can also be direct results. If the measurement threshold is also used as the model input, the model can directly output the measurement decision on whether to start or stop the measurement of the neighbour cell. Based on predicted result, the UE adjusts measurement actions flexibly, breaking the limitations of traditional static triggers.

For example, if the AI predicts that the serving cell's signal (such as RSRP) will remain above the measurement threshold for a period in the future, even if the current real-time signal is slightly below the threshold, the UE can delay starting neighbour cell measurements, which can avoid unnecessary "ping-pong measurements" caused by temporary signal fluctuations and significantly reducing power consumption. Conversely, if the prediction shows the serving cell's signal will continue to deteriorate, the UE can start measurements in advance. In terms of measurement objects, the UE can also additionally utilize the neighbour cell signal prediction to prioritize measuring neighbour cells with better predicted signal quality, instead of measuring all preconfigured cells, further improving efficiency. The collaboration between UE and network follows a clear framework: the network configures basic parameters such as thresholds for the UE; after the UE generates signal predictions and optional measurement decisions through the AI model, it can report the results as required by the network.

Evaluation of this mechanism is conducted through system-level and link-level simulations, referencing evaluation assumptions for AI/ML PHY BM and AI mob. Key evaluation KPIs include RSRP prediction accuracy and other performance metrics (e.g., decision prediction accuracy), with the traditional s-measure mechanism and cell reselection measurement mechanism serving as benchmarks.

This mechanism may bring potential new specification requirements, in addition to KPIs, it also include the core requirement such as prediction delay, measurement period definitions. On the impact for other WG's specifications, necessary signalling and procedure based on AI prediction results are needed.

Based on the above analysis, we have further summarized it in accordance with the exemplary template agreed upon at the last meeting, as follows:

| Sub-use case 5a (Original case 11 - AI - based prediction for enhanced s - measure mechanism) |  | vivo |
| --- | --- | --- |
| AI model input | Input in training | Necessary input: - Current signal quality data of serving cell (such as RSRP, etc.) Other possible input: - network-configured measurement thresholds (such as ssb-RSRP, csi-RSRP, if any) |
|  | Input in inference | Necessary input: - Current signal quality data of serving cell (such as RSRP, etc.) Other possible input: - network-configured measurement thresholds (such as ssb-RSRP, csi-RSRP, if any) |
| AI model output | Label in training (if applicable) | Necessary: - Signal quality data of serving cell (such as RSRP, etc.) Other possible input: - Measurement decision labels based on network-configured measurement thresholds |
|  | Output in inference | Necessary: - Predicted value of future signal quality of serving cell Other possible input: - Measurement decision |
| Assumption on training | Training type | Offline training/Online training |
|  | Label construction (if applicable) | - Based on system-level simulation - Based on filed data |
| model location for inference |  | - UE-sided model |
| Collaboration/interaction between UE and NW |  | - NW side configures basic parameters for the UE, potential including measurement thresholds (such as ssb-RSRP), measurement parameter related (such as SMTC DRX), etc.; - After UE side generates predicted measurement results (or measurement decisions), it reports the results as required by the network; - NW can schedule UE according to measurement decisions |
| Evaluation methodology |  | System level simulation and link level simulation |
| Evaluation assumption |  | The evaluation assumption on AI/ML PHY BM and AI mob time domain RRM measurement prediction can be as the starting point |
| Evaluation KPI |  | RSRP prediction accuracy and other possible performance metrics (e.g., decision prediction accuracy) |
| Evaluation benchmark |  | Non-AI measurement initiation mechanism, such as traditional S-measure measurement mechanism and cell reselection measurement mechanism |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | Potential new requirements: - Prediction delay requirements - Measurement period measurement in the AI-assisted measurement scenarios - RSRP prediction accuracy and other possible performance metrics (e.g., decision prediction accuracy) - Potential scheduling requirements Impact to other WG’s specifications: - Necessary signaling - Necessary procedure based on AI prediction results |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Similar to AI mob use case- time domain prediction with one cell |

Besides, we have supplemented the Expected benefits and Dependence with other WG-led use cases brought by this use case, as shown in the following table.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
| - 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple, vivo | - Enhance Measurement Efficiency - Reduce Unnecessary UE Power Consumption | Dependence with RAN2-led AI mobility use case- time domain prediction with one cell in 5GA | Study how to use AI future prediction results to assist in initiating neighbouring cell measurements and defining relevant RRM requirements |

Proposal 3: RAN4 to consider the Dynamic Adaptation of Measurement Procedure, which uses AI models to forecast the future signal quality trend of the serving cell based on predicted future RSRP of serving cell (e.g., predicting RSRP changes over a period) and then dynamically adjusts measurement strategies, including when to start or stop measurements.

    - AI-based Optimization for Measurement Gap for Inter-frequency Measurement

In Rel-17, RAN4 discussed and introduced the Pre-configured Measurement Gap, which enhances measurement flexibility through pre-configuration and conditional activation/deactivation mechanisms. According to TS 38.133 Section 9.1.7, Pre-MG allows the network to pre-configure gap patterns via RRC signalling and supports two activation/deactivation modes:

- UE autonomous mode: The UE adjusts the state based on events (such as BWP switching, SCell activation, measurement object changes, etc.).

- Network-controlled mode: The network controls the state via preConfGapStatus.

Compared to traditional gap configuration, Pre-MG reduces signalling overhead and introduces activation/deactivation mechanisms. However, it also has significant limitation. The activation/deactivation is driven by events that have already occurred and the UE responds to all predefined events regardless of signal stability, possibly leading to redundant gap activation in stable scenarios or untimely measurements in scenarios where signal quality changes rapidly.

Meanwhile, in the Rel-19 study item on AI/ML mobility, one of the scenarios focuses on quality prediction for inter-frequency cells, where measurements from one frequency band are used to predict the quality of another frequency band, thereby reducing or replacing the need for actual measurements on inter-frequency cells.

In the context of inter-frequency prediction, two scenarios may be considered. In one scenario, inter-frequency prediction eliminates the need for measurement entirely, allowing decisions (such as for handover) to be made directly based on the prediction results and no measurement will be performed for inter-frequency cells. In the other scenario, necessary inter-frequency measurement is still need. For example, in certain cases, even if the quality of an inter-frequency cell is predicted to be poor, the UE still needs to check the quality of the inter-frequency cell by actual measurements, which requires the use of measurement gaps.

Therefore, based on the understanding of the second scenario above, we believe that one possible enhancement direction for RAN4 is to combine AI-driven inter-frequency prediction with the activation/deactivation logic of Pre-MG.

One possible combination is to integrate inter-frequency prediction into the Pre-MG decision as a triggering condition to the existing event-based logic. For Network-controlled mode, the AI prediction results serve as an information for network decision-making; For UE-controlled mode, AI prediction can be a basis for determining the Pre-MG state. For example, when an AI model predicts a change in inter-frequency signal quality in the near future, this prediction result can serve as a supplementary triggering condition for activating/deactivating the MG. Either the UE or the network side can determine whether and when to activate/deactivate the MG based on the prediction results.

It potentially can help to optimize the traditional event-driven Pre-MG by introducing AI prediction as a trigger condition. In scenarios with stable signals, it can deactivate unnecessary gaps in advance, extending gap-free transmission time.

Based on the analysis, we have supplemented the Expected benefits and Dependence with other WG-led use cases brought by this use case, as shown in the following table.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
| 5a | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung | MG reduction | Dependence with RAN2-led AI/ML mobility- inter-frequency domain prediction in 5GA | Identify which use cases can serve the purpose of measurement gap reduction |

Proposal 4: RAN4 to consider to integrate AI/ML-based inter-frequency prediction as a trigger condition for Pre-configured Measurement Gap (Pre-MG) activation/deactivation, to enhance measurement timeliness in dynamic scenarios and reduce unnecessary gaps in stable conditions.

    - RX beam sweeping reduction

The Rel-19 AIML BM defined two core use cases for Tx beam prediction in spatial and temporal domains: BM-Case1 and BM-Case2. In the finalized standardization in RAN4, it mainly focused solely on individual DL Tx beam prediction case. And for Tx-Rx beam pair prediction, it is reflected via UE capability that a UE can indicate to the network whether it supports the capability to provide associated Rx beam information simultaneously when acquiring predicted Tx beam information. This capability definition opens the possibility for research on the utilization of Rx beam knowledge when predicting beam pairs.

Further, for Tx-Rx beam pair prediction, we also have identified some potential benefits: Compared to individual DL Tx beam prediction, our preliminary simulation evaluation shows that Tx-Rx beam pair prediction can help improve Tx beam prediction performance. Additionally, in the current procedure, even if the network side has obtained the predicted Tx beam, without UE capability, the UE still needs to verify and determine the optimal Rx through Rx beam sweeping, which requires a relatively long time and may result in the previously predicted optimal Tx beam having already changed and become invalid by the time this process is completed.

At the last meeting, many companies also mentioned the use case of Rx beam sweeping reduction. For this use case, the first thing that needs to be clarified is the input and output of the model. For the prediction of Rx beam, there must be a clear assumption of Tx beam. Two potential directions include:

- Direction 1: Simply predict the Rx beam. The input may be the subset of Rx beam corresponding to a specific Tx beam, and the output is the full set of Rx beam corresponding to the specific Tx beam.

- Direction 2: Predict through Tx-Rx beam pair. The input is a sub-set of Tx-Rx beam pair (where Tx beam can be set A of Tx beam or set B of Tx beam), and the corresponding output is the full set of Tx-Rx beam pair.

For Direction 1, if the specific Tx beam here refers to the best Tx beam corresponding to each Rx beam, the best Tx beam corresponding to each Rx beam may not be exactly the same. At the same time, as we mentioned above, considering that the terminal may rotate and move at any time, the best Tx beam corresponding to the Rx beam may also change. Therefore, there is a certain inaccuracy in predicting the Rx beam under the assumption of a specific Tx beam. This is why we want to directly predict the full set of Tx-Rx beam pair through the prediction of Tx-Rx beam pair, so as to obtain the optimal Rx beam results under different Tx beams.

For Direction 2, RAN4 can start the research with Tx beam being set A of Tx beam, and indirectly realize the prediction of set A using set B for the Rx beam only in the spatial domain.

Therefore, we propose to continue exploration based on beam pair prediction to achieve Rx beam sweeping reduction.

Based on the above analysis, we have further summarized it in accordance with the exemplary template agreed upon at the last meeting, as follows:

| Sub-use case 7a |  | vivo |
| --- | --- | --- |
| AI model input | Input in training | Measurements of target reference signals using a reduced set of RX beams with corresponding subset/full set of Tx beams |
|  | Input in inference | Measurements of target reference signals using a reduced set of RX beams with corresponding subset/full set of Tx beams |
| AI model output | Label in training (if applicable) | Properties of the full set of RX beams (with corresponding set A Tx beams) for the reference signals measured with subset RX beams (with corresponding set A or set B Tx beams) |
|  | Output in inference | Tx-Rx beam pair prediction result |
| Assumption on training | Training type | offline training |
|  | Label construction (if applicable) | Labeled with actual full RX beams (with corresponding set A Tx beams) and the subset RX beam measurements (with corresponding set A or set B Tx beams) of the same reference signals |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | NW informs UE deployment configuration, e.g., Application ID |
| Evaluation methodology |  | system level simulation, and link level simulation. |
| Evaluation assumption |  | The evaluation assumption on AI/ML PHY BM can be as the starting point |
| Evaluation KPI |  | - RSRP prediction accuracy - Beam prediction accuracy |
| Evaluation benchmark |  | Compare the performance of AI/ML-based reduced RX beam sweeping with legacy full RX beam sweeping in FR2 |
| Preliminary evaluation results |  |  |
| High level potential specification impact |  | Potential new requirement - Reduced measurement period for FR2 measurement due to reduced Rx beam sweeping factor - TCI state known condition - Performance requirements, including beam prediction accuracy and RSRP accuracy |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Model complexity: - UE-side computational load and memory resource complexity for beam pair prediction Testability issue |

Based on the analysis, we have supplemented the Expected benefits and Dependence with other WG-led use cases brought by this use case, as shown in the following table.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi | - Reduce Rx beam sweeping - Potential RS reduction (If in the model input, set B of Tx beam is considered) | Dependence with RAN1-led AI/ML PHY -beam management in 5GA and potential 6G | - Prediction performance evaluation for Tx-Rx beam pair prediction - Corresponding requirement definition introduced by Tx-Rx beam pair prediction |

Proposal 5: RAN4 considers using beam pair prediction to achieve Rx beam sweeping reduction.

    - AI-based Optimization for RLM/BFD/CBD Measurement Mechanism

In the current design, the Radio Link Monitoring (RLM) mechanism requires the UE to periodically measure RLM-RS (SSB or CSI-RS) to assess downlink quality, which may present certain measurement restrictions as defined in current spec:

- For SSB-based RLM, when an SSB overlaps in the time domain with CSI-RS or other SSBs under specific conditions (e.g., different SCS, UE does not support simultaneous reception with different numerologies), the UE must choose one measurement in case of conflict. This may lead to extended RLM measurement periods.

- For CSI-RS-based RLM, in FR2 frequency bands, symbol-level collisions between CSI-RS and SSBs or other CSI-RS resources can trigger similar restrictions, further increasing measurement uncertainty.

To reduce UE power consumption and alleviate the impact of measurement restrictions, one possible approach is to reduce the measurement frequency for RLM-RS. We can consider extending AI prediction technology to RLM measurement prediction. The difference between this prediction and the AI/ML for predicting RLF scenario in the Rel-19 AI/ML Mobility Study lies in the fact that RLF prediction directly or indirectly forecasts whether an RLF event will occur at a future time based on actual measurements or based on predicted and optionally actual L1-SINR results. And here, the prediction for RLM measurement is more focused on replacing actual measurements with predictions to reduce the measurement of RLM-RS, at least under stable cell quality condition. Similarly, the measurement prediction of BFD/CBD can also be considered.

Proposal 6: RAN4 to consider AI-based RLM/BFD/CBD measurement prediction.

- Summary

Observation 1: AI demonstrates significant potential for accurate DPD modelling while maintaining lower computational complexity and hardware cost.

Observation 2: The output power and PAPR of UL signals for data collection significantly impact the performance of the resulting model.

Observation 3: Both AI/ML-based DPD and DPoD demonstrate promising potential in compensating for PA non-linearity with similar complexity.

Proposal 1: RAN4 study AI based solution for PA non-linearity compensation in 6G, including both AI DPD and AI DPoD.

Proposal 2: Update the evaluation procedure to capture both AI DPD and AI DPoD, as listed below:

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - For both AI DPD and AI DPoD

        - FFS suitable AI/ML model, type of training.

        - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD and DPD performance and ensuring comparability across different implementations

        - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models), measurement-based datasets (from multiple UEs).

      - For AI DPD

        - All modulation orders are expected to be evaluated

        - FFS on detailed baseline assumption for UE requirements, including ACLR, SEM, IBE, EVM, etc.

      - For AI DPoD

        - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

        - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For AI DPoD, under a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. For AI DPD, under a given baseline assumption of UE emission requirements, conduct TX MPR and power boosting simulation.

    - The potential MPR reduction and power boosting enabled by AI-DPoD /AI DPD are evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The potential FBR bandwidth reduction of AI DPD can also be considered.

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

Proposal 3: RAN4 to consider the Dynamic Adaptation of Measurement Procedure, which uses AI models to forecast the future signal quality trend of the serving cell based on predicted future RSRP of serving cell (e.g., predicting RSRP changes over a period) and then dynamically adjusts measurement strategies, including when to start or stop measurements.

Proposal 4: RAN4 to consider to integrate AI/ML-based inter-frequency prediction as a trigger condition for Pre-configured Measurement Gap (Pre-MG) activation/deactivation, to enhance measurement timeliness in dynamic scenarios and reduce unnecessary gaps in stable conditions.

Proposal 5: RAN4 considers using beam pair prediction to achieve Rx beam sweeping reduction.

Proposal 6: RAN4 to consider AI-based RLM/BFD/CBD measurement prediction.

- Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, Apple, RAN4 115

- R4-2514637, WF on 6G AI, WF on 6G AI