3GPP TSG RAN WG4 Meeting #117	R4-2521444

Dallas, USA, Nov. 17-21, 2025

Agenda item:			7.12.3

Source:	vivo

Title:	Genernal discussion on RRM impact on RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN#109 meeting, the latest WID [1] was approved. The description of objectives is copied as:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

The RAN4 task specifies the highlighted sections above, primarily covering both RRM measurement prediction and measurement event prediction for the UE-sided model.

In the last meeting, RAN4 conducted a preliminary discussion on issues related to scenarios and sub-use cases, as well as simulation scenarios and assumptions. The outcome of the discussion were captured in WF [2]. In this contribution, we further provide our views on issues regarding RRM requirements for RRM measurement prediction for AI mobility based on the summary reached in the last meeting[2].

- Discussion

  - Factors to be considered(for core part)

Issue 2-2-2: Prediction accuracy requirements for measurement prediction Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement. - Discuss whether the following parameters need to be the side condition of the accuracy requirements: - OW/PW length - MRRT - Skipping pattern - UE speed - L1 filtering approach and sample number

The factors potentially affecting the Prediction accuracy requirements for measurement prediction are summarized above based on the contributions from companies in the last meeting. We will analyze these parameters one by one.

- OW window and PW window

In the SI discussions, RAN4 reached the following conclusions regarding the observation window:

For intra-frequency temporal domain prediction, the impact on prediction accuracy by UE speed, MRRT, and OW/PW length will be considered. The number of measurements performed in OW could be also considered.

According to the RAN2 agreement, the OW length depends on UE implementation. However, from RAN4 perspective, different OW lengths may lead to different prediction performances. And in the SI phase simulation by RAN2, increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. Therefore, RAN4 needs to at least consider different OW lengths in the evaluation to determine the minimum OW length that satisfies the defined prediction accuracy.

Proposal 1: RAN4 to at least consider different OW lengths in the evaluation to determine the minimum OW length that satisfies the defined prediction accuracy.

For the prediction window, obviously, a longer PW length correlates with decreased prediction accuracy. Therefore, during the stage of evaluating and defining requirements, RAN4 needs to consider the prediction performance corresponding to different PW lengths. In addition, for the AI/ML BM-Case 2 prediction, the number of time instances and the time gap between instances are configured by the network. Therefore, when evaluating, the candidate values of PW length also need to refer to the possible configured values of the network (if determined by RAN2).

Proposal 2: During the stage of performance evaluation and defining requirements, RAN4 needs to consider the prediction performance corresponding to different PW lengths

- MRRT and skipping patterns

These two parameters are mainly for Case B. For Case B, the main difference compared to Case A is that certain actual measurements will be skipped and replaced with predicted results. Regarding the specific skipping pattern, RAN2 reached relevant conclusions during the RAN2#131 meeting. As shown below, it was agreed that for the skipping pattern in the temporal domain of Case B, the interpretation is from NW perspective, referring to the SSB configuration used by the network to indicate the timing of its own SSB transmission, rather than from the UE perspective regarding how the UE actually skips measurements. This SSB configuration potentially impacts the MRRT parameter, which will effects prediction performance. Further details are expected to depend on additional conclusions from RAN2.

RAN2#131 meeting Agreements

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

Therefore, regarding whether and how to consider Case B, we suggest waiting for RAN2's conclusion on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

Proposal 3: For MRRT and Skipping pattern for Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

- UE speed

According to the evaluation of RAN2 in SI, for temporal domain prediction, higher UE speed correlates with decreased prediction accuracy. For prediction in the frequency domain, based on the following observation, the impact of UE speed on prediction accuracy in this scenario is relatively small. However, considering the further consideration of measurement errors and changes in the RAN4 evaluation modeling method (if any, such as how to model the frequency-correlation for frequency domain prediction), we suggest that the factor of UE speed should be considered in all scenarios.

For FR1 inter-frequency predictions in co-located scenario, the following observations are made: - The UE speed has minor impact on the prediction accuracy;

Proposal 4: During the evaluation phase, RAN4 needs to consider the impact of different UE speeds on prediction performance

- L1 filtering approach and sample number

The L1 filtering approach and sample number are mainly affected by whether a sliding or non-sliding window is used for this part. For the non-sliding window, each model input is based on new L1 samples. In contrast, for the sliding window, only one sample is input into the model as new information. The amount of information learned by the model differs between the two implementation methods. From the perspective of implementation, non-sliding filtering is more in line with reality. Therefore, we recommend considering the non-sliding window as the baseline, and the sample number reuses the sample number defined by the legacy L1 filtering.

Proposal 5: For performance evaluation, RAN4 to consider the non-sliding window as the baseline, and the sample number reuses the sample number defined by the legacy L1 filtering

  - What kind of core requirements to be defined

What kind of core requirements to be defined Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the follow requirements need to be defined: - Measurement delay/period requirements (OPPO, CMCC, ZTE, Samsung, QC) - Measurement and scheduling restrictions in OW/PW (QC, Samsung) - Prediction delay requirements (OPPO) - Prediction reporting delay requirements (OPPO, HW, QC) - OW/PW length (HW, LGE, Samsung) - Data collection delay requirements (Ericsson) - SCell related requirements (Samsung, QC) - UE mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements (QC)

The Measurement delay/period requirements, Prediction delay requirements, and Prediction reporting delay requirements (in our understanding, it includes Measurement delay/period requirements, Prediction delay requirements and reporting delay) can be discussed together. For the entire prediction process, it includes the actual cell-level measurement, the processing required for inference, and the reporting of prediction results. Based on the outcomes discussed in AI/ML beam management, we defined the prediction delay in the core part. This delay is intended to define the time gap from when the UE performs measurements for inference to when it reports the prediction results. And according to the agreed CR[3] from the previous meeting, RAN4 has defined an overall prediction period delay, which takes into account the above-mentioned Measurement delay/period requirements, Prediction delay requirements, and reporting delay.

Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+ time for the first available reporting occasion Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K (number of samples) *[M]*N*P K as defined by RAN1 Issue 2-2: Reporting delay Agreement for reporting delay: Reuse d’ as to be agreed by RAN1 d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting Final specifications to be agreed after RAN1 includes d’ in the specs

For AI mob, RAN4 can start discussions on whether to follow the same logic to define an overall delay or to define requirements for each part separately. In particular, regarding the measurement period delay, based on our analysis above, the OW length and the number of measurements performed within the OW are considered for the part where the UE performs measurements for inference. Furthermore, whether a sliding or non-sliding window is used for this part will also affect the length of the delay.

Proposal 6: The entire prediction process includes actual cell-level measurement, the processing required for inference prediction, and the reporting of prediction results. For the delays corresponding to these three processes, RAN4 should consider whether to define an overall delay for prediction reporting delay (including the following three) or to define requirements for each respective delay.

- Measurement delay/period requirements,

- Prediction delay requirements

- Reporting delay

In addition, scheduling restrictions in OW/PW need to be considered, which is impacted by the skipped measurement occasions for temporal domain case B and the skipped inter-frequency measurement occasions in the frequency domain. In addition to scheduling restrictions, additional considerations for frequency domain prediction are required: Since inter-frequency cells are obtained through prediction based on the current cell, this particular characteristic may impact requirements such as cell known condition and CSSF. RAN4 needs to take these into account in subsequent discussions.

Proposal 7: RAN4 should identify the potential RRM impacts resulting from the fact that for temporal domain case B an frequency domain, those measurement occasions that are skipped, such as scheduling restrictions, cell known condition and CSSF.

In addition to the above RRM requirements, requirements introduced by LCM procedures also need to be considered. This depends on further procedure and signaling design from RAN2; RAN4 can further discuss whether related LCM requirements are necessary based on further conclusions from RAN2.

Proposal 8: RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design.

  - Performance monitoring

For performance monitoring, RAN2 conducted a general analysis of several monitoring methods during the SI phase. Currently, it remains open whether monitoring will be performed by the network side, the UE side, or via NW decision with UE assistance. Essentially, RAN4 needs to await further conclusions from RAN2 regarding the specific procedures for monitoring.

Captured in TR38.744 Performance of UE-sided model can be monitored in either network side or UE side after UE is configured with monitoring configuration and inference configuration. A monitoring window can be configured, over which the performance monitoring metric can be calculated. For network-sided monitoring, UE can report ground-truth measurement result and inference output to network. And it is up to network implementation to perform monitoring and make further management decision. For UE-sided monitoring, UE can report performance result, i.e. RSRP difference, to network based on ground-truth measurement result and inference output. It is up to network implementation to make management decision based on received performance result. For UE-sided monitoring, it can be considered for UE to make management decision based on network’s configuration and report the decision to network instead of performance result.

Specifically, at the RAN2#131 meeting, RAN2 reached the following conclusions on performance monitoring. From these conclusions, unlike AIML BM, an additional monitoring window has been introduced here. This monitoring window may impact the RRM requirements of the core part (e.g., defining a performance monitoring delay). For instance, the UE needs to perform measurements within this monitoring window (measurement delay for performance monitoring). In the case of UE-assisted monitoring (where the UE calculates the performance monitoring metric), it may be necessary to define the time within which the UE needs to complete monitoring and report the results.

RAN2# 131 meeting agreement: A monitoring window can be configured, over which the performance monitoring metric can be calculated.

Proposal 9: RAN4 to consider the RRM impact on Performance monitoring, taking into account further relevant conclusions from RAN2 (how the monitoring window works, monitoring/decision reporting schemes—periodic reporting or event-triggered reporting, UE/NW decision).

On the other hand, regarding KPIs related to performance monitoring, since the KPI discussions for RRM prediction are defined by RAN4, similarly, for performance monitoring, our understanding is that the definition of metrics also falls within RAN4's scope of discussion. Therefore, RAN4 also needs to define them.

Proposal 10: For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any).

- Summary

Proposal 1: RAN4 to at least consider different OW lengths in the evaluation to determine the minimum OW length that satisfies the defined prediction accuracy.

Proposal 2: During the stage of performance evaluation and defining requirements, RAN4 needs to consider the prediction performance corresponding to different PW lengths

Proposal 3: For MRRT and Skipping pattern for Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

Proposal 4: During the evaluation phase, RAN4 needs to consider the impact of different UE speeds on prediction performance

Proposal 5: For performance evaluation, RAN4 to consider the non-sliding window as the baseline, and the sample number reuses the sample number defined by the legacy L1 filtering

Proposal 6: The entire prediction process includes actual cell-level measurement, the processing required for inference prediction, and the reporting of prediction results. For the delays corresponding to these three processes, RAN4 should consider whether to define an overall delay for prediction reporting delay (including the following three) or to define requirements for each respective delay.

- Measurement delay/period requirements,

- Prediction delay requirements

- Reporting delay

Proposal 7: RAN4 should identify the potential RRM impacts resulting from the fact that for temporal domain case B an frequency domain, those measurement occasions that are skipped, such as scheduling restrictions, cell known condition and CSSF.

Proposal 8: RAN4 further discusses whether related LCM requirements are needed, depending on RAN2's design.

Proposal 9: RAN4 to consider the RRM impact on Performance monitoring, taking into account further relevant conclusions from RAN2 (how the monitoring window works, monitoring/decision reporting schemes—periodic reporting or event-triggered reporting, UE/NW decision).

Proposal 10: For RRM measurement prediction, it is within RAN4's scope of discussion to define the relevant metrics for performance monitoring (if any).

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO

- R4-2514630, Draft CR on RRM core requirements for support of AIML for NR Air Interface for beam management, vivo