3GPP TSG-RAN WG4 Meeting #117		R4-2521452
Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on 5G-6G MRSS

Source:	OPPO

Agenda item:	8.9

Document for:	Approval

Introduction

In RAN #108 meeting, the study item on 6GR was approved in [1], the study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases. One of objectives is to study MRSS:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

  - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

  - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

  - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In RAN4 #116bis meeting, about MRSS, it was agreed that

- MRSS should be considered in the ultimate RAN4 decision, including RAN4’s input to interim milestone.

- Regarding some of the system parameters (e.g. SCS, sync raster, channel raster etc.), separated agenda will be designated for the scenarios of with (in spectrum sharing) and without (in system parameter) MRSS considered.

  - If feasible, RAN4 strives to define unified system parameters for the scenarios with and without MRSS.

  - Revisit this agenda arrangement after Feb. meeting in 2026 for potentially merging the discussion under the single agenda.

- Before RAN4 receives concrete design from RAN1/2, RAN4 discussion will be based on the agreed hypothesis.

And the consensus was achieved to further study 6G-5G MRSS:

- To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study

  - System parameters

  - RF requirements

  - RRM requirements

  - Interference handling

- Taking following table as starting point to discuss which system parameters are impacted by MRSS.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

In this contribution, we further discuss MRSS based on the progress and open issues in last meeting.

Discussion

Operating frequency range for MRSS

In last RAN4 meeting, for operating frequency range or band for MRSS between 6GR and NR, it was agreed:

Issue 1-1-2: Scenarios for MRSS between 6GR and NR

- Agreements

  - For operating frequency range or band for MRSS between 6GR and NR

    - Start RAN4 6G-5G MRSS study in FR1

    - Whether to consider 6G-5G MRSS in FR2-1 is further discussed

But few operators have shown strong interest in the FR2-1 MRSS in lasting meeting discussion. Operators may need more time to consider the 6G deployments and economic benefit, especially, for FR2-1. Therefore, 6G-5G MRSS in FR2-1 (24.25 GHz – 52.6GHz) can be considered base on the interest of industry.

Few operators have shown strong interest in the FR2-1 MRSS in lasting meeting discussion.

6G-5G MRSS in FR2-1 should be considered base on the interest of industry.

Numerology

If NR/6G resources sharing via FDM semi-static, NR and 6G have fixed assigned frequency resource, and there have both of NR signal and 6G signal at one time in the carrier/channel as shown in Figure 1:

Figure 1 NR/6G resources sharing via FDM

From the UE side, regardless of a 6G UE or a 5G UE will receive both of 6G signal and 5G signal at the assigned MRSS carrier at one time.

If a 6G UE is allowed dual connection and is configured both of 5G frequency resource and 6G frequency resource, the 5G signal and 6G signal should keep orthogonality in frequency domain and to make the 6G UE could use one FFT to treat both of 5G signal and 6G signal.

If a 6G UE is only allowed to connect 6G when it is assigned MRSS carrier, the 5G signal shared spectrum with it is an interference, vice versa. To reduce the impact between different RAT on one MRSS carrier, 5G signal and 6G signal should also keep orthogonality in frequency domain.

SCS alignment is necessary to avoid OFDM sub-carrier cross interference when 5G signal and 6G signal sharing frequency resource in one carrier/channel.

If NR/6G resources sharing frequency resource via TDD semi-static, NR and 6G only share the time resource, they will occupy all frequency resource in whole carrier/channel separately, as shown in Figure 2:

Figure 2 NR/6G resources sharing via TDM semi-static

From both of UE side and BS side, no co-channel interference need consider since there is only one kind of signal at one time, only need consider the switching time between 5G configuration and 6G configuration.

There is no co-existence issue for NR/6G resources sharing via TDM semi-static only, the switching time between 5G configuration and 6G configuration need be considered.

If NR/6G resources sharing time/frequency resource via both of TDD and FDM dynamically, but at one time, it still is a FDM mode sharing between NR signal and 6G signal, as shown in Figure 2:

Figure 2 NR/6G resources sharing dynamically

The mutual influence between 5G and 6G sharing the one carrier/channel also need avoid for NR/6G resources sharing dynamically. In addition, it also needs consider the switching time between 5G configuration and 6G configuration.

NR/6G resources dynamically sharing not only need avoid OFDM sub-carrier cross interference when 5G signal and 6G signal share frequency resource in one carrier/channel, but also need consider the switching time.

Based above analysis, to avoid the mutual influence between 5G and 6G sharing the one carrier/channel, 5G signal and 6G signal need keep the orthogonality, therefore, 5G signal and 6G signal should adopt the same SCS.

For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel.

For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study.

Channel raster

6G design also need consider the coexist with 5G within the spectrum that can be shared by 5G and 6G, most bands lower than 2700MHz used100kHz channel raster and those bands higher than 3300MHz used SCS based channel raster. Then in the case of Cell bandwidth and UE specific CBW is configured with ODD/EVEN or EVEN/ODD RBs then the alignment of the two centers is impossible for 100kHz channel raster. Therefore, the enhanced 10kHz channel raster was defined for NR operating bands with 100 kHz channel raster. Therefore, 6G channel raster design need consider the compatibility with all of these 5G channel rasters. And the same issue also needs consider for MRSS.

6G channel raster design for MRSS need consider the compatibility with all of these 5G channel rasters.

For the 5G-6G MRSS bands, the 6G channel raster design shall consider legacy 5G channel raster design, e.g., 100khz channel raster.

For the 5G-6G MRSS bands in FR1, 5kHz channel raster can be adopted.

7.5khz channel raster shifting was introduced for LTE/NR UL sharing, since channel frequency is located between two sub-carriers for LTE UL and over one sub-carrier for NR UL. Therefore, 7.5khz channel raster shifting is not needed if 6G UL channel frequency is the same with that of NR UL located over one sub-carrier.

7.5khz channel raster shifting was introduced for LTE/NR UL sharing, it’s due to channel frequency is located between two sub-carriers for LTE UL and over one sub-carrier for NR UL.

For the 5G-6G MRSS bands in FR1, 7.5khz channel raster shifting is not needed if 6G UL channel frequency is located over one sub-carrier.

Sync raster

Sync raster is the frequency location of SSB, whether the sync raster of 6G will be impacted by 6G-5G MRSS, somehow, depends on the design of SSB and the SSB sharing between 5G and 6G each other.

If 5G/6G sharing the SSB each other, the 5G sync raster should be a subset of the 6G sync raster to facilitate a 6G UE could detect the 5G sync raster.

If 5G/6G not sharing the SSB each other, the 5G UE mis-detecting 6G SSB and 6G UE mis-detecting 5G SSB should be avoided.

- If RAN1 has avoided the mis-detection from SSB design, e.g., the sequence of SSB between 5G and 6G are orthometric, RAN4 don’t need any special treat for the sync raster.

- If not, the sync raster may need stagger between 5G and 6G.

Whether the sync raster of 6G will be impacted by 6G-5G MRSS depends on the SSB design and SSB sharing between 5G and 6G each other.

  - If 5G/6G sharing the SSB each other, the 5G sync raster should be a subset of the 6G sync raster to facilitate a 6G UE could detect the 5G sync raster.

  - If 5G/6G not sharing the SSB each other, the 5G UE mis-detecting 6G SSB and 6G UE mis-detecting 5G SSB should be avoided.

    - If RAN1 has avoided the mis-detection from SSB design, RAN4 don’t need any special treat for the sync raster.

    - If not, the sync raster may need stagger between 5G and 6G.

Waveform, modulation and channel bandwidth

In our understanding, waveform, modulation and channel bandwidth are the deployment issues for 6G-5G MRSS operation, it will not be impacted by the 6G-5G MRSS operation.

Therefore, waveform, modulation and channel bandwidth should be leaved to BS implementation and network’s scheduling restriction.

It’s unnecessary to restrict the waveform, modulation and channel bandwidth for 6G-5G MRSS operation in RAN4 spec.

Conclusion

This paper discussed MRSS and proposed:

Operating frequency range for MRSS

Observation 1:  Few operators have shown strong interest in the FR2-1 MRSS in lasting meeting discussion.

Proposal 1:  6G-5G MRSS in FR2-1 (24.25 GHz – 52.6GHz) should be considered base on the interest of industry.

Numerology

Observation 2:  SCS alignment is necessary to avoid OFDM sub-carrier cross interference when 5G signal and 6G signal sharing frequency resource in one carrier/channel.

Observation 3:	There is no co-existence issue for NR/6G resources sharing via TDM semi-static only, the switching time between 5G configuration and 6G configuration need be considered.

Observation 4:	NR/6G resources dynamically sharing not only need avoid OFDM sub-carrier cross interference when 5G signal and 6G signal share frequency resource in one carrier/channel, but also need consider the switching time

Proposal 2:  For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel.

Proposal 3:  For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study.

Channel raster

Observation 5:  6G channel raster design for MRSS need consider the compatibility with all of these 5G channel rasters.

Proposal 4: For the 5G-6G MRSS bands, the 6G channel raster design shall consider legacy 5G channel raster design, e.g., 100khz channel raster.

Proposal 5: For the 5G-6G MRSS bands in FR1, 5kHz channel raster can be adopted.

Observation 6:   7.5khz channel raster shifting was introduced for LTE/NR UL sharing, it’s due to channel frequency is located between two sub-carriers for LTE UL and over one sub-carrier for NR UL.

Proposal 6:  For the 5G-6G MRSS bands in FR1, 7.5khz channel raster shifting is not needed if 6G UL channel frequency is located over one sub-carrier.

Sync raster

Proposal 7:  Whether the sync raster of 6G will be impacted by 6G-5G MRSS depends on the SSB design and SSB sharing between 5G and 6G each other.

- If 5G/6G sharing the SSB each other, the 5G sync raster should be a subset of the 6G sync raster to facilitate a 6G UE could detect the 5G sync raster.

- If 5G/6G not sharing the SSB each other, the 5G UE mis-detecting 6G SSB and 6G UE mis-detecting 5G SSB should be avoided.

  - If RAN1 has avoided the mis-detection from SSB design, RAN4 don’t need any special treat for the sync raster.

  - If not, the sync raster may need stagger between 5G and 6G.

Waveform, modulation and channel bandwidth

Proposal 8:  It’s unnecessary to restrict the waveform, modulation and channel bandwidth for 6G-5G MRSS operation in RAN4 spec.

Reference

RP-251881 New SID: Study on 6G Radio, NTT DOCOMO	RAN#108

R4-2514646 WF for [116bis][108] 6G spectrum sharing, CMCC, RAN4 #116bis