3GPP TSG-RAN WG4 Meeting #117	R4-2521455

# Dallas, Texas US, 17th – 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | 0089 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_FR1_7MHz_BW-Perf) CR to TS 38.176-2 with clarification for channel bandwidths below 10 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification that for IAB channel BW below 10 MHz are not supported. When 7MHz channel bandwidth was introduced, RAN4 agreed that this channel bandwidth won’t be applicable to NCR and IAB. For IAB specification it was already agreed in Rel-16 that channels below 10 MHz are not supported. During 7MHz WI works, a note to IAB 38.174 and 38.176-1 was added that channels below 10MHZ are not supported, however to 38.176-2 a note was missing. However, to spec 38.176-2 a note with 7MHz was added (R4-2511544). Thus, there is mismatch in IAB specs. This CR aligns notes across IAB specs that channels below 10 MHz are not supported in IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove a note that 7MHz channel BW is not supported and addition a note that exist already in IAB core 38.174 and test 38.176-1 specifications that channel BW below 10 MHz are not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will remain unclear if 7MHz channel BW is supported or not, being inconsistent with IAB core and test specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 5	Operating bands and channel arrangement

For the IAB operation in NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.174 [2], clause 5 and its relevant clauses.

For radiated testing purposes in the present document, FR1 and FR2-1 operating bands are considered.

NOTE:	 Channel bandwidths below 10 MHz are not supported in the present version of the specification.

==============End of change==============