3GPP TSG-RAN WG4 Meeting #117	R4-2521465

# Dallas, Texas US, 17th – 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.908 | CR | 0006 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.908 with aligment for definitions and test procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is CR to TR 38.908 with updates that align technical report text with lates updates in BS core and test specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections and text improvements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligment between technical specifications and technical report on definitions and test procedure for EEIRP mask. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is CR for endorsed on RAN4#116bis CR R4-2514450 |  |  |  |  |  |  |  |  |

==============First change==============

# 5	RF requirement

OTA spatial emission requirement is defined to set upper limits on radiated power in specific directions. This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-Space) satellite receiver.

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 to 7075 MHz, shall not exceed the limits specified in table 5-1.

The expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed:

‒	over azimuth angles from −180º to +180º with reference to BS horizontal boresight, and

‒	over the specified angular range θHLi ≤ θ < θHHi , above the horizon, and

‒	over the supported beam directions.

Table 5-1: EEIRP limits as function of elevation above horizon

| Bin number | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<30 | 16 |
| 6 | 30≤<60 | 15 |
| 7 | 60≤<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

The definitions of elevation angle θHLi and θHHi are illustrated in Figure 5-1.

Figure 5-1: Definitions of θHLi and θHHi angles.

==============Next change==============

### 6.5.3	Test procedure

The test procedure is based on a CATR/IAC where the BS is placed on a positioner and rotated so the required measurement direction is in line with the measurement antenna. The procedure has similarities to existing EIRP and TRP measurement procedures.

The test procedure is as follows:

The test range shall be calibrated the test range, according to calibration method described in TR 37.941, clause 8.3

1. Place the BS at the positioner, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS with the coordinate system used by the test system. The configured EIRP for each of the test beams shall be in direct relation to the intended use of the BS.

3. The measurement device characteristics shall be: Detection mode: True RMS.

4. Set the BS to transmit on both polarizations according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, clause 4.9.2.

5. Orient the positioner (and the BS) to the angle at location for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in clause 6.4.

6. Configure test beam(s) and test equipment for measuring test beam k for k=1…K, as defined in clause 4.9.3.2.

7. Measure EIRP as a sum of two orthogonal polarizations:

EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denote two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beams.

8b. Repeat step 5 and 7 for all M and N positioner angles

9a. Repeat step 5 and 8a for all M and N positioner angle.

9b. Repeat step 6 and 8b for all K test beams.

10. Calculate EEIRPi  from measured EIRPkθn,φm for bins i=1..7 as in clause 6.4.

==============End of change==============