3GPP TSG- WG4 Meeting #117	R4-2521470

# Dallas, US, Nov 17 -  Nov 21

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRM performance requirements to support beam management use case of NR AI/ML Air Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To define RRM performance requirements agreed for the beam management use case of rel.19 NR of NR Air interface WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Introduce RS resource prediction accuracy requirements for FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS38.133 does not include performance requirement for the AI/ML for NR Air interface for BM feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.X RS resource prediction accuracy requirements for FR2

#### 10.1.X.1 SSB based RS resource prediction accuracy requirements

Unless otherwise specified, the accuracy requirements for prediction accuracy of SSB based RS resources in this clause apply to the case when only one SSB based RS resource of the serving cell is configured to be predicted for SSB based RS resource prediction in FR2 when the UE is configured with reportQuantity-r19 set to 'p-ssb-index-r19'.

The accuracy requirements in table 10.1.X.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.X.1- 1: SSB based RS resources prediction accuracy requirements in FR2

| Succes rate | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tolerance Normal condition | Tolerance Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
| ≥[95%] |  |  |  | Minimum Io |  |  | Maximum Io |
|  | dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  | [x] | ([x]+[3]) | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: The successful rate of correct prediction should be greater than or equal to [95%] NOTE 2: The correct prediction is considered if the difference between measured L1-RSRP of predicted strongest SSB based RS resource and the measured L1-RSRP of actual strongest SSB based RS resource is within the tolerance margin NOTE 3: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 4: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 5: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

#### 10.1.X.2 CSI-RS based RS resource prediction accuracy requirements

Unless otherwise specified, the accuracy requirements for prediction accuracy of CSI-RS based RS resources in this clause apply to the case when only one CSI-RS based RS resource of the serving cell is configured to be predicted for CSI-RS based RS resource prediction in FR2 when the UE is configured with reportQuantity-r19 set to 'p-cri-r19'.

The accuracy requirements in table 10.1.20.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in table 10.1.X.2-1.

Table 10.1.X.2-1: CSI-RS based RS resource prediction accuracy requirements in FR2

| Success rate | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ≥[95%] | Tolerance Normal condition | Tolerance Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  | Minimum Io |  |  | Maximum Io |
|  | dB | dB | dB | dBm / SCSCSI-RS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 60 kHz | SCSCSI-RS = 120 kHz |  |  |
|  | [x] | ([x]+[3]) | ≥-3 | Same value as CSI-RS_RP in Table B.2.4.2-2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: The successful rate of correct prediction should be greater than or equal to [95%] NOTE 2: The correct prediction is considered if the difference between measured L1-RSRP of predicted strongest CSI-RS based RS resource and the measured L1-RSRP of actual strongest CSI-RS based RS resource is within the tolerance margin NOTE 3: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 4: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 5: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |

##### <End of Change 1>