3GPP TSG-RAN WG4 Meeting #117	R4-2521473

# Dallas, USA, 17th - 21th, Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FRC for 15KHz for IoT NTN phase 3 in TS 36.108 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19 IoT NTN ehnacement WI, RAN4 has introduced NPUSCH requirement with OCC feature for both 15KHz and 3.75KHz SCS with single tone scenario, new FRC was introduced for 15KHz SC |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new table FRC table related with 15KHz SCS for NPUSCH in section of A.7.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPUSCH requirement with 15KHz SCS for OCC feature can not be verfied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.181 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.7	Fixed Reference Channels for NB-IoT NPUSCH format 1

A.7.1	One PRB

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 | A7-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 |
| Number of allocated subcarriers | 1 | 12 | 1 |
| Diversity | No | No | No |
| Modulation | BPSK | QPSK | BPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 | 0/1 |
| Payload size (bits) | 32 | 136 | 32 |
| Allocated resource unit | 2 | 1 | 2 |
| Code rate (target) | 1/3 | 2/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.56 | 0.29 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 | 96 |
| Total symbols per resource unit | 96 | 144 | 96 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) | 4 |

==============End of change==============