3GPP TSG-RAN WG4 Meeting #117           	 R4-2521475

Dallas, USA, 17th - 21th, Nov, 2025

Agenda item:	6.19.4

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for NR NTN

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup for NR NTN phase 3 was discussed, the related agreement was captured into the WF in [2] as following

- SAN demodulation requirements - Test setup for OCC length 2 - Frequency offset of two multiplexed UEs - Consider setting the same frequency offset for two UEs with 200Hz - Companies are encouraged to double check the simulation results and updated in the next meeting if any - Channel model - Apply the same channel model with NTN-TDLC5-200 to multiplex UEs - Test metric - SNR at 70% of maximum throughput - Measure the PUSCH throughputs per UE - UE1 uses orthogonal sequence index 0 - UE2 uses orthogonal sequence index 1 - Set the same SNR for PUSCH from two UEs if there is no performance difference - FFS how to write the specification.e.g.. define PUSCH demodulation requirements for each UE

In this contribution, the initial simualtion resutls are provided for alignment purpose.

# 2	Simulation Results

In this section, based on agreed simulation assumption, the initial simulation results are provided for alignment purpose

Table 1: Simulation Assumption for PUSCH requirement with supporting inter-slot OCC

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | U, V=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |

Table 2: Initial simulation results for 15KHz with supporting inter-slot OCC, mapping A

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1/1 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -7.14 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -7.09 |
| 2 | 0 | 1/2 | 15KhZ | 5MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -10.4 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -10.32 |

Table 3: Initial simulation results for 15KHz with supporting inter-slot OCC, mapping B

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1/1 | 15KhZ | 5MHz | (0,10) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -7.17 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -7.09 |
| 2 | 0 | 1/2 | 15KhZ | 5MHz | (0,10) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -10.38 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -10.31 |

Table 3: Initial simulation results for 30 KHz with supporting inter-slot OCC, mapping A

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1/1 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -6.52 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -6.53 |
| 2 | 0 | 1/2 | 30KhZ | 10MHz | (2,11) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -9.75 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -9.79 |

Table 4: Initial simulation results for 30 KHz with supporting inter-slot OCC, mapping B

| CASE | UE | Tx/Rx | SCS | BW | DMRS | Channel | Target Tput per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1/2 | 30KhZ | 10MHz | (0,10) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -6.52 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -6.55 |
| 2 | 0 | 1/2 | 30KhZ | 10MHz | (0,10) | NTN-TDLC5-200 Low | 70% | MCS5 | 200Hz | 2 | 2 | -9.79 |
|  | 1 |  |  |  |  |  |  |  | 200Hz |  |  | -9.82 |

# 3	Conclusion

In this contribution, the initial simulation results are provided for alignment purpose.

# References

R4-2515038, Way Forward for [116][331] NR_NTN_Ph3_demod