3GPP TSG-RAN WG4 Meeting #117            	R4-2521480

Dallas, USA, 17th - 21th, Nov, 2025

Agenda item:	6.12.3

Source:	Samsung

Title:	View on BS demodulation requirement for MIMO with 3Tx

Document for:	Discussion

- Introduction

In the last RAN4#116bis meeting, the test scope of performance part of NR MIMO phase 5 was under discussed, the related agreement was captured into WF in [1] as following

- General scope - RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions - FFS on whether to introduce test applicability rule for 3Tx - Test assumption for PUSCH demodulation requirements (if introduced) - SCS and CHBW for FDD - 15kHz SCS, 5MHz, 50MHz - SCS and CHBW for TDD - 30kHz SCS, 10MHz, 100MHz

In this contribution, the view on the remaining issue of PUSCH requirement with 3Tx was provided.  Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Discussion

Measurement uncertainty and test tolerance

For PUSCH performance test, the corresponding MU and TT are defined as following in TS38.141-1:

| Test cases | MU [dB] | Derivation of MU | TT [dB] |
| --- | --- | --- | --- |
| PUSCH 1Tx | 0.6 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB | 0.6 |
| PUSCH 2Tx | 0.8 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB | 0.8 |
| PUSCH 4Tx | 1 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±1 dB | 1 |

For 3 TX, if we apply the same rule for 2Tx and 4Tx, three fading channel simulators will be used for the test per receiver antenna branch and power uncertainty of each fading profile is 0.5dB. Then, the fading profile power uncertainty is sqrt (3*0.5^2) =0.87. The MU of 3 Tx PUSCH test is 0.92. Similar as 2Tx and 4Tx, assuming the test tolerance is same as MU, then, the MU and TT for 3TX can be derived as

| Test cases | MU [dB] | Derivation of MU | TT [dB] |
| --- | --- | --- | --- |
| PUSCH 3Tx | 0.9 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.87 dB | 0.9 |

Since the uncertainty is mainly generated from TE side during the performance testing, the input from TE vendors is highly appreciated to confirm the feasibility of TT value, whether to apply the same value of 2Tx or 4Tx, or apply the new value as 0.9 for 3Tx scenario.

Proposal 1: RAN4 should discuss the following options for the MU and TT for PUSCH 3Tx tests

- Option 1: Applying 0.8dB for MU and TT with fading profile power uncertainty ±0.8dB

- Option 2: Applying 1dB for MU and TT with fading profile power uncertainty ±1dB

- Option 3: Applying 0.9dB for MU and TT with fading profile power uncertainty ±0.87dB

Test Applicability rule

NR supports 1-port, 2-port, 4-port and 8-port PUSCH. RAN4 has introduced requirement with 1Tx, 2Tx and 4Tx in previous release.  In the last meeting, RAN4 agreed to introduce requirement with 3Tx. To reduce the test effort, company proposed to introduce the test applicability rule for different Tx.

From the baseband processing perspective, it is different with different Tx, including different DMRS configuration, MIMO processing. Meanwhile, 1Tx, 2Tx and 3Tx are mainly targeting for handhold UEs, while 4Tx is targeting for CPE or FWA. Even with 1Tx, 2Tx and 4Tx, RAN4 has never introduced the test applicability rule for them. Therefore, we think RAN4 should not introduce the test applicability rule for 3Tx, considering the different baseband processing.

Proposal 2: RAN4 should not introduce the test applicability rule for PUSCH requirement with different Tx

Regarding Rx side, in Rel-15, RAN4 has introduced PUSCH requirement with 1 or 2Tx for 2/4/8Rx antenna configuration. To reduce the test efforts, RAN4 has introduced the test applicability rule for different antennas as following

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

For 4Tx scenario, since only 4 layers requirement with 4Rx and 8Rx, RAN4 has also introduced the test applicability rule as following

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

For 3Tx scenario, similar as 4Tx, the requirement introduced only with the maximum supported layer as 3layer, then the 3Tx antenna tests with low MIMO correlation level shall apply only for the number of Rx antennas is larger or equal to 3. Since RAN4 only introduced the typical antenna configurations with 4Rx and 8Rx for 3Tx requirement, no dedicated requirement was introduced with 3Rx for 3Tx requirement, then, the similar test applicability rule for 4Tx can be reused to perform the 3Tx antenna tests as following

Proposal 3: RAN4 can reuse the similar test applicability rule to perform 3Tx antenna tests with the following

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 3 and 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

# 3	Simulation Results

In this section, the initial simulation results with 3Tx-3 layers transmission were provided for alignment purpose.

Table 1: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 15KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 15KHz | 5MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.432 |
| 2 | 3Tx/4Rx | 15KHz | 5MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.594 |
| 3 | 3Tx/4Rx | 15KHz | 5MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.016 |
| 4 | 3Tx/4Rx | 15KHz | 50MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.521 |
| 5 | 3Tx/4Rx | 15KHz | 50MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.7 |
| 6 | 3Tx/4Rx | 15KHz | 50MHz | TDLA30-10 | A | (2,11) | MCS20 | 17.252 |

Table 2: Initial simulation results of PUSCH with 3Tx- 3layer transmission, 30KHz SCS

| Case | Tx/Rx | SCS | BW | Channel | Mapping Type | DMRS | MCS | SNR@70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3Tx/4Rx | 30KHz | 10MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.669 |
| 2 | 3Tx/4Rx | 30KHz | 10MHz | TDLC300-100 | A | (2,11) | MCS16 | 13.712 |
| 3 | 3Tx/4Rx | 30KHz | 10MHz | TDLA30-10 | A | (2,11) | MCS20 | 15.090 |
| 4 | 3Tx/4Rx | 30KHz | 100MHz | TDLB100-400 | A | (2,11) | MCS 2 | -1.724 |
| 5 | 3Tx/4Rx | 30KHz | 100MHz | TDLC300-100 | A | (2,11) | MCS16 | 14.371 |
| 6 | 3Tx/4Rx | 30KHz | 100MHz | TDLA30-10 | A | (2,11) | MCS20 | 16.900 |

# 4	Conclusion

In this contribution, the view on the remaining issue of test scope and test setup of PUSCH requirement with 3Tx transmission was provided. Meanwhile, the initial simulation results are provided for different MCS and channel bandwidth for alignment purpose.

Proposal 1: RAN4 should discuss the following options for the MU and TT for PUSCH 3Tx tests

- Option 1: Applying 0.8dB for MU and TT with fading profile power uncertainty ±0.8dB

- Option 2: Applying 1dB for MU and TT with fading profile power uncertainty ±1dB

- Option 3: Applying 0.9dB for MU and TT with fading profile power uncertainty ±0.87dB

Proposal 2: RAN4 should not introduce the test applicability rule for PUSCH requirement with different Tx

Proposal 3: RAN4 can reuse the similar test applicability rule to perform 3Tx antenna tests with the following

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 3 and 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

# References

R4-2514806, WF on NR_MIMO_demod , Samsung