3GPP TSG-RAN WG4 Meeting #117           	R4-2521483

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.7.4

Source:	Samsung

Title:	Simulation results and remaining issues of PUSCH demodulation requirement for ATG

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup for performance part of ATG for Rel-19 was under discussion. The related agreement was captured into WF in [1] as following

- Test scope - Antenna configuration for incremental test - 2T2R with X-pol low correlation. - MCS for incremental test - 64QAM table 1 MCS 20 - Test applicability rule for 2Tx - Unless otherwise stated, PUSCH requirement tests in clause 8.2.1 with 2Tx 2-layer test cases shall apply only for BS declared to be supported (see D.123 in table 4.6-1).

In this contribution, based on the agreed simulation assumption, the ideal simulation results are provided for alignment

# 2	Discussion

In the previous meetings, the test setup for PUSCH with UL MIMO with 2Tx was discussed. Based on agreed the test setup, companies provided the simulation results for alignment and requirement derived.

Measurement uncertainty and test tolerance

For PUSCH performance test, the corresponding MU and TT are defined as following in TS38.141-1:

| Test cases | MU [dB] | Derivation of MU | TT [dB] |
| --- | --- | --- | --- |
| PUSCH 1Tx | 0.6 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB | 0.6 |
| PUSCH 2Tx | 0.8 | Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB | 0.8 |

In current Rel-18 ARG performance test, the value of TT with 0.6dB was added for test requirement

| 8.2.4 Performance requirements for PUSCH for high speed train | SNRs as specified | 0.3 dB | Formula: SNR + TT T-put limit unchanged |
| --- | --- | --- | --- |
| 8.2.14 Performance requirements for PUSCH for ATG | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |

According to the MU derivation, the test system uncertainty calculation includes the SNR uncertainty and fading profile power uncertainty. For Rel-18 ATG WI, AWGN channel with fixed frequency offset was agreed as channel condition to derive the requirement for PUSCH with 1Tx. Since there is no fading channel considered, similar as HST scenario, the MU derivation for PUSCH requirement in ATG scenario, it is not proper to consider the 0.6 dB for TT value. Therefore, we suggest to update the value as 0.3dB

Proposal 1:  RAN4 should discuss whether to update the TT value as 0.3dB and corresponding requirement in spec 38.141-1 and 38.141-2 for ATG requirement in Rel-18

- MU and TT: 0.3dB

For Rel-19 ATG scenario, the static channel with fixed frequency offset was agreed as channel condition to derive the requirement for PUSCH with UL MIMO with 2Tx. In our view, it would be good to keep 0.3dB as MU and TT. considering there is no fading profile power uncertainty. Since the uncertainty is mainly generated from TE side during the performance testing, the input from TE vendors is highly appreciated to confirm the feasibility of TT value.

Proposal 2:  RAN4 should discuss whether to set the TT value as 0.3dB to derive the corresponding requirement in spec 38.141-1 and 38.141-2 for ATG requirement in Rel-19. The input from TE vendors is highly appreciated to confirm the feasibility of TT value.

- MU and TT: 0.3dB

# 2	Simulation Results

In this section, both ideal and impairment results are provided based on the agreed simulation assumption as following

Table 1: Simulation assumption for PUSCH requirement under ATG scenario

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| CHBW |  | FDD: 5MHz TDD:10MHz |
| MCS |  | 64QAM Table 1 MCS 20 |
| Antenna configuration |  | 2T2R with X-pol low correlation |
| Propagation conditions and Frequency offset |  | FDD 15kHz SCS: H= 1j1-j+200Hz TDD 30kHz SCS: H= 1j1-j+500Hz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

Table 2: Initial simulation results for PUSCH with 2Tx/2Rx

| Case | Tx/Rx | SCS | BW | Channel | Frequency offset | Mapping Type | DMRS | MCS | SNR@70% TP (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2Tx/2Rx | 15KHz | 5MHz | Static channel | 200Hz | A | (2,11) | MCS 20 table 1 | 13.4 |
| 2 | 2Tx/2Rx | 30KHz | 10MHz | Static channel | 500Hz | A | (2,11) | MCS 20 table 1 | 13.6 |

# 3	Conclusion

In this contribution, the view on remaining issue for Rel-19 ATG PUSCH requirement with UL MIMO was provided. Meanwhile, the ideal and impairment simulation results are provided for alignment and requirement derivation,

Proposal 1:  RAN4 should discuss whether to update the TT value as 0.3dB and corresponding requirement in spec 38.141-1 and 38.141-2 for ATG requirement in Rel-18

- MU and TT: 0.3dB

Proposal 2:  RAN4 should discuss whether to set the TT value as 0.3dB to derive the corresponding requirement in spec 38.141-1 and 38.141-2 for ATG requirement in Rel-19. The input from TE vendors is highly appreciated to confirm the feasibility of TT value.

- MU and TT: 0.3dB

# References

R4-2514796, WF on NR_ATG_enh_demod, CMCC