3GPP TSG-RAN WG4 Meeting #117            	R4-2521486

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.6.3.3

Source:	Samsung

Title:	Discussion on demodulation requirement for NTN NGSO testing

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

In the last RAN4#116bis meeting, the demodulation requirement for NTN testing was under discussion, the related agreements are captured in the WF [2] as following

- Channel model - Eccentricity of satellite orbit - No need to capture “The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero” in the specification, because e≠0 for the initial ephemeris information developed in Rel-19 RAN4. - Additional margin to IoT-NTN UE demodulation requirements when the time-varying Doppler shift and propagation delay model is applied - For NB-IoT, add 0.5dB to the existing NPDSCH requirements - For eMTC, add 1 dB to the existing PDSCH requirements. - Impact of sampling frequency offset - RAN4 does not need to consider the impact of sampling frequency offset for UE demodulation requirements. - How to ensure the satellite elevation angles more than 30 degrees during the test - Interested companies are encouraged to study how TE ensures to apply the same Doppler shift and propagation delay model for each subtest. FFS how to capture in the specification. - Option 1: For each subtest, TE should start from the same initial ephemeris information (i.e., positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182) - FFS on applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model Background In Rel-17 RAN4 defined four PDSCH demodulation requirements for NR NTN (1-1, 1-2, 1-3, and 1-4). In this WI, RAN4 has agreed to add 2 new test cases (1-5 and 1-6) by applying the time-varying Doppler shift and propagation delay model for NGSO to tests 1-1 and 1-2, as follows. Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 1-5; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.3+0.5]. 1-6; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [7.6+0.5]. Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied.. For UE declaring Rel-19 UE, - Option 1 - Applicable tests are 1-5, 1-6, 1-3 and 1-4. - Option 2 - Applicable tests are 1-5, 1-6. For UE declaring Rel-17/18 UE, - Option 1a: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 1b: - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6. - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4. - Option 2: - Applicable tests are 1-1, 1-2, 1-3 and 1-4. Note the same test applicability rule is used for IoT-NTN UE demodulation requirements..

In this contribution, the view on the applicability rule of Rel-19 tests with time-varying Doppler shift and propagation delay model was provided.

# 2	Discussion

Test applicability rule

In Rel-17/18, the UE demodulation requirement was specified under the zero Doppler and constant propagation delay condition which may be not realistic.  For Rel-19, the time-varying doppler shift and propagation delay was considered and modeled to specify the UE demodulation requirement.

From the real deployment of point, companies proposed to apply the time-varying doppler shift and propagation delay for all the existing NTN test cases defined in Rel-17/18.

In the previous meeting, based on WID, to minimum the number of demodulation performance requirements, RAN4 has already agreed and selected the test cases for NR NTN and IoT NTN UE demodulation requirement with applying the time-varying doppler shift and propagation delay channel model, Therefore, only subset of previous test cases was selected to replace the existing test cases for Rel-19 UEs testing as following

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| 1-5 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | [0.3+0.5] |
| 1-6 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | [7.6+0.5] |
| Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied. |  |  |  |  |  |  |  |

In general, for a UE declaring Rel-19 UE, it should pass the existing cases defined in Rel-19 spec. Since the test 1-5 and 1-6 are newly added to replace the test case of 1-1 and 1-2. Therefore, for a UE declaring Rel-19 UE, the applicable cases should be test cases 1-3,1-4 and 1-5 and 1-6.

Proposal 1: For a UE declaring Rel-19 UE, the applicable cases should be test cases 1-3, 1-4, 1-5 and 1-6.

Regarding the Rel-17/18 UEs, as mentioned, this channel model can provide more realistic condition compared with zero Doppler and constant propagation delay condition, then, if Rel-17/18 UE can declare to meet the requirements with timing-varying channel model specified in Rel-19, we think the corresponding test cases for Rel-17/18 UE can be skipped for testing. Other cases are still be tested pending on UE capability for NR NTN and IoT NTN with the assumption of zero doppler and constant propagation delay condition. Therefore, in our understanding, for a UE declaring Rel-17/18 UE, if UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests 1-5 and 1-6, then it can skip the testing for test cases 1-1 and 1-2. While this UE still need to pass the test cases 1-3 and 1-4 pending on UE capability. Since the Rel-17/18 UE still need to pass the test cases 1-3 and 1-4 pending on UE capability with constant frequency offset and propagation delay, we are open to further to discuss the Rel-17/18 UE should not apply the time-varying Doppler and propagation delay channel model, considering the impact of performance requirements should be further evaluated.

Proposal 2: For a UE declaring Rel-17/18 UE, if UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6.  Meanwhile, this UE still need to pass the test cases 1-3 and 1-4 pending on UE capability. Also, FFS on whether Rel-17/18 UE should not apply time-varying Doppler and propagation delay channel model, considering the impact of performance requirements should be further evaluated

# 3	Conclusion

In this contribution, the view on the test applicability rule of Rel-19 tests with time-varying Doppler shift and propagation delay model was provided.

Proposal 1: For a UE declaring Rel-19 UE, the applicable cases should be test cases 1-3, 1-4, 1-5 and 1-6.

Proposal 2: For a UE declaring Rel-17/18 UE, if UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6.  Meanwhile, this UE still need to pass the test cases 1-3 and 1-4 pending on UE capability. Also, FFS on whether Rel-17/18 UE should not apply time-varying Doppler and propagation delay channel model, considering the impact of performance requirements should be further evaluated

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2512525, Way Forward for [116][324] NTN_testing_NGSO_channel_model_and_demod