3GPP TSG-RAN WG4 Meeting #117            	R4-2521487

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.6.3.1.2

Source:	Samsung

Title:	View on the applicability rule for NTN channel model

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

In RAN4#116bis meeting and RAN#109 meeting, there was a discussion on the applicability of NTN testing by using NGSO. In this contribution, the view on the applicability rule of Rel-19 tests with time-varying Doppler shift and propagation delay model from both demodulation and RRM perspective.

# 2	Discussion

Demodulation perspective

In Rel-17/18, the UE demodulation requirement was specified under the zero Doppler and constant propagation delay condition which may be not realistic.  For Rel-19, the time-varying doppler shift and propagation delay was considered and modeled to specify the UE demodulation requirement.

From the real deployment of point view, companies proposed to apply the time-varying doppler shift and propagation delay for all the existing NTN test cases defined in Rel-17/18.

In Rel-17/18 NTN, RAN4 has specified the PDSCH requirements for FR1-NTN band as following cases

Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1.

Also, the following requirements was introduced in FR2-NTN band

PDSCH Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.3-3.1 FDD; 200 / 120; QPSK, 0.30; NTN-TDLC5-1200; 1x1; 70; 3.5. 1-2; R.PDSCH.3-4.1 FDD; 200 / 120; 16QAM, 0.48; NTN-TDLC5-1200; 1x1; 70; 11.2. 1-3; R.PDSCH.3-3.1 FDD; 200 / 120; QPSK, 0.30; NTN-TDLC5-1200; 1x1; 70; 3.5. 1-4; R.PDSCH.3-3.1 FDD (Note 1); 200 / 120; QPSK, 0.30; NTN-TDLC5-1200; 1x1; 70; 4.2. Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled.. PDCCH Table 11.2.3.1.1-1: Minimum performance requirements with 1 Tx Antenna Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 1-1; 200; 132; 1; 8; R.PDCCH.1-1.1 FDD; NTN-TDLC5-1200; 1x1 Low; 1; 4.6. 1-2; 200; 132; 2; 16; R.PDCCH.1-1.2 FDD; NTN-TDLC5-1200; 1x1 Low; 1; 3.2.

The assumption is that UE can do the pre-compensation, only considering the constant frequency offset and propagation delay.

Observation 1: RAN4 has introduced PDSCH and PDCCH requirements for both FR1-NTN and FR2-NTN.

According to WID, RAN4 will specify the TE-emulated channel model with varying Doppler and Delay shifts for NR-NTN and IoT-NTN in FR1-NTN bands in FR1-NTN bands and the corresponding LTE bands. From time varying Doppler and Delay shift channel modeling perspective, it is expected that there is no difference between FR1-NTN and FR2-NTN bands.

Based on the specified channel model, RAN4 will specify the UE demodulation requirements with applying the channel model. Meanwhile, to reduce the test efforts, the number of demodulation performance requirements should be minimized. Then, RAN4 has discussed and down selected some cases for both NR NTN and IoT NTN in FR1-NTN bands.

Observation 2: According to WID, RAN4 will only specify UE demodulation requirement with TE-emulated channel model with varying Doppler and Delay shifts for NR-NTN and IoT-NTN in FR1-NTN bands in FR1-NTN bands and the corresponding LTE bands.

Therefore, the UE demodulation requirements testing for NR NTN is based on assumption of constant frequency offset and propagation delay.

Meanwhile, considering the testing time of vision of served satellite, the case with longer testing time will also be applied for timing-varying channel model, it can only apply for testing with constant frequency offset and propagation delay assumption, such as the test cases for NB-IoT with 128 repetitions.

Observation 3: Considering the test effort and test time, RAN4 only down selects the basic cases with applying TE-emulated channel model with varying Doppler and Delay shifts for NR NTN and IoT NTN for testing. Other cases are still tested based on assumption of constant frequency offset and propagation delay.

Furthermore, to assess the performance impact of time-varying Doppler and propagation delay channel model, RAN4 has evaluated the impact of channel model with considering residual estimation frequency and timing error for these selected cases. Based on companies’ evaluation, companies agreed to add additional margin on existing requirements.

Observation 4: RAN4 has investigated the impact of channel model on performance requirement with considering residual estimation frequency and timing error these selected cases. And additional margin on existing requirement was added based on companies’ evaluation.

Therefore, if applying the time-varying Doppler and propagation delay channel model for all the existing test cases defined in Rel-17/18, since it is out of scope according to WID, we think it should be discussed firstly in RAN-P meeting whether to revise the WID.

Meanwhile, similar as selected cases with applying channel model, the impact of all the remaining cases defined in Rel-17/18 should be evaluated which will require huge simulation workload.

Observation 5: Pending on RAN-P meeting discussion whether to update the WID to include the time-varying Doppler and propagation delay channel model for all the existing test cases defined in Rel-17/18.

RRM perspective

#### Issue 2-2-1: Extend the applicability of the new channel model for the existing test cases WayForward - Further discuss the options. - Option 1: apply to only UL transmit timing test cases. - Option 2: apply to all NTN FR1 RRM tests. - Companies are encouraged to provide a more concrete analysis of the impact of RRM test cases.

Similarly, for RRM requirement, RAN4 also have a discussion on the applicability of NTN testing by using NGSO.

There are two options:

One option is that the NTN testing by using NGSO channel, it is applied for NTN uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands according to WID.

The other option is that the NTN testing by using NGSO channel, it is applied for all NTN test cases.

As mentioned before, the motivation of the WID for RRM is:

  - UE timing tests are key test cases to verity the UE performance for varying doppler and delay shifts.

  - If UE can pass such timing test, which means UE has no issues to handle NGSO and varying doppler and delay shifts.

To avoid such workload of analysis for all RRM tests more than 80 tests, that is why the current WID only designed for RRM uplink timing test cases. For option 2, the main motivation is to avoid to implement the test mode for AWGN channel at all. However, the test mode of AWGN channel for implementation cannot be avoided unless RAN4 finish all analysis for RRM tests

Based on above observations, we think the applicability of NGSO channel for NTN tests is only for selected demodulation performance test cases for NR NTN and IoT-NTN in FR1-NTN bands unless further assessment the impact of other remaining cases defined Rel-17 and Rel-18. And RAN4 should investigate how to perform the NB-IoT test with requiring large repetition by applying the time-varying doppler and propagation delay channel model if agreed to revise the WID

And the applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands unless further observation for all RRM tests if agreed to revise the WID

Proposal 1: The applicability of NGSO channel for NTN tests is only for selected demodulation performance test cases for NR NTN and IoT-NTN in FR1-NTN bands unless further assessment the impact of other remaining cases defined Rel-17 and Rel-18. And RAN4 should investigate how to perform the NB-IoT test with requiring large repetition by applying the time-varying doppler and propagation delay channel model if agreed to revise the WID

Proposal 2: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands unless further observation for all RRM tests.

# 3	Conclusion

In this contribution, the view on the test applicability rule of Rel-19 tests with time-varying Doppler shift and propagation delay model for demodulation and RRM requirement was provided.

Demodulation perspective

Observation 1: RAN4 has introduced PDSCH and PDCCH requirements for both FR1-NTN and FR2-NTN.

Observation 2: According to WID, RAN4 will only specify UE demodulation requirement with TE-emulated channel model with varying Doppler and Delay shifts for NR-NTN and IoT-NTN in FR1-NTN bands in FR1-NTN bands and the corresponding LTE bands.

Observation 3: Considering the test effort and test time, RAN4 only down selects the basic cases with applying TE-emulated channel model with varying Doppler and Delay shifts for NR NTN and IoT NTN for testing. Other cases are still tested based on assumption of constant frequency offset and propagation delay.

Observation 4: RAN4 has investigated the impact of channel model on performance requirement with considering residual estimation frequency and timing error these selected cases. And additional margin on existing requirement was added based on companies’ evaluation.

Observation 5: Pending on RAN-P meeting discussion whether to update the WID to include the time-varying Doppler and propagation delay channel model for all the existing test cases defined in Rel-17/18.

Proposal 1: The applicability of NGSO channel for NTN tests is only for selected demodulation performance test cases for NR NTN and IoT-NTN in FR1-NTN bands unless further assessment the impact of other remaining cases defined Rel-17 and Rel-18. And RAN4 should investigate how to perform the NB-IoT test with requiring large repetition by applying the time-varying doppler and propagation delay channel model if agreed to revise the WID

Proposal 2: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands unless further observation for all RRM tests.

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2512525, Way Forward for [116][324] NTN_testing_NGSO_channel_model_and_demod