3GPP TSG-RAN WG4 Meeting #117            	R4-2521491

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.12.7

Source:	Samsung

Title:	Discussion and simulation results for AI based CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#116 meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - No performance monitoring delay requirements are specified in RAN4 for CSI prediction - CSI requirements for PMI prediction reporting - Introduce a new clause under Clause 6 (6.X) in 38.101-4 as “Reporting of predicted PMI” - Doppler and MCS - Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD - Introduce test for TDD with the details FFS - Generalization - Test setup for generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: The combination of option 1 and 2 - It is FFS on how to quantify the generalization - Activation - No activation requirement in RAN4 spec for CSI prediction will be introduced - Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4

Meanwhile, in order to facilitate the AI/ML model evaluation and alignment, the updated simulation assumption is agreed as

For information - By RAN4 #116bis meeting, RAN4 has 6 inputs for step2 and 5 inputs for step3, all for FDD. - For the new updated step3 results, which is more important to define the CSI prediction performance requirement, seems the variance among different companies is still quite heavy, potentially due to the generated wireless channel by different companies(e.g., different simulation platform), the utilized model(e.g., different backbone, detailed design), the channel estimation approach, and the type1 baseline performance simulated and claimed by different companies. - For the simulation assumption, as agreed in RAN4 #116 bis meeting, “Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD, Introduce test for TDD with the details FFS”, following revising will be captured in the updated simulation. - Remove the 50Hz doppler and remove the mixed Doppler assumption - keep MCS 17 and MCS 19, remove MCS 13 - Use 2RX for FDD - Use 4RX for TDD (update is allowed if any new agreement) - Remove the assumptions on 32Tx ports Way forward for next step simulation for CSI prediction - For step-3, - Interested companies are encouraged to provide more CSI prediction simulation results in RAN4#117 meeting, for both FDD and TDD.

In this contribution, the view on the test setup discussion is provided. Meanwhile, the simulation results for AI/ML based CSI prediction calibration and comparison are provided.

# 2	Test setup for requirement

In this section, we provide our view on the general test setup for specifying the PMI requirement based on AI/ML CSI prediction related with periodic CSI-RS configuration in TDD scenario and Aperiodic CSI-RS configuration for PMI requirement

Aperiodic CSI-RS configuration

According RAN1 agreement, both periodic and aperiodic CSI-RS configuration are considered as following

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms

RAN4 has already discussed the periodic CSI-RS configuration based on RAN1 agreement in the previous meeting, and agreed to apply the test setup in [3] as starting point for performing step-3 throughput simulation.

Regarding the aperiodic CSI-RS configuration, the number of CSI-RS burst with K resource and time interval m slots can be configured based on Rel-18 MIMO eType II codebook. And for Rel-18 eType II doppler codebook test, only 1 time distance for prediction was considered, and K as 4 for both FDD and TDD, as following

Agreement in RAN4#110bis Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook - For FR1 FDD 15khZ SCS case, test setup is as below descriptions - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11 - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21 - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

Since the Aperiodic CSI-RS configuration is optional for AI/ML based CSI prediction, meanwhile, considering the limited time schedule for this WI, only 3 meeting is left based on the original plan. It is more complex to find the proper the test setup to verify the performance of AI/ML based CSI prediction. Therefore, we propose to only specify the periodic CSI-RS configuration for AI/ML based CSI prediction requirement

Proposal 1: Only periodic CSI-RS configuration is considered in Rel-19 for defining the PMI requirement with AI/ML based CSI prediction.

TDD configuration

In legacy PMI test, the requirements with both TDD and FDD are specified. In current evaluation for AI/ML based CSI prediction, it is mainly based on the FDD scenario.  For TDD configuration, the following configuration can be considered as starting point for evaluation.

Test setup for TDD with Periodic CSI-RS configuration (based on R.PDSCH.2-38.1 TDD) For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note:If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3

According this test configuration, we can observe

  - CSI report is transmitted in slot#10n+8 (n=4, 5,6, …,) to estimate precoder in slot#10n+9 (n=4,5,6,…,)(i.e., δ=1)

  - Test equipment apply the estimated precoder in slot#10n+{4, 5, 6, 11, 12, 13} (n=4,5, 6,…)

Meanwhile, the last CSI-RS resource is transmitted at a downlink slot should be slot#(n-8).  Therefore, we propose to revise the note according to current test setup for TDD with periodic CSI-RS configuration.

Proposal 2: Update the test step for TDD with periodic CSI-RS configuration as following

For TDD test setup with Periodic CSI-RS configuration

- CSI-RS periodicity: 5 ms

- CSI feedback periodicity: 5 ms

  - CSI report is transmitted in lot#10n+8 (n=4, 5,6, …,) to estimate precoder in slot#10n+9 (n=4,5,6,…,)(i.e., δ=1)

- Scheduling delay (from CSI feedback to apply in scheduling): 6 slots

Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-8), this reported PMI cannot be applied at the gNB downlink before slot#(n+6).

- Test equipment apply the estimated precoder in slot#10n+{4, 5, 6, 11, 12, 13} (n=4,5, 6,…)

MCS

In the last previous meeting, RAN4 has agreed to introduce tests for 20Hz Doppler with MCS [17 or 19] with 16Tx ports and 2Rx in FDD scenario. For TDD, the details need to be further discussed. Since 4Rx will be considered for TDD case, the expected SNR for 90% of maximum throughput is lower compared with FDD with 2Rx with same MCS level. To ensure the robust SNR conditions, it is better to consider the high MCS for TDD scenario with 4Rx antenna configuration. Therefore, MCS [19 or 22] can be considered for initial simulation

Proposal 3: Consider the MCS [19 or 22] for simulation in TDD scenario with 4Rx antenna configuration

Scheduling for PMI requirement

Regarding the reporting delay between CSI-RS and UCI containing the report with the predicted PMI, in current evaluation, 4 ms separation between CSI-RS and predicted PMI report was considered.

Meanwhile, RAN1 is discussing the CSI processing timeline with following agreement in the previous meeting

Agreement For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature

The negative values of t shall be supported to shorted inference latency compared to non-AI based scheme if using dedicated AI hardware and high degree of parallelization for AI based CSI processing (e.g., matrix operations)

Based on UE feature discussion, RAN1 supported value of t for the relaxation of Z and Z’ timeline, and the value is pending on UE reported capability. The value of t can be large than 0 or small than 0 pending on different SCS.

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Based on UE capability, we think RAN4 should update the schedule timeline for requirement according to the value of t pending on UE capability report

Proposal 4: RAN4 will update the schedule timeline for requirement according to the value of t pending on UE capability report

# 3	Simulation Results

According to the WF in [2], companies are encouraged to provide the CSI prediction simulation results for next meeting, including both SGSC and throughput based on agreed simulation assumption.

For performing the throughput simulation, the test setup for FDD with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD) was applied

Test setup for FDD with Periodic CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

3.2	Throughput

In the performance evaluation, the raw channel matrices estimated based on CSI-RS signals are feed into the AI model as input data per SNR, where the SNR range in our simulation is from -20dB to 20dB.The CSI-RS interval is defined as 5ms. The AI model predicted the channel information in the next one CSI-RS slot, based on the historical 5 CSI-RS channel information, i.e., the observation window is 5/5ms and the prediction window is 1/5ms/5ms. The following shows the initial throughput performance of AI/ML-based CSI prediction schemes under various UE moving speed scenarios.  As benchmark schemes, the throughput achieved by random PMI with type -I single panel codebook, achieved by achieved non-AI scheme (sample and hold) scheme based on Rel-16 eType II Doppler codebook are provided for comparison separately.

In the last meeting, as agreed, in addition to Tput ratio of AI/ML based CSI prediction compared with Rel-15 Type I random PMI, the Tput ratio of AI/ML based CSI prediction compared with Rel-16 eType II CB(sample and hold) is also encouraged to provide. According to the WF, we also output the Tput ratio of Rel-16 eTypen II CB (sample and hold ) compared with Rel-15 Type I random PMI.

- γ1_AI/ML：AI/ML based CSI prediction is compared with Rel-15 Type I random PMI

- γ2_AI/ML：AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold)

- γ3_AI/ML：Rel-16 eType II CB (sample and hold) is compared with Rel-16 eType II CB(sample and hold)

Firstly, we have updated the results with 16x4 antennas for further alignment

Figure 4.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 17, 20Hz  (To be updated)

Figure 5.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 17, 50Hz (to be updated)

Figure 6.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI,  16Tx/4Rx, MCS 19, 20Hz (To be updated)

Figure 7.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI, 16Tx/4Rx, MCS 19, 50Hz (To be updated)

Furthermore, based on agreement in last meeting, we provide the results for FDD with 16x2 antenna configuration for both MCS 17 and MCS 19 under 20Hz

Figure 8.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI, 16Tx/2Rx, MCS 17, 20Hz

Figure 9.  Throughput comparison with AI based scheme, non-AI scheme (Rel-18 eType II Doppler with K=1) and Rel-15 Type I random PMI, 16Tx/2Rx, MCS 19, 20Hz (To be updated)

In table 1, the SNR value corresponding to 90% of maximum throughput of AI/ML based-CSI prediction, Rel-18 eType II Doppler Codebook (with K=1) compared with Rel-15 random Type 1 PMI, as well as the related Tput ratio is summarized for 16x4 antenna configuring in FDD scenario

Table 1 SNR value corresponding to 90% of maximum throughput of AI /ML based CSI prediction Comparison for 16x4 antenna configuration in FDD (To be updated)

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction vs random γ1_AI/M | Prediction vs Sample&Hold γ2_AI/M | Sample&Hold vs Random γ3_AI/M |
| --- | --- | --- | --- | --- | --- | --- |
| 20Hz | 17 |  | 4.05 |  |  |  |
|  | 19 |  |  |  |  |  |
| 50Hz | 17 |  | 5.19 |  |  |  |
|  | 19 |  | 6.52 |  |  |  |

In table 2, the SNR value corresponding to 90% of maximum throughput of AI/ML based-CSI prediction, Rel-18 eType II Doppler Codebook (with K=1) compared with Rel-15 random Type 1 PMI, as well as the related Tput ratio is summarized for 16x2 antenna configuring in FDD scenario.

Table 2 SNR value corresponding to 90% of maximum throughput of AI /ML based CSI prediction Comparison for 16x2 antenna configuration in FDD (To be updated)

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction vs random γ1_AI/M | Prediction vs Sample&Hold γ2_AI/M | Sample&Hold vs Random γ3_AI/M |
| --- | --- | --- | --- | --- | --- | --- |
| 20Hz | 17 | 8.52 | 10.32 | 2.7 | 1.13 | 2.2 |
|  | 19 |  |  |  |  |  |

3.2	Generalization

In this part, we further check the generalization of AI/ML model. In our current results, the model training is per SNR value.  The following is our initial results related the model generalization with SNR,

Figure 10. Throughput comparison with AI based scheme, with different SNR, 16Tx.4Rx, 20Hz, test SNR=-5dB,

Figure 11.  Throughput comparison with AI based scheme, with different SNR, 16Tx.4Rx, 50Hz, testing SNR =0dB

According to our results, we observe with the AI/ML model training under high SNR condition, can achieve better performance compared with the AI/ML model training with the test SNR. With the minor noise impact, AI/ML can learn better channel correlation property under high SNR condition compared with low SNR condition. For one specific testing SNR, with different model training by different dataset, the performance will be different. Then, it may be difficult to align the result and specific the proper requirement. Therefore, RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

Observation 1: With the AI/ML model training under high SNR condition, better performance can be achieved compared with the AI/ML model training with the test SNR.

Proposal 5: RAN4 should further check the generalization related with SNR for AI/ML based CSI prediction

Proposal 6: RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

# 4	Conclusion

In this contribution, the view on test setup of demodulation requirement for AI/ML based CSI prediction is provided. Meanwhile, the simulation results including Throughput performance in Step 3 are provided for AI/ML model alignment.

Proposal 1: Only periodic CSI-RS configuration is considered in Rel-19 for defining the PMI requirement with AI/ML based CSI prediction.

Proposal 2: Update the test step for TDD with periodic CSI-RS configuration as following

For TDD test setup with Periodic CSI-RS configuration

- CSI-RS periodicity: 5 ms

- CSI feedback periodicity: 5 ms

Proposal 3: Consider the MCS [19 or 22] for simulation in TDD scenario with 4Rx antenna configuration

Proposal 4: RAN4 will update the schedule timeline for requirement according to the value of t pending on UE capability report

Observation 1: With the AI/ML model training under high SNR condition, better performance can be achieved compared with the AI/ML model training with the test SNR.

Proposal 5: RAN4 should further check the generalization related with SNR for AI/ML based CSI prediction

Proposal 6: RAN4 should clarify the SNR range of dataset for AI/ML model training if the final results among companies have large gap.

# References

R4-2502856, WF on AI/ML for NR air interface

R4-2505105, WF on AI/ML for NR Air Interface

R4-2508102, WF on updated simulation assumptions for AI based CSI prediction