3GPP TSG-RAN WG4 Meeting #117           	R4-2521492

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.21.3

Source:	Samsung

Title:	View on demodulation requirement for IoT NTN TDD

Document for:	Discussion

- Introduction

In the last RAN4#116bis meeting, the test scope and test setup for performance part of IoT  NTN for TDD operation was under discussed, the related agreement was captured in to WF in [1] as following

- SAN demodulation - NPUSCH format 1 - Channel Model - NTN_TDLA100-1 - NTN_TDLC5-1 - Antenna configuration: 1T1R - Repetition Number: 4 - Subcarrier Spacing: 15KHz - Allocated Subcarrier: 12 - TDD pattern: D/U=8 - NPUSCH format 2 - Channel Model - NTN-TDLA100-1 - Antenna Configuration: 1T1R - Repetition Number - Option 1: 8 - Option 2: 4 with Tx Duration of 2 (Total Duration of 8ms) - Sub carrier Spacing: 15KHz - Allocated Subcarrier: 1 - NPRACH - Channel Model - Option 1: NTN_TDLA100-1 - Option 2: NTN-TDLC5-1 - Antenna Configuration: 1T1R - Repetition Number and Tx Duration - For format 0 - Option 1: 4 with 2Tx duration (total duration of 8ms) - Option 2: 8 with 8Tx duration - Option 3: 16 with 16 Tx duration - For format 1 - Option 1: 2 with 4Tx duration (Total duration of 8ms) - Option 2: 8 with 8 Tx duration - TDD: D/U =8

In this contribution, the view on the remaining issues of test setup to suport TDD operation was provided. Meanwhile, based on agreed initial simualtion assuption, the initial simualtion results are provided for alignment purpose.

# 2	Test Setup

In this section, the view on the remaining issue of test setup for SAN demodulation requirement to support TDD pattern is provided.

NPUSCH1 and NPUSCH 2

Frequency offset

Regarding the frequency offset setting, during the Rel-18 discussion, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the same value pre-compensation will be applied at the UE side within one segment. Due to the moving of satellite, the additional frequency doppler shift offset will be introduced, which may make the dropper shift at the end of segment different with the 1st repetition. For requirement definition, RAN4 agreed to apply the largest accumulate offset with considering the Tx duration, and only one segment transmission was considered for requirement,

In the current test for IoT NTN FDD scenario, RAN4 has specified the following test cases for both NPUSCH format 1 and NPUSCH format 2 as following cases

Table 1: Existing test cases for both NPUSCH format1 and NPUSCH format 2

Observation 1: The frequency offset was set as 8 Hz with assuming 16 repetitions for NPUSCH 1 with 15 KHz SCS and 12 tones scenario, where the total Tx duration time is 16ms

Observation 2: The frequency offset was set as 16Hz with assuming 16 repetitions for NPUSCH 2 with 15 KHz SCS and 12 tones scenario, where total Tx duration time is 16ms

In current test for IoT NTN with TDD scenario, since RAN4 has agreed the number of repetition value is 4 for NPUSCH 1, and number of subcarriers is 12, then, the related frequency offset should be configured as 2Hz.

Proposal 1: Configure the value of frequency offset is 2Hz, when defining the requirement for NPUSCH1 with 15KHz SCS and 12 tones scenario.

Repetition number for NPUSCH 2

Regarding the number of repetitions of NPUSCH 2, the Tx duration time of NPUSCH2 for one repetition is 2ms considering RAN4 agreed to only specify the requirement with 15KHz SCS and 1 tone scenario.  Given the number of limitations for TDD operation, where only 8 continuous UL slots are available. With large number of repetition transmission, the whole NPUSCH2 transmission will be segmented for different segments based on the available continuous slots. Meanwhile, the frequency offset will be accumulated with the Tx duration was increasing with including the DL slots.  In previous RAN4 discussion for IoT NTN FDD, RAN4 has agreed to only define requirement for one segment transmission. Given the number of limitations for TDD operation and the previous agreement for IoT NTN, we would like to configure the number of repetitions as 4 with one segment transmission within the 8 continuous UL slots. Then, the corresponding frequency offset should be configured as 4Hz.

Proposal 2: Configure number of repetitions as 4, when defining the requirement for NPUSCH2 with 15KHz SCS and 1 tone scenario.

Frequency offset for NPUSCH 2

Based on previous analysis, and our preference for number of repetitions, we would like to configure the frequency offset as 4Hz, when defining the requirement for NPUSCH2 with 15KHz SCS and 1 tone scenario.

Proposal 3: Configure the value of frequency offset is 4Hz, when defining the requirement for NPUSCH2 with 15KHz SCS and 12 tone scenarios.

Test Metric for NPUSCH 2

In the last meeting, companies provide the initial results for NPUSCH requirement for IoT NTN TDD. The simulation results collected can be referred in [2], where the corresponding test metric for NPUSCH F2 was based on the 70% of maximum throughput. For NPUSCH2, since it is mainly for ACK/NACK related control information transmission, the typical assumption of test metric for NPUSCH2 was based on 1% of ACK missed detection probability to derive the targeting SNR requirement. Therefore, the corresponding test metric should be refined as 1% of ACK missed detection probability to derive the targeting SNR requirement.

Proposal 4: Update the test metric for NPUSCH2 requirement with 1% of ACK missed detection probability

Timing offset

Besides frequency offset, during Rel-18 IoT NTN discussion, another factor related with timing offset was under discussed considering the satellite moving. RAN4 has agreed that not consider timing offset drift model in TS 36.10 but consider it in TS 36.181 as following agreement made in R4-2305965.

Agreement: Whether to consider timing offset drift model - The total timing offset doesn’t exceed the CP. - Not consider timing offset drift model in TS36.108 but consider it in TS36.181 Agreement: Timing offset drift model - Apply the additional transmitting timing drift to PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2. - Not consider additional transmitting timing drift for PRACH and NPRACH performance requirements - Apply the gradual transmission timing offset model with step size Δt. See the table below for the step size per channel. Channel; Repetition; Tx duration in one segment; Step size Δt. eMTC PUSCH CE mode A; 8; 8ms; [0.01] µs per subframe. eMTC PUSCH CE mode B; 256; 256ms; [0.01] µs per subframe. eMTC PUCCH format 1a; 8; 8ms; [0.01] µs per subframe. NPUSCH format 1, 3.75KHz, 1 tone; 4; 256ms; [0.32] µs per RU. NPUSCH format 1, 15KHz, 12 tones; 16; 16ms; [0.01] µs per RU. NPUSCH format 2, 3.75KHz, 1 tone; 16; 128ms; [0.08] µs per RU. NPUSCH format 2, 15KHz, 1 tone; 16; 32ms; [0.02] µs per RU.

Similar as IoT NTN in FDD scenario, we prefer not to consider the timing offset drift model for IoT NTN in TDD scenario in TS 36.108.  And adding the gradual transmission timing offset for NPUSCH format1 and NPUSCH format 2 in TS 36.181.

Proposal 5: Apply the additional transmitting timing drift to NPUSCH format 1 and NPUSCH format 2. Not consider additional transmitting timing drift for NPRACH performance requirements. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 15KHz, 12 tones | [0.01] µs per RU |
| NPUSCH format 2, 15KHz, 1 tone | [0.02] µs per RU |

Channel Model for NPRACH

Regarding the channel model for NPRACH requirement, in the last meeting, both NTN-TDLA and NTN-TDLC channel models are considered as candidate for requirement. Based on exiting requirement for IoT NTN NPRACH requirement in FDD scenario, only one channel condition was specified as following

Table 2: Existing test cases for NPRACH requirement for FDD scenario

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD Number of TX antennas; Number of RX antennas; TX Duration; Repetition number; Propagation conditions (Annex D); Frequency offset; SNR [dB]. Preamble format 0; Preamble format 1. 1; 1; 8; 8; NTN-TDLA100-1; 200 Hz; 13.3; 13.7. 1; 1; 16; 16; NTN-TDLA100-1; 200 Hz; 9.8; 9.6. 1; 2; 8; 8; NTN-TDLA100-1; 200 Hz; 6.0; 6.1. 1; 2; 16; 16; NTN-TDLA100-1; 200 Hz; 1.6; 1.7.

Observation 3: Only one channel condition with NTN-TDLA100-1 was considered for requirement, when defining requirement for NPRACH

Based on agreement in the last meeting, RAN4 has already agreed to define requirement for both NTN-TDLC and NTN-TDLA channel conditions for NPUSCH1. From the test coverage of different channel models perspective, RAN4 has already considered two channel models. Since there is no different baseband processing with different channel models, to reduce the test efforts and simulation efforts, our preference is only considering one channel model for requirement.  Considering the channel of NTN-TDLA is more complex due to multiple paths compared to NTN-TDLC channel condition, and following the existing test setup for NPRACH requirement in FDD, we would like to consider NTN-TDLA channel only, when defining NPRACH requirement in TDD scenario.

Proposal 6: Configure the NTN-TDLA100-1 channel, when defining NPRACH requirement in TDD scenario.

Number of Repetition for NPRACH

Regarding the configuration of number of repetitions for NPRACH transmission, during the last meeting, serval options are proposed to further down selection for requirement.

The length of NPRACH and number of repetitions will determine the length of Tx duration. As our preference, the format 0 and format 1 are considered. The following are the structure of NPRACH formats in TDD scenario based on specification.

Table 3:  Structure of NPRACH formats in TDD scenario

| Format | Description | G | P | N | CP length | Nominal cell size |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Two symbol groups followed by a guard time fit into 1 UL subframe | 2 | 4 | 1 | 4778 Ts (~155.5us) | ~23.3km |
| 1 | Two symbol groups followed by a guard time fit into 2 UL subframes | 2 | 4 | 2 | 8192 Ts (~266.7us) | ~40.0km |

Figure 1: Structure of NPRACH format 0

Figure 2: Structure of NPRACH format 0

Observation 4:  The Tx duration of NPRACH format 0 is 2ms per each repetition. The Tx duration of NPRACH format 1 is 4ms per each repetition.

To align with TDD with D/U=8, we think the number of repetitions for NPRACH format0 can be considered as 4, and number of repetitions for NPRACH format 1 can be considered as 2, according to the number of repetitions supported for NPRAH TDD operation.

Proposal 7: RAN4 consider the number of repetitions for NPRACH format 0 as 4, and number of repetitions for NPRACH format 1 as 2 when defining NPRACH requirement for TDD operation

Test applicability rule for different antenna configuration

In the previous release for NTN requirement discussion, RAN4 has defined the requirement for both 1Rx and 2Rx antenna configuration based on manufacture declaration. To reduce the test effort, RAN4 has introduced the test applicability rule for different antenna configuration. For the requirement of IoT NTN TDD, since only 1Rx requirement was considered, then, the test of IoT NTN requirement is only applied to one antenna, while the specific connector used for testing is based on manufacturer declaration

Proposal 8: RAN4 should introduce the test applicability rule for IoT NTN requirement, where the test of IoT NTN requirement is only applied to one antenna, while the specific connector used for testing it based on manufacturer declaration

Applicability rule for TDD requirement

Similar as TN specification, we think the requirements for TDD are optional and only valid for SAN supporting this new TDD patten. New applicability rule should be introduced in RAN4, the related test should be performed based on manufacturer declaration.

Proposal 9: The requirements for TDD are optional and only valid for SAN supporting TDD pattern. RAN4 should introduce applicability rule for IoT NTN TDD requirement. And the related test should be performed based on manufacturer declaration

# 3	Simulation Results

In this section, based on agreed the test setup and preference for remaining issue, we provide our preliminary results for IoT NTN with TDD pattern as illustrated in table 4 to table 6, respectively.

Table 4:  Initial simulation results for NPUSCH 1 with 15KHz SCS, and multiple tones scenario

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions (Annex D) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 2 Hz | 4 | 4 | 70% | -1.75 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 2 Hz | 4 | 4 | 70% | -3.56 |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | -3.95 |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | -6.46 |

Table 5:  Initial simulation results for NPUSCH 1 with 15KHz SCS, and multiple tones scenario

| Number of TX antennas | Number of RX antennas | Propagation conditions (Annex D) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | Tx Duration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 4 Hz | 8 | 4 | 5.82 |

Table 6:  Initial simulation results for NPRACH requirement in TDD

| Number of TX antennas | Number of RX antennas | Tx Duration | Repetition number | Propagation conditions (Annex D) | Frequency offset | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 4 | 8 | NTN-TDLA100-1 | 200 Hz |  |  |
| 1 | 1 | 4 | 8 | NTN-TDLC5-1 | 200 Hz |  |  |

# 4	Conclusion

In this contribution, the view on the remaining issues of test setup for IoT NTN requirement was provided. Meanwhile, the initial simulation results are provided for initial alignment purpose.

Frequency offset for NPUSCH1

Observation 1: The frequency offset was set as 8 Hz with assuming 16 repetitions for NPUSCH 1 with 15 KHz SCS and 12 tones scenario, where the total Tx duration time is 16ms

Observation 2: The frequency offset was set as 16Hz with assuming 16 repetitions for NPUSCH 2 with 15 KHz SCS and 12 tones scenario, where total Tx duration time is 16ms

Proposal 1: Configure the value of frequency offset is 2Hz, when defining the requirement for NPUSCH1 with 15KHz SCS and 12 tones scenario.

Repetition number for NPUSCH 2

Proposal 2: Configure number of repetitions as 4, when defining the requirement for NPUSCH2 with 15KHz SCS and 1 tone scenario.

Frequency offset for NPUSCH 2

Proposal 3: Configure the value of frequency offset is 4Hz, when defining the requirement for NPUSCH2 with 15KHz SCS and 12 tone scenarios.

Test Metric for NPUSCH 2

Proposal 4: Update the test metric for NPUSCH2 requirement with 1% of ACK missed detection probability

Timing offset

Proposal 5: Apply the additional transmitting timing drift to NPUSCH format 1 and NPUSCH format 2. Not consider additional transmitting timing drift for NPRACH performance requirements. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 15KHz, 12 tones | [0.01] µs per RU |
| NPUSCH format 2, 15KHz, 1 tone | [0.02] µs per RU |

Channel Model for NPRACH

Observation 3: Only one channel condition with NTN-TDLA100-1 was considered for requirement, when defining requirement for NPRACH

Proposal 6: Configure the NTN-TDLA100-1 channel, when defining NPRACH requirement in TDD scenario.

Number of Repetition for NPRACH

Observation 4:  The Tx duration of NPRACH format 0 is 2ms per each repetition. The Tx duration of NPRACH format 1 is 4ms per each repetition.

Proposal 7: RAN4 consider the number of repetitions for NPRACH format 0 as 4, and number of repetitions for NPRACH format 1 as 2 when defining NPRACH requirement for TDD operation

Test applicability rule for different antenna configuration

Proposal 8: RAN4 should introduce the test applicability rule for IoT NTN requirement, where the test of IoT NTN requirement is only applied to one antenna, while the specific connector used for testing it based on manufacturer declaration

Applicability rule for TDD requirement

Proposal 9: The requirements for TDD are optional and only valid for SAN supporting TDD pattern. RAN4 should introduce applicability rule for IoT NTN TDD requirement. And the related test should be performed based on manufacturer declaration

# References

R4-2515160 Way Forward for [116bis][312] Rel-19 Demodulation_Part2