3GPP TSG-RAN WG4 Meeting #117		 			                           R4-2521501
Dallas, US, November 17th – 21st, 2025
 
Agenda Item: 7.6.4.1

Source:	SKY Perfect JSAT, CHTTL, SES

Title: Discussion on NTN enhancement HD-FDD for Ku band small- type Antenna

Document for:	Discussion

# Introduction

Since the previous RAN4 #116bis meeting, the study on the NTN Core part of HD-FDD operation in Ku-band with FR1 numerology for Rel-20 “Enhanced requirements for NR NTN and IoT NTN Phase 2” has commenced. This Work Item aims to introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20 × 20 cm) by defining a new VSAT type. This contribution clarifies whether the scope of the small-type VSAT antenna should include both GSO and LEO, describes the expected features when GSO is considered for the small-type antenna and assesses the feasibility of establishing a link under these conditions.

# Analysis and Discussion

## Review of Previous Meeting

As described in WF in the RAN4#116bis (R4-2515082) [1], the Work Framework addresses whether to target a new VSAT type for small antennas as ‘VSAT that supports LEO only’ and/or ‘VSAT that supports both GSO and LEO.’ It also clearly specifies concrete aspects for evaluating their implementation and feasibility.

Issue 1-2-1: New VSAT type • WF o Study potential new VSAT type(s) for small antenna with the following candidates:  VSAT that support LEO only  VSAT that support both GSO and LEO Issue 1-2-2: Assumptions for VSAT with small antenna • WF o Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects:  Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

## Small Size Antenna for GSO: Existing Implementations and Market Evidence

In the market, there are Ku-band electronically steered mobile VSAT antennas with specifications such as weight below 6 kg, EIRP of 31 dBW at boresight, G/T of 5.1 dB/K at boresight, noise figure of 3.2 dB, gain of 28 dBi at boresight, and dimensions of 30 cm × 30 cm, assuming HD-FDD operation with a common Tx/Rx aperture [2].

Use cases for such small antennas communicating with GSO include earthquake warning systems, nuclear power plant monitoring, vehicle location management and route optimization in logistics and transportation, remote monitoring of on-site equipment and work progress, data collection in agriculture, emergency communications (including voice), and maritime applications such as vessel navigation management and sea-based communications.

### Future Needs

Our company is a GEO operator and currently provides satellite communication services, including narrowband, low-throughput offerings, to many customers for purposes such as information gathering and emergency voice calls. Looking ahead, there is a growing need to ensure highly reliable narrowband communication solutions, including for vehicle-mounted applications. Service provision through 5G NTN using small-type antennas is expected not only from LEO but also from GSO, which can deliver highly reliable narrowband communication.

### Feasibility Study Approach

In the previous RAN4 meeting, it was agreed to conduct implementation and feasibility studies for the small antenna.

In this contribution, link budget analysis has been carried out to assess the feasibility of providing communication services using the specifications of antennas, as described above, intended for GSO systems currently available in the market.

### UL Link Budget Analysis

Link budget analysis for the uplink has been carried out using the same parameters as those applied in the previous RAN4 work NR_NTN_Ku_bands for evaluating the Ku-band minimum peak EIRP. Satellite specifications are consistent with those employed in the coexistence study simulations.

For the worst-case scenario with an elevation angle of 30 degrees, the Signal-to-Noise Ratio (SNR) was calculated. In this case, the VSAT EIRP was derived by applying a 3 dB reduction to the boresight EIRP of 61 dBm, resulting in an effective EIRP of 58 dBm.

The results of the link budget analysis are as follows. For the uplink, calculations were performed using smaller channel bandwidths to ensure link establishment. Although the minimum channel bandwidth for Ku-band FR1 in Release 19 is 10 MHz, calculations were also carried out for 3 MHz and 5 MHz, as these are existing channel definitions for 5G NR FR1 [3].

For channel bandwidths of 3, 5, and 10 MHz with an SCS of 15 kHz, the number of resource blocks (RBs) is 16, 25, and 54, respectively, resulting in effective bandwidths of 2.88 MHz, 4.5 MHz, and 9.72 MHz.

Table 2.4.1-1: Uplink Link Budget Analysis Results

| Satellite type | GEO | GEO | GEO |
| --- | --- | --- | --- |
| Elevation angle | 30 deg | 30 deg | 30 deg |
| Distance (VSAT-SAN) | 38612 | 38612 | 38612 |
| UL frequency (MHz) | 14500 | 14500 | 14500 |
| Free space path loss (dB) | 207.41 | 207.41 | 207.41 |
| Atmosphere loss (dB) | 2 | 2 | 2 |
| BW (MHz) | 9.72 | 4.5 | 2.88 |
| EIRP (dBm) | 58.0 | 58.0 | 58.0 |
| SAN noise figure (dB) | 3.0 | 3.0 | 3.0 |
| SAN Rx gain (dBi) | 44 | 44 | 44 |
| Implementation margin(dB) | 2 | 2 | 2 |
| SNR (dB) | -11.3 | -7.9 | -6.0 |

The resulting Signal-to-Noise Ratio (SNR) values for the uplink link budget analysis are as follows:

Channel bandwidth of 10 MHz: SNR = –11.3 dB (link establishment may be feasible)

Channel bandwidth of 5 MHz: SNR = –7.9 dB (link establishment feasible with low MCS)

Channel bandwidth of 3 MHz: SNR = –6.0 dB (link establishment feasible with low MCS)

Observation 1: Using the specifications of a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight EIRP of 61 dBm, the uplink link budget analysis was performed. The resulting Signal-to-Noise Ratio (SNR) values are as follows:

Channel bandwidth of 10 MHz: SNR = –11.3 dB,	Link establishment may be feasible

Channel bandwidth of 5 MHz: SNR = –7.9 dB,	Link establishment feasible with low MCS

Channel bandwidth of 3 MHz: SNR = –6.0 dB,	Link establishment feasible with low MCS

Although calculations were performed for channel bandwidths of 10 MHz, 5 MHz, and 3 MHz, actual deployments are expected to utilize even narrower bandwidths. Therefore, using smaller bandwidths is expected to further improve the feasibility of link establishment.

### DL Link Budget Analysis

Link budget analysis for the downlink has been carried out using the same parameters as those applied in the previous RAN4 work NR_NTN_Ku_bands for evaluating the Ku-band EISREFSENS. Satellite specifications are consistent with those employed in the coexistence study simulations.

For the worst-case scenario with an elevation angle of 30 degrees, the Signal-to-Noise Ratio (SNR) was calculated. In this case, the VSAT G/T was derived by applying a 3 dB reduction to the boresight G/T of 5 dB/K, resulting in an effective G/T of 2 dB/K.

For Release 19, the minimum channel bandwidth for Ku-band FR1 is 10 MHz. With an SCS of 15 kHz, the number of resource blocks (RBs) is 54, resulting in an effective bandwidth of 9.72 MHz, which was used for the link budget analysis.

Table 2.4.2-1: Downlink Link Budget Analysis Results

| Satellite type | GSO | GSO |
| --- | --- | --- |
| Elevation angle (VSAT) | 90 deg | 30 deg |
| Distance (VSAT-SAN) | 35786km | 38612km |
| DL frequency (MHz) | 11000 | 11000 |
| BW (MHz) | 9.72 | 9.72 |
| SAN EIRP density (dBW/MHz) | 43 | 40 |
| SAN Tx EIRP(dBm) | 83 | 80 |
| Atmosphere loss | 2 | 2 |
| FSPL(dB) | 204.34 | 205 |
| Received signal level at OTA | -123.46 | -127.12 |
| Boltzmann Constant(dBW/Hz.K) | -228.60 | -228.60 |
| G/T(dB/K) | 5 | 2 |
| IM(dB) | 2 | 2 |
| SNR(dB) | 8.3 | 1.6 |

The resulting Signal-to-Noise Ratio (SNR) value for the downlink link budget analysis is 1.6 dB, confirming that the link can be successfully established.

Observation 2: Using the specifications of a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight G/T of 5 dB/K, the downlink link budget analysis was performed. The resulting Signal-to-Noise Ratio (SNR) is 1.6 dB, confirming that the link can be successfully established.

For a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight EIRP of 61 dBm and a boresight G/T of 5 dB/K, the link budget analysis shows that uplink connectivity is more reliably achieved when using channel bandwidths such as 3 MHz or 5 MHz, although it  can still be established even with a channel bandwidth of 10 MHz.

Based on these results, it is suggested for RAN4 to consider discussing the use of 3 MHz and 5 MHz channel bandwidths to address the challenges with small antennas for GSO.

Proposal 1: RAN4 to consider discussing the use of 3 MHz and 5 MHz channel bandwidths to address the challenges with small antennas for GSO.

Based on the confirmation that link establishment is achievable with a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight G/T of 5 dB/K, and considering the potential for future use cases and market expectations, it is proposed for RAN4 to consider the following antenna specifications as a potential candidate for the baseline of “Small Type Antenna for GSO and LEO”:

Size: 30 cm × 30 cm

Number of antenna elements: 16 × 16

Antenna gain: 28 dBi @ boresight

EIRP: 31 dBW @ boresight

G/T: 5.1 dB/K @ boresight

Noise figure (NF): 3.2 dB

Support: Both GSO and LEO

Proposal 2: RAN4 to consider the following antenna specifications as a potential candidate for the “Small Type Antenna for GSO and LEO”:

Size: 30 cm × 30 cm

Number of antenna elements: 16 × 16

Antenna gain: 28 dBi @ boresight

EIRP: 31 dBW @ boresight

G/T: 5.1 dB/K @ boresight

Noise figure (NF): 3.2 dB

Support: Both GSO and LEO

In contribution [4], the number of elements and basic specifications for “Small Type Antenna for GSO and LEO” have been studied from the perspective of antenna size. It is recommended that RAN4 consider these as candidates in the study of “Small Type Antenna for GSO and LEO”.

In addition, for the following WF, it is proposed that RAN4 consider VSAT solutions that support both GSO and LEO.

Issue 1-2-1: New VSAT type • WF o Study potential new VSAT type(s) for small antenna with the following candidates:  VSAT that support LEO only  VSAT that support both GSO and LEO

Proposal 3: RAN4 to consider VSAT solutions that support both GSO and LEO, as HD-FDD Small Type antenna.

# Conclusions

Observation 1: Using the specifications of a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight EIRP of 61 dBm, the uplink link budget analysis was performed.  The resulting Signal-to-Noise Ratio (SNR) values are as follows:

Channel bandwidth of 10 MHz: SNR = –11.3 dB,	Link establishment may be feasible

Channel bandwidth of 5 MHz: SNR = –7.9 dB,	Link establishment feasible with low MCS

Channel bandwidth of 3 MHz: SNR = –6.0 dB,	Link establishment feasible with low MCS

Observation 2: Using the specifications of a commercially available electronically steered antenna for GSO (30 cm × 30 cm) with a boresight G/T of 5 dB/K, the downlink link budget analysis was performed. The resulting Signal-to-Noise Ratio (SNR) is 1.6 dB, confirming that the link can be successfully established.

Proposal 1: RAN4 to consider discussing the use of 3 MHz and 5 MHz channel bandwidths to address the challenges with small antennas for GSO.

Proposal 2: RAN4 to consider the following antenna specifications as a potential candidate for the “Small Type Antenna for GSO and LEO”:

Size: 30 cm × 30 cm

Number of antenna elements: 16 × 16

Antenna gain: 28 dBi @ boresight

EIRP: 31 dBW @ boresight

G/T: 5.1 dB/K @ boresight

Noise figure (NF): 3.2 dB

Support: Both GSO and LEO

Proposal 3: RAN4 to consider VSAT solutions that support both GSO and LEO, as HD-FDD Small Type antenna.

# References

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, Mediatek, RAN4#116bis

- HiSky, “Smartellite™ Dynamic 16X16 KU”, [Online]. Available:https://hiskysat.com/product/

- TS38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, Release 19.

- R4-2521531, Discussion on the Rel.20 FR1-NTN Ku band VSAT enhancement, CHTTL, SKY Perfect JSAT Corporation, RAN4#117