3GPP TSG-RAN WG4 Meeting #117		R4-2521513
Dallas, USA, Nov. 17-21, 2025

Title:	6G coexist study

Source:	OPPO

Agenda item:	8.4.3

Document for:	Approval

Introduction

The 6G coexistence study was discussed in last RAN4 meeting, and got below WF [1]. This paper shares our view on this aspect.

Issue 2-1-1: Coexistence studies Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?). SBFD Agreement: For SBFD in 6G SI, RAN4 will leverage the Rel-19 and Rel-20 outcomes, and additional scope can be considered if the necessity is identified later. NTN Way Forward: Additional input from satellite companies is expected for RAN4#117: - List of requirements to be further studied (using the same format proposed in 1.1.2) - List of coexistence studies to be considered (using the format proposed in 1.3.1).

Discussion

Prioritization of coexistence study scenarios

Coexistence study is one of the most important aspects need to be considered in the 6G SI, meanwhile, it is also one of the most time-consuming topics. In last meeting, many scenarios and candidate frequencies have been proposed. It can be seen in below table 1. Some prioritization is needed to be more efficient in the evaluation.

Table 1 candidate coexist scenarios for 6G

| Deployment scenarios | Urban macro, Urban micro, Suburban macro, Rural macro, Dense urban, Indoor hotspot |
| --- | --- |
| Carrier frequencies | 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz |
| BS assumptions | Up to 64/288/576/2304/4096 Tx and Rx elements |

There are too many candidate coexistence evaluation scenarios for the start of 6G SI, prioritization is needed.

Among all the frequencies, 7GHz as the most promising high FR1 band for 6G can be considered as starting point. In 5G, around 7GHz band has been evaluated in TR38.921. With the new simulation assumptions in 6G era, it is good to better understand the difference between 5G and 6G.

Regarding the deployment scenarios, Urban macro can be used as starting point.

Around 7GHz coexistence has been studied in 5G TR38.921, which can be used as starting point to compare with 6G new simulation assumptions.

7GHz with Urban macro is considered as starting point for 6G coexistence study.

Coexistence study configurations

BS antenna configurations

In RAN1, there were some agreements for their 6G air interface evaluations like the BS/UE antenna configurations, CBW, frequency [2]. Specifically for the around 7GHz, below are extracts from the agreements which can be used also for RAN4 coexistence study purpose.

RAN1 Agreement on evaluation assumptions for 6GR air interface:

For around 7GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | TBD | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 3 | 1536 | 256 | TBD | (0.5, 0.8)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Consider using the BS antenna modeling in RAN1 6G air interface evaluation assumption for RAN4 coexistence study. Further down selection of antenna configurations is needed.

CBW

Below table is the RAN1 agreement on the CBW/aggregated CBW for the air interface evaluation purpose. Meanwhile, RAN1 also made agreement on the max CBW for network side is 400MHz with 30khz scs and FFS on UE side CBW. These agreements can be considered in RAN4 evaluation.

RAN1 Agreement on evaluation assumptions for 6GR air interface:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated BW | Follow system bandwidth per carrier frequency in TR 38.914 as 1) Around 700 MHz: Up to 60 MHz 2) Around 2GHz: Up to 200 MHz 3) Around 4GHz: Up to 300 MHz 4) Around 7GHz: Up to 400MHz 5) Around 15GHz: Up to 400MHz 6) Around 30GHz: Up to 1GHz |  |  |  |  |
| Simulation BW | Around 700 MHz: 20MHz, 60MHz |  |  |  |  |
|  | Around 2 GHz: 20MHz, 100MHz, 200MHz |  |  |  |  |
|  | Around 4 GHz: 20MHz, 100MHz, 200MHz, 300MHz |  |  |  |  |
|  | Around 7 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 15 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 30GHz: 100MHz, 400MHz, 800MHz |  |  |  |  |
|  | Note: other simulation BW could be considered. |  |  |  |  |
| Note: The layout for each scenario will be separately discussed, including the carrier frequency combination for single layer and/or two layers. |  |  |  |  |  |

RAN1 agreed the max CBW for network side is 400MHz with 30khz scs and FFS on UE side CBW.

RAN4 use 400MHz as the max CBW in NW side and consider 200MHz as the max CBW in UE side for the coexistence evaluation purpose.

Update of TR38.921 configurations

The coexistence study configurations in TR38.921 are extracted as below. These configurations can be reused as much as possible with necessary modifications in blue color for 6G. For other configurations like network layout, path loss, etc. can refer to TR38.921.

For the scheduled CBW per UE, considering currently the spectrum utilization for these new CBW is unclear, for simplicity here propose to use same CBW as the channel BW.

| Parameters | Urban macro |
| --- | --- |
| Carrier frequency | 7 GHz |
| Channel bandwidth | 200 MHz 400 MHz |
| Scheduled channel bandwidth per UE (DL) | Same as CBW |
| Scheduled channel bandwidth per UE (UL) | Same as CBW (1 user, Note 3) CBW/3 (3 user) |
| The number of active UE (DL) (Note 1) | 1 |
| The number of active UE (UL) (Note 1) | 1 or 3 |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm | 43 (Note 2) |
| UE max TX power in dBm | 23 |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 6 (@7 GHz) |
| UE Noise figure in dB | 9 |
| Handover margin | 3 dB |
| Note 1 Same as the number of BS beam(s); Note 2: BS max TX power is defined per polarization; Note 3: 1 user scheduling is a baseline assumption for coexistence evaluation; |  |

Reused the coexistence study configurations in TR38.921 as much as possible for 6G study.

For the scheduled CBW per UE, consider use same value as CBW for 1user case, and use CBW/3 for 3 users case considering now the spectrum utilization for the new CBW is unknown.

Conclusion

This paper discussed the RAN4 6G coexistence study scenarios and configurations and got below observations and proposals.

Observation 1: 	There are too many candidate coexistence evaluation scenarios for the start of 6G SI, prioritization is needed.

Observation 2: 	Around 7GHz coexistence has been studied in 5G TR38.921, which can be used as starting point to compare with 6G new simulation assumptions.

Proposal 1: 	7GHz with Urban macro is considered as starting point for 6G coexistence study.

Proposal 2: 	Consider using the BS antenna modeling in RAN1 6G air interface evaluation assumption for RAN4 coexistence study. Further down selection of antenna configurations is needed.

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | TBD | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 3 | 1536 | 256 | TBD | (0.5, 0.8)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Observation 3: 	RAN1 agreed the max CBW for network side is 400MHz with 30khz scs and FFS on UE side CBW.

Proposal 3: 	RAN4 use 400MHz as the max CBW in NW side and consider 200MHz as the max CBW in UE side for the coexistence evaluation purpose.

Proposal 4: 	Reused the coexistence study configurations in TR38.921 as much as possible for 6G study.

Proposal 5: 	For the scheduled CBW per UE, consider use same value as CBW for 1user case, and use CBW/3 for 3 users case considering now the spectrum utilization for the new CBW is unknown.

| Parameters | Urban macro |
| --- | --- |
| Carrier frequency | 7 GHz |
| Channel bandwidth | 200 MHz 400 MHz |
| Scheduled channel bandwidth per UE (DL) | Same as CBW |
| Scheduled channel bandwidth per UE (UL) | Same as CBW (1 user, Note 3) CBW/3 (3 user) |
| The number of active UE (DL) (Note 1) | 1 |
| The number of active UE (UL) (Note 1) | 1 or 3 |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm | 43 (Note 2) |
| UE max TX power in dBm | 23 |
| UE min TX power in dBm | -33 |
| BS Noise figure in dB | 6 (@7 GHz) |
| UE Noise figure in dB | 9 |
| Handover margin | 3 dB |
| Note 1 Same as the number of BS beam(s); Note 2: BS max TX power is defined per polarization; Note 3: 1 user scheduling is a baseline assumption for coexistence evaluation; |  |

Reference

R4-2514638 WF on 6G BS RF and co-existence, Ericsson, #116bis 2025-10

RAN1#122bis Chair notes.