3GPP TSG-RAN WG4 Meeting # 117	R4-2521516

# Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion for Rel-20 IoT-NTN HPUE

Agenda item:	7.6.3

Document for:	Discussion

Introduction

In the RAN4#115, the WF on NR_IoT_NTN_Ph2was approved. In this discussion, we share our point of view for the HPUE requirements for NR_IoT_NTN_Ph2 based on the approved WF [1].

Discussion

UE RF front end heating issue

Table 1 Way forward for for SAR solution and UE RF front end assumption for IoT-NTN HPUE [1]

- Issue 2-2-2: SAR solution for PC1.5 NB-IoT NTN UE Agreement: - Re-use existing P-MPR method for PC1.5 NB-IoT NTN UE. - Issue 2-2-3: UE RF front end assumptions and related accommodations WayFoward: - FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements) - The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118.

In the previous RAN4 meeting, it was agreed to reuse the existing P-MPR method as the SAR solution for PC1.5 NB-IoT NTN UE. This approach leverages the established mechanism used in 5G to ensure compliance with SAR regulations under high-power transmission conditions.

Although the P-MPR approach appears promising in mitigating SAR-related issues, its applicability to RF front-end protection is not yet confirmed. Traditionally, P-MPR has been applied to address SAR constraints, but its extension to RF front-end overhead management requires further validation.

Proposal 1: Study the feasibility of extending the P-MPR mechanism beyond SAR regulation, to also cover UE RF front-end protection under prolonged high-power transmission scenarios.

PC1.5 IoT-NTN MPR requirement

Table 2 Way forward for for SAR solution and UE RF front end assumption for IoT-NTN HPUE [1]

- Issue 2-2-6: MPR requirements for PC1.5 NB-IoT NTN UE Agreement: - Take the following tables as starting point for the MPR for PC1.5 NB-IoT NTN UE (Based on MPR = maximum {0, PC1 MPR - 2dB}) Table 2-2-6-1 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone Modulation; π/2 BPSK, π/4 QPSK; π/2 BPSK, π/4 QPSK. SCS; 15 kHz; 3.75 kHz. Tone positions for 1 Tone allocation; 0-1; 2-9; 10-11; 0-5; 6-41; 42-47. MPR; ≤ [0] dB; [0] dB; ≤ [0] dB; ≤ [1.5] dB; [0] dB; ≤ [1.5] dB. Note 1: For single-tone allocation, the listed modulations and SCS are supported.. Table 2-2-6-2 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone Modulation; QPSK. SCS; 15 kHz. Tone positions for 3 Tones allocation; 0-2; 3-5 and 6-8; 9-11. MPR; ≤ [0] dB; ≤ [0] dB; ≤ [0] dB. Tone positions for 6 Tones allocation; 0-5 and 6-11. MPR; ≤ [0.5]dB; ≤ [0.5] dB. Tone positions for 12 Tones allocation; 0-11. MPR; ≤ [2] dB. Note 1: For multi-tone allocation, the listed modulation and SCS are supported.. - Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed - Option 1 - Calibration points for PC1.5 (based on NR PC2 PA): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss /or [5] dB - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc / or [31] dBc - CIM3: 60 dBc - NR ACLR: 31dBc - Option 2 - DFT-s-OFDM 1RE (Single Tone) - Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK] - [3] or [4] dB post PA loss - 0dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

In the previous RAN4 meeting, agreement was reached to use the following tables as the starting point for defining Maximum Power Reduction (MPR) for PC1.5 NB-IoT NTN UE, based on the formula:

MPR=max(0, PC1 MPR−2dB)

During the last meeting, calibration points for PC1.5 NB-IoT NTN UE were also discussed. It was noted that the calibration conditions are largely similar to those of PC1 NB-IoT NTN UE, with the exception of a potentially higher post-PA loss (e.g., +1 dB additional loss). This increased loss may result in a need for more relaxed MPR values than those derived from the PC1 baseline. Given this, an initial assumption for MPR could be:

MPR=max(0, PC1 MPR−1.5dB)

This slightly relaxed value may better reflect the actual RF performance and thermal constraints of PC1.5 UEs under the assumed calibration conditions. Detailed simulation results are planned to be submitted at the next RAN4 meeting.

Proposal 2: Consider initial MPR values for PC1.5 NB-IoT NTN UE as shown below table.

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0.5 dB | 0 dB | ≤ 0.5 dB | ≤ 2.0 dB | 0 dB | ≤ 2.0 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | ≤ 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1.0dB |  | ≤ 1.0dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2.5 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Conclusion

Proposal 1: study the feasibility of extending the P-MPR mechanism beyond SAR regulation, to also cover UE RF front-end protection under prolonged high-power transmission scenarios.

Proposal 2: Consider initial MPR values for PC1.5 NB-IoT NTN UE as shown below table.

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0.5 dB | 0 dB | ≤ 0.5 dB | ≤ 2.0 dB | 0 dB | ≤ 2.0 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | ≤ 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1.0dB |  | ≤ 1.0dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2.5 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

- References

# [1] R4-2515157, WF for [116bis][318] NR_IoT_NTN_Ph2 (CHTTL)