3GPP TSG RAN WG4 Meeting #	117                                                   R4-2521521

Dallas, US, November 17th – 21th, 2025

Agenda Item:	7.6.2

Source:	CSCN

Title:	Views on general aspects of NTN topics

Document for:	Discussion/Decision

Introduction

In RAN#109 meeting, a new WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 in Rel-20 is approved with objectives related to HPUE for NTN, HD-FDD for NTN and DL CA for NTN as following [1].

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes. HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note: Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable. DL intra-band intra-SAN CA for NTN Core Part - Specify RF and RRM requirements for NR NTN CA for UE/VSAT and SAN core requirements with 2DL component carrier and 1UL - Contiguous carriers in Ka band (Band n512) - 400 MHz + 400 MHz channel bandwidths - Non-contiguous carriers in L band (Band n255) - 5 MHz + 5 MHz channel bandwidths The DL intra-band intra-SAN CA objectives will commence upon completion of HPUE for NTN Core Part (PC1.5 for IoT-NTN) and RAN1/RAN2 availability. HPUE for NTN Performance Part - None HD-FDD for NTN Performance Part - RRM performance and demodulation requirements as needed DL CA for NTN Performance Part - SAN conformance, RRM performance and demodulation requirements for intra-SAN DL CA as needed

Considering only the work plan for the HPUE for IoT-NTN and HD-FDD phase 1 is provided in the last meeting [2], we provide our views on the general aspects about the objectives of HPUE for NR-NTN and HD-FDD phase 2 in this contribution.

Discussion

  - NTN HPUE

According to the objectives related to NTN HPUE, the band agnostic requirements for PC1.5 UE specified for TN are expected to apply to NR NTN PC1.5 UE as well. Although some results of work item for NR-TN HPUE were reused for NR-NTN HPUE in Rel-19, this kind of approach is still not an explicit methodology defined in RAN4. Namely, not entire requirements for certain NR-NTN UE power class were directly specified as a whole based on the spec for corresponding NR-TN power classes, but discussed and decided case by case or parameter by parameter in precedent.

Due to the quite different aspects between NR-TN and NR-NRN, e.g. deployment scenarios, propagation channel conditions, localized regulations, co-existence within intra/inter RAT and etc., which have significant effects on HPUE’s requirements, it is very meaningful to confirm all the band agnostic requirements for NR-NTN UE power class (e.g. PC1.5) can be met in the study for corresponding NR-TN power class.

Proposal 1: Clarify that all band agnostic requirements for NR-NTN UE power class (i.e. PC1.5) can reuse those for corresponding NR-TN power class.

Further, in our understanding, it is reasonable to extend the existing basket approach used in TN to include the spectrum related work for NTN HPUE, rather than to create separate WIs for each individual FR1-NTN band. So, it is critical to propose and approve the expected basket WI timely, e.g. in Q4, 2025. While the unique requirement for NTN, if any, could be identified and studied specifically.

Proposal 2: Suggest to approve the basket WI for spectrum related work for NR-NTN UE at next Plenary.

  - NTN HD-FDD

From the perspective of RAN1/RAN2, the RedCap NTN HD-FDD solution [3] focuses on the collision handle rules design for the HD-FDD working scenarios, no matter which band or numerology it is applied to, Hence, to our minds there is no difference for leveraging existing NTN HD-FDD solution to support NTN HD-FDD VSATs in either Ka band or Ku band with FR1 or FR2 numerology.

On the other hand, even if exists any possible impact on RAN1/RAN2 regarding Phase 2, it is feasible to identify the work plan for demonstrating RAN1/RAN2 availability firstly. Further, this work can be carried out independently, rather than upon completion of Phase 1 or after Q2 of 2026, which is critical to accomplish the item of HD-FDD for NTN timely considering tight schedule of Rel-20 study.

Proposal 3: Trigger the study of RAN1/RAN2 availability for Phase 2 as early as possible.

Conclusion

In this contribution, we propose some views on general aspects of NTN related topics.

Proposal 1: Clarify that all band agnostic requirements for NR-NTN UE power class (i.e. PC1.5) can reuse those for corresponding NR-TN power class.

Proposal 2: Suggest to approve the basket WI for spectrum related work for NR-NTN UE at next Plenary.

Proposal 3: Trigger the study of RAN1/RAN2 availability for Phase 2 as early as possible.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2

[2] R4-2515080, Work plan for Enhanced requirements for NR NTN and IoT NTN Phase 2

[3] RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3