3GPP TSG-RAN WG4 Meeting #117	R4-2521524

Dallas, USA, November 17th - 21st, 2025

Source:	Nokia

Title:	Discussion on Conditional Intra-CU LTM Test Cases

Agenda item:	6.17.2.3

Document for:	Discussion

Introduction

In RAN4#116bis, the following test cases were agreed for CLTM.

| Test case | Baseline test case | Test case | Company |
| --- | --- | --- | --- |
| 1 | LTM R18 A.6.3.4.1 | RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | China Telecom |
| 2 | LTM R18 A.6.3.4.2 | RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | Huawei |
| 3 | LTM R18 A.6.3.4.3 | RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | Nokia |
| 4 | LTM R18 A.6.3.4.4 | RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1 | Ericsson |
| 5 | LTM R18 A.7.3.4.1 | RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2 | ZTE |
| 6 | LTM R18 A.7.3.4.2 | RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2. | vivo |

In this contribution, we provide our views on open issues related to these test cases.

Discussion

Early DL synchronization in CLTM test cases

Options for early DL synchronization in CLTM test cases were discussed in RAN4#116bis under Issue 1-7.

Issue 1-7: early DL sync < Way forward > - Option 1: With early DL sync. - Option 2: With and without early DL sync. - Option 3: For each FR, one test case with and without early DL sync, with early DL sync only for other test cases.

As shown below, in Rel-18 cell switch test cases, early TCI state activation was either included or not included in the same test case depending on UE support of the feature. We believe that we can follow the same Rel-18 approach for CLTM test cases.

#### A.6.3.4.1 RACH-based Intra-frequency PCell switch from FR1 to FR1 ##### A.6.3.4.1.1 Test Purpose and Environment This test is to verify the requirement for the NR FR1-NR FR1 RACH-based intra-frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation. ##### A.6.3.4.1.2 Test Parameters Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.6.3.4.1.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.6.3.4.1.2-2 and A.6.3.4.1.2-3. The test consists of 2 tests, and UE is required to pass one among Test 1, Test 2. - Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 - Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18

RAN4 to cover both with and without early DL sync tests in the CLTM cell switch test cases. These can be defined so that, depending on UE capability, early TCI state activation is included or not in the same test case.

Subsequent CLTM test cases

Options for subsequent CLTM cell switch test cases were discussed in RAN4#116bis under Issue 2-1.

Issue 2-1: Test case list for Subsequent CLTM < Way forward > - Option 1: Define test cases for subsequent CLTM. - For FR1: - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM. - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM. - For FR2: the same proposal as FR1. - Option 2: Not define test case for subsequent CLTM as CLTM.

Support for subsequent CLTM cell switch is not an optional feature, so a UE supporting CLTM will also support subsequent CLTM. Subsequent CLTM cell switch delay requirement is the same as the initial cell switch delay requirement, except for the starting point and the lack of parameter TRRC. As the subsequent CLTM cell switch procedure needs an initial CLTM cell switch to be performed by the UE, we believe that we can include both initial and subsequent cell switches under the same agreed test cases (i.e., no need for separate test cases for subsequent CLTM). The purpose of these test cases would be to verify the subsequent CLTM cell switch delay requirements, which in our view is sufficient.

No need for defining separate test cases for subsequent CLTM cell switch. The test cases agreed for CLTM can be defined so that subsequent CLTM cell switch delay requirements are verified.

SSB-based and CSI-RS-based triggering

Following the requirements, the measurement time value differs when assuming SSB-based and CSI-RS-based triggering in CLTM. Hence, we believe that SSB-based triggering can be assumed in the agreed test cases, while one more test case for CSI-RS-based triggering can be added to the test case list (e.g., RACH-less intra-frequency L1-triggered CLTM PCell switch from FR1 to FR1 with CSI-RS-based triggering).

SSB-based triggering is to be assumed in the agreed test cases. RAN4 to add a new test case for CSI-RS-based triggering (e.g., RACH-less intra-frequency L1-triggered CLTM PCell switch from FR1 to FR1 with CSI-RS-based triggering).

Conclusion

The following proposals were made:

Proposal 1: RAN4 to cover both with and without early DL sync tests in the CLTM cell switch test cases. These can be defined so that, depending on UE capability, early TCI state activation is included or not in the same test case.

Proposal 2: No need for defining separate test cases for subsequent CLTM cell switch. The test cases agreed for CLTM can be defined so that subsequent CLTM cell switch delay requirements are verified.

Proposal 3: SSB-based triggering is to be assumed in the agreed test cases. RAN4 to add a new test case for CSI-RS-based triggering (e.g., RACH-less intra-frequency L1-triggered CLTM PCell switch from FR1 to FR1 with CSI-RS-based triggering).