3GPP TSG-RAN WG4 Meeting #117	R4-2521528

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	Discussion on General Aspects for Two-sided AI/ML Models

Agenda item:	7.9.2

Document for:	Discussion

Introduction

In this contribution, we share our views on the NR_AIML_air_Ph2 workplan and RAN4 response to the upcoming RAN checkpoint.

Discussion

Updates to the workplan

The workplan for the NR_AIML_air_Ph2 work item was submitted by the moderator in RAN4#116bis [1]. According to this workplan and as shown below, the performance requirement discussion is planned to start in RAN4#121.

| RAN4#121 | 1 | 0.5 | - Finalize LCM framework - Draft specifications and finalize models in format ready to be captured in 3GPP | - Discuss requirement side conditions for performance requirement - Discuss RRM performance requirement side conditions |
| --- | --- | --- | --- | --- |
| RAN4#122 | 1 | 0.5 | - Specifications, dataset and model capture complete. | - Performance requirements - RRM performance requirements |

We believe that RAN4 should start the discussion on the metrics to use for defining the performance requirements before the actual start of the performance part. Hence, the workplan should be updated to include this discussion in the core part.

The NR_AIML_air_Ph2 workplan should be updated to include in the core part the discussion on the metrics to use for defining the performance requirements.

Response to RAN checkpoint

The following agreements were made in RAN4#116bis regarding the relation between RAN1 inter-vendor training collaboration directions and RAN4 test decoder options, and the response to the RAN checkpoint.

Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4. How to respond to the RAN checkpoint • Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. • Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. • FFS on detailed steps to develop the model • RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint.

As feedback from RAN4 is expected for RAN#110, we propose to consider the following aspects in the response:

List of agreements made in RAN4#116bis and RAN4#117 about models and datasets

Agreements made in RAN4#116bis … Agreements made in RAN4#117 …

Steps to develop the test decoder and reference encoder

Step 1: Create dataset [Ongoing] Step 2: Align on the model backbone, model hyperparameters and the scalability solution [Ongoing] Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion

Feedback expected from RAN1

In this part, RAN4 can ask RAN1 to share some design considerations that can be useful for dataset/model creation. While the expectation is that RAN1 may reuse the models created by RAN4 for inter-vendor training collaboration, RAN4 may create these models from a testing perspective only (e.g., considering a limited set of configurations). Hence, any assistance information from RAN1 at the beginning of the WI can increase the chances that the models created by RAN4 will be eventually used by RAN1 for inter-vendor training collaboration.

RAN4 to consider the following aspects as part of the response to the RAN checkpoint:

- List of agreements made in RAN4#116bis and RAN4#117 about models and datasets

- Ongoing and next steps to develop the fully specified test decoder in RAN4

- Feedback expected from RAN1 (e.g., some design considerations that can be useful for dataset/model creation)

Conclusion

The following proposals were made in this contribution:

Proposal 1: The NR_AIML_air_Ph2 workplan should be updated to include in the core part the discussion on the metrics to use for defining the performance requirements.

Proposal 2: RAN4 to consider the following aspects as part of the response to the RAN checkpoint:

	List of agreements made in RAN4#116bis and RAN4#117 about models and datasets

	Ongoing and next steps to develop the fully specified test decoder in RAN4

	Feedback expected from RAN1 (e.g., some design considerations that can be useful for dataset/model creation)

- References

- R4-2514253, “Workplan for Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Ericsson, RAN4#116bis, October 13-17, 2025, Prague, Czech Republic.