# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521540

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements on NTN phase 3

Agenda item:	4.25.3

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved. RAN4 shall define the RRM requirements for downlink coverage enhancement based on the latest revised WID as below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

Discussion

In past meetings, RAN4 has mainly discussed the RRM requirements on SSB periodicity larger than 20ms. There are still some issues to be discussed in this meeting.

Downlink coverage enhancement is a new feature for R19 UE, and RAN4 replied LS to RAN2 on SMTC enhancements in the last meeting towards two issues as follows:

- support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

For the first bullet, we think it is feasible for downlink coverage enhancement. The case is downlink coverage enhancement from the same satellite, and a new default SSB periodicity was introduced by RAN1 which is 160ms. To our understanding, it is not enough to capture all neighbouring cell SSBs if we still use one SMTC periodicity with different offsets. Thus, we deem that it is reasonable to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbour cell measurements. The SMTC periodicity is changed and there is RRM requirements impact.

- For the second bulllet, we observe that UEs conduct measurement based on previous capabilities which are mandatorily support 2 parallel SMTC measurements and optionally 4 parallel SMTC measurements. NTN cells cover a relatively larger area, UE only need to measure the SMTCs of nearby cells within a certain period. For instance, if a UE is located at one corner of a cell, it may not need to measure the cell at the opposite corner. From our perspective, the UE capability is not changed and it is up to UE implementation to choose the SMTCs to be measured.

- In legacy meeting, the wayforward could be seen as below:

## [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6) RAN4 to further discuss the following in RAN4#116-bis: - For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant: - The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE. - Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

- It is said that the SMTC reference in the subsequent requirements refer to the SMTCs selected by the UE. However, RAN2 has a description on smtclist4:

- smtc4list - Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

- It is noted that for a UE that supports less SMTCs than what is included in smtc4list, it is up to select which SMTCs to consider. This means the SMTC selection is up to UE implementation but not all SMTCs will be measured based on UE capabilities, which supports up to 4 SMTCs. If network configures more than 4 SMTCs, UE shall measure less than 4SMTCs. However, in RAN4 spec, it says UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. It is believed that there is a controversial between RAN2 understanding and RAN4 specification description. We shall clarify this point since this impacts defining RAN4 requirements.

- Observation 1: There is a controversial between RAN2 understanding and RAN4 specification description on SMTC selection if more than 4SMTCs are configured by network.

- Proposal 1: RAN4 shall clarify the principle on SMTC selection if more than 4SMTCs are configured with RAN2 understanding.

- For two different SMTC preodicities, RAN2 has one definition as below:

twoSMTC-Periodicities-r19 Indicates whether the UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17.

If UE is capable of twoSMTC-Periodicities-r19, UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. If UE does not support this capability, UE will have same periodicity with smtc1. This will impact the description of RAN4 specification. The concrete description could be discussed in this meeting.

Observation 2: If UE is capable of twoSMTC-Periodicities-r19, UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. If UE does not support this capability, UE will have same periodicity with smtc1.

Proposal 2: RAN4 shall consider RAN4 specification description with RAN2 outcome on two different SMTC priodicities.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN downlink coverage enhancement.

- Observation 1: There is a controversial between RAN2 understanding and RAN4 specification description on SMTC selection if more than 4SMTCs are configured by network.

- Proposal 1: RAN4 shall clarify the principle on SMTC selection if more than 4SMTCs are configured with RAN2 understanding.

Observation 2: If UE is capable of twoSMTC-Periodicities-r19, UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. If UE does not support this capability, UE will have same periodicity with smtc1.

Proposal 2: RAN4 shall consider RAN4 specification description with RAN2 outcome on two different SMTC priodicities.

- 4References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331