# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521542

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for Ku band NTN

Agenda item:	4.2.7.4

Document for:	Approval

Introduction

At RAN #107 meeting the revised WI“Introduction of Ku bands for NR NTN”[1] was approved. In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #116 meeting we will specify the remaining issues on the topics. In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

Discussion

- During last meeting, only one issues shall be clarified as below:

- For soft satellite switch, when SSB of the target satellite does not fall in any SMTC, the scheduling restriction is defined based on a window: - The offset of the window is the latest subframe that is no later than the SSB of the target satellite, - The duration of the window is the smallest number in subframe that is no smaller than the burst length of the SSB of the target satellite, - The periodicity of the window is the periodicity of the SSB of the source satellite

- The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The above table states when SSB of the target satellite does not fall in any SMTC, that means SMTC period equals to target satellite SSB period. Since SMTC period always equals to or larger than SSB periodicity. Otherwise, it is the error case. If all SSB do not fall in any SMTC period, UE will not measure that kind of SSB at all from UE perspective. If all SSB do not fall in any SMTC, the soft satellite switching is failure instead of introducing another window like SMTC. If it is introduced, we deem that this is the generic issue not only about the satellite switching. Consequently, it is not reasonable to introduce a new window same as legacy.

Proposal 1: RAN4 shall not introduce a new window to cover SSB which does not fall in any SMTC.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Proposal 1: RAN4 shall not introduce a new window to cover SSB which does not fall in any SMTC.

- References

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN.