# 3GPP TSG-RAN WG4 Meeting #117                                 R4-2521546

# Dallas, USA, 17th – 21th Nov, 2025

Source:  ZTE Corporation, Sanechips

Title:  Discussion on the RRM requirements of AI/ML Beam management

Agenda Item:	6.11.4

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some initial views on RRM requirements for beam management from the general requirements framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - Beam management - Spatial-domain DL beam prediction - Temporal DL beam prediction

Beam management case 1 is the spatial-domain DL beam prediction for set A of beams based on measurement result of set B beams which the set B beams is the subset beam of set A, the best beams shall be estimated through AI/ML based BM methods with L1 measurements on a beam subset as the figure shown:

Figure 1 AI beam prediction (Spatial domain)

Another sub use case is beam management case 2 which is the time-domain DL beam prediction for set A of beams based on the historic measurement results of set B of beams. The best beam in the future can be predicted through AI/ML based BM methods based on the beam quality at the current and historical time as the figure shown:

Figure 2 AI beam prediction (Time domain)

In this paper, we mainly provide the analysis on the RRM requirements for performance metrics, prediction delay and TCI state activation based on RAN1 and RAN2’s progress and agreements for BM case1 and case 2.

2.1 Performance metrics

Different metrics/KPIs have been discussed and were captured in [2], RAN4 has discussed the related performance metrics for several meetings, the following test metrics are identified and could be considered:

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

We find that performance metrics mainly include two parts, which are RSRP accuracy and beam prediction accuracy. In this section, we shall analyze these two different KPIs respectively.

2.1.1 Absolute RSRP accuracy

RAN4 has discussed several meetings on how to define the RSRP accuracy, including the absolute RSRP accuracy and the relative RSRP accuracy.

In previous meeting, the absolute RSRP accuracy has already been defined as below:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Based on the above agreement, we shall distinguish what is the benchmark of top-K beams. Since the formula only tells us the index i may be any beam index and not sure the index of top-K beams shall be predicted beams or groundtruth beams. There are two choices which are the top-K beams based on groundtruth L1-RSRP and the top-K beams based on predicted L1-RSRP. In my opinion, the top-K beams shall be the predicted beams instead of the groundtruth beams. Since the purpose of the absolute RSRP accuracy is to verify the performance of AI model based on the predicted beams or UE reported several optimal predicted beams. Thus, RAN4 shall check the reported predicted beams with groundtruth beams whether the reported top-K beams are predicted better or worse. For instance, the top-4 reported beam indices are beam 1, beam 2, beam 3 and beam 4, the L1-RSRP sequence of groundtruth beams indices are beam 1, beam 4, beam 5, beam 6, beam 3, beam 2 and others. If the index equals 1, the absolute RSRP accuracy may be small, however, if the index equals 2, the absolute RSRP accuracy may be large. Then, it means that the performance of AI model is not good enough. On the contrary, for the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

On the other hand, if the index equals 1, the absolute RSRP accuracy may be small, however, if the index equals 2, the absolute RSRP accuracy may be large. Then, it means that the performance of AI model is not good enough. What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied. From my perspective, a margin could be considered, that is, if the L1-RSRP difference of same beam exceeds the margin, the difference is large. Otherwise, the difference is small. However, the absolute RSRP accuracy in legacy can be seen as below:

Table 1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

To my understanding, option 1 and option 2 are fine for us, the proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

- Observation 1: The top-K beams are reported or predicted beams or top-K beams are groundtruth beams.

- Observation 2: For the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

- Observation 3: What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied.

Observation 4: One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

- Proposal 1: For the absolute RSRP accuracy, the index i shall be the top-K beams based on the predicted beams or UE reported beams instead of the groundtruth beams.

- Proposal 2: One margin for the absolute RSRP accuracy shall be studied in order to verify the performance of AI system. The proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

In the last meeting, we also had one remaining issue on absolute RSRP accuracy as below:

- Proposals - Option 1: applicable only to top predicted beam - Option 2: should apply to all K predicted beams - Option 3: postpone this discussion to the performance part as it relates to accuracy - Option 4: others - Recommended WF - Option 3

We firstly agreed that absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least. But whether to apply to other beams has not been decided yet. To our understanding, the absolute RSRP accuracy shall be applied to all K predicted beams.

Actually, RAN1 has already defined the absolute RSRP reporting format as below:

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

We observe that RSRP#1 is absolute predicted RSRP, the remaining RSRP#2~#K are differential predicted RSRP with reference to the largest predicted RSRP, which is similar to R15 CSI reporting defined in RAN1 spec:

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size. If the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance.

If we follow the same manner as RAN1, we have to admit that the absolute RSRP format shall only used for Top-1 predicted beams which can reduce the signaling overhead.

Nevertheless, RAN4 shall study the accuracy requirements based on RAN4’s spec. In current RAN4 spec, we do not observe the limitation on absolute RSRP accuracy:

Unless otherwise specified, the requirements for absolute and relative SSB based L1-RSRP accuracy in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement and all SSBs of cell(s) with different PCI from serving cell configured for L1-RSRP measurement in FR1.

We could see the above definition and applicability, which the absolute SSB based L1-RSRP accuracy in this part apply to all SSBs not only to the largest measured SSB. If absolute RSRP accuracy only applies to Top-1 predicted beam like R15 CSI reporting, there is no doubt that the relative ranking can be confirmed. But only absolute RSRP accuracy can reflect the real ranking and real measurements, i.e. Top-1 predicted beam 1 is -90dB (-89dB actually), Top-2 predicted beam 2 is -92dB (-85dB actually), the ranking seems correct if we only focus on relative ranking, but the absolute RSRP accuracy has 7dB error on the predicted beam except Top-1 predicted beam. Consequently, in order to get a comprehensive guarantee on the AI/ML model performance, RAN4 shall not consider what RAN1 has decided but rather focus on how RAN4 should proceed.

Overall, we prefer to considering absolute RSRP accuracy requirement applies to Top-1 of predicted beams and other beams.

Observation 5: In current RAN4 spec, we do not observe the limitation on absolute RSRP accuracy.

Proposal 3: RAN4 shall consider absolute RSRP accuracy requirement applying to Top-1 of predicted beams and other beams.

2.1.2 Relative RSRP accuracy

In the last meeting, RAN4 has eventually reached the agreement on defining the relative RSRP accuracy requirements as below:

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

We know that this metric refers to AI mobility, the relative RSRP accuracy in AI mobility is cell level but in AI beam is the beam level. The whole principle for defining relative RSRP accuracy is that the predicted L1-RSRP minus ground truth beams which is similar to legacy definition:

Legacy including ICBM related: The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements. LTM related: The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured on one SSB configured in LTM-CSI-ResourceConfig compared to the L1-RSRP measured from another SSB configured in LTM-CSI-ResourceConfig from the cell(s) on the same frequency in FR2.

From our perspective, the reported L1-RSRP shall be predicted RSRP not the measured RSRP. If the reported L1-RSRP is reported measured RSRP, this components will disappear based on current formulation which is not reasonable. Consequently, the reported L1-RSRP shall be reported predicted L1-RSRP not the measured RSRP.

Observation 6: The whole principle for defining relative RSRP accuracy is that the predicted L1-RSRP minus ground truth beams which is similar to legacy definition.

Observation 7: If the reported L1-RSRP is reported measured RSRP, this components will disappear based on current formulation.

Proposal 4: The reported L1-RSRP shall be reported predicted L1-RSRP not the measured RSRP.

2.1.3 Metrics/KPIs for beam ID prediction

In previous meeting, we spent lots of time on discussion of beam prediction accuracy, the agreement could be seen as below:

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Agreement: - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

We observe that modified option 3 is adopted. There are two options x is larger than 0 and x equals to 0, to my understanding, at least x equals to 0 shall be supported since it means Top-K/1(%) if x=0. Based on simulation results, we find that Top-1/1 does not work at all and this performance metric has already been precluded in last meeting. Top-K/1 is the relaxation of Top-1/1, UE vendors has extended margin on this performance metric and the prediction accuracy can be up to 97% even the measurement error has been added. Thus, we deem that at least x equals to 0 shall be considered. For x>0, this shall be decided on simulation results. However, one thing shall be noticed that x shall not be too large. If the value of x is set relatively high, the performance metric will show some excellent results, making it easy for UE to pass the test and indirectly suggesting that AI performance is very good. But if the margin is set too large, the threshold for performance validation becomes low enough that any UE can pass the test, leading to the realization that AI’s predictive capabilities are actually not as good as legacy methods. This would be meaningless. Consequently, the value of x shall be considered carefully.

Observation 8: Top-K/1 is the relaxation of Top-1/1, UE vendors has extended margin on this performance metric and the prediction accuracy can be up to 97% even the measurement error has been added.

Observation 9: If the margin is set too large, the threshold for performance validation becomes low enough that any UE can pass the test, leading to the realization that AI’s predictive capabilities are actually not as good as legacy methods.

Proposal 5: At least x=0 shall be considered in performance metrics.

Proposal 6: The concrete value of x when x>0 shall be defined based on simulation results.

Proposal 7: The value of x shall not be set too large in order to guarantee the AI performance test and prevent UE easily passes the test.

Another issue is whether Top-K/N is introduced. In the last meeting, we spent lots of time on discussing this issue without any consensus. However, we observed that the interpretation of Top-K/N is not aligned by companies. For this issue, we would like to split two parts as below:

- Whether to introduce Top-N beams.

- From network perspective, we prefer to only adopting Top-K/1 as final metric which also has high simulation results in the last section. Thus, Top-N beams are not necessary for us. However, most companies have strong will to define that because of consideration of the whole system performance, i.e. the maximum ground truth beam among Top-K beams has small difference or equal to Top-1 beam if only Top-1 beam is considered. The test will definitely pass. Nonetheless, if the rest of beams in Top-K predicted beams are worse beams, the whole AI/ML performance will be wrongly judged as successful. For this reason, most companies would like to introduce Top-N beam to ensure high reliability of AI/ML method. Consequently, we support to introduce Top-N beams for beam ID prediction.

- Observation 10: the maximum ground truth beam among Top-K beams has small difference or equal to Top-1 beam if only Top-1 beam is considered. The test will definitely pass. Nonetheless, if the rest of beams in Top-K predicted beams are worse beams, the whole AI/ML performance will be wrongly judged as successful.

- Proposal 8: RAN4 shall support to introduce Top-N beams for beam ID prediction to ensure high reliability of AI/ML.

- How to define Top-K/N% metric.

- The next step is to define Top-K/N%, but we observed that different companies have different interpretation on this metric, we summarize three different understandings as below:

- One of Top-N beams contains in Top-K predicted beams; (like RAN1’s agreement on performance monitoring)

- All of Top-N beams contains in Top-K predicted beams with corresponding order.

- All of Top-N beams contains in Top-K predicted beams without corresponding order.

- For interpretation A, we know the motivation of this understanding, which is that the difference among Top-N beams is too small so that can be ignored. UE may not report correct order and the best beam will be the second best beam or the third best beam, thus the proponents would like to select one of Top-N beams as benchmark. However, we think the test will be passed easily if only one of Top-N in Top-K predicted beams. From network perspective, this interpretation shall be precluded.

- For interpretation B, the motivation of this understanding is the opposite side of interpretation A. The detailed understanding is as below:

- The ground-truth RSRP among 1st  predicted beams larger than or equal to the ground-truth RSRP of the 1st strongest  genie-aided beam(s) – x1 dB,

- The ground-truth RSRP among 2nd  predicted beams larger than or equal to the ground-truth RSRP of the 2nd strongest  genie-aided beam(s) – x2 dB,

- …

- The ground-truth RSRP among Nth predicted beams larger than or equal to the ground-truth RSRP of the Nth  strongest genie-aided beam(s) – xV dB

- It appears that the proposed solution may impose more stringent requirements than those previously agreed upon, as it involves RSRP verification of each predicted beam in Top-N beams. Compared to interpretation A, this metric will not cause test passes easily, but this metric has large probability that the test will not be passed since the conditions are quite strict. We would like to deprioritize this interpretation.

- For interpretation C, this understanding is proposed by us. We observe that this metric not only consider the whole system performance but no order is requested. We only guarantee all Top-N beams contain in Top-K beams, Top-N beams are measured by UE which can be assumed as groundtruth beams. That is, if all groundtruth beams contain in AI/ML predicted Top-K beams, the test will be passed. Consequently, interpretation C can be the compromise understanding.

- Proposal 9: Different interpretation of Top-K/N% could be seen as below:

- One of Top-N beams contains in Top-K predicted beams; (like RAN1’s agreement on performance monitoring)

- All of Top-N beams contains in Top-K predicted beams with corresponding order.

- All of Top-N beams contains in Top-K predicted beams without corresponding order.

- Observation 11: For interpretation A, the test will be passed easily if only one of Top-N in Top-K predicted beams.

- Observation 12: For interpretation B, compared to interpretation A, this metric will not cause test passes easily, but this metric has large probability that the test will not be passed since the conditions are quite strict.

- Observation 13: For interpretation C, only guarantee all Top-N beams contain in Top-K beams, Top-N beams are measured by UE which can be assumed as groundtruth beams. That is, if all groundtruth beams contain in AI/ML predicted Top-K beams, the test will be passed.

- Proposal 10: The definition of Top-K/N% shall be all of Top-N beams contains in Top-K predicted beams without corresponding order.

2.2 Measurement period for inference

In the last meeting, prediction delay requirements have been discussed with agreements as below:

Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

We have studied the whole AI beam prediction delay which contains three parts. Actually, we shall define the separate delay requirements and then combine with them to confirm the whole AI beam prediction delay.

For measurement delay, the legacy requirements is below:

Table 2 Measurement period for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Based on the current spec, the legacy L1-RSRP measurement period is based on one-shot measurement. One shot measurement means the number of measured sample is one. For L1-RSRP measurement period, the number of M equals 1 or 3, where M=3 includes two samples to do the AGC and only one sample is used for measurement. The L1-RSRP measurement period owns sharing factor ”P”, number of sample “M” and Rx beam sweeping factor “N” in FR2. This measurement is about Set B measurement which is legacy L1-RSRP measurement, we have already agreed with reusing legacy values for M,N,P for case 1. For case 2, UE shall measure several historical samples and then to predict the future instances. Thus, we deem that M shall be extended by scaling factor T, that is, T*M. we do not prefer to introduce additional UE capability since the concrete value for T is much better. If additional UE capability is introduced, we shall also discuss the concrete value for different capability. Otherwise, it will be up to UE implementation. So we would like to define the one worst value of T as traditional RRM requirements defining principle. For T, we would like to follow RAN1’s agreement as below:

Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

We find that the number of predicted future time instances is [1,2,4,8], the total measurement samples may refer to the number of predicted future time instances. If T=1, the measurement samples may be too less to predict more accurate prediction of future time instance. If T=8, the measurement samples may be too much and the training model will be over-fitting. Consequently, we prefer T equals 2 or 4, this could be discussed in this meeting.

In the last meeting, we still have one agreement as below:

Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1

It is observed that M is with bracket. To our understanding, the M values shall be removed directly since the concept of K and M are duplicated. In legacy L1 measurement, M represents the number of measurement samples. If we follow RAN1’s definition, K also means number of samples. If M is not precluded, K*M seems weird. Consequently, we deem that the measurement period for inference for case 2 shall be K*N*P, where K means number of samples.

Observation 14: The total measurement samples may refer to the number of predicted future time instances. If T=1, the measurement samples may be too less to predict more accurate prediction of future time instance. If T=8, the measurement samples may be too much and the training model will be over-fitting.

Proposal 12: For measurement delay in case 2, the T value could follow the number of future time instances proposed by RAN1, which is [2 or 4].

Proposal 13: The measurement period for inference for case 2 shall be K*N*P, where K means number of samples.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML beam management:

- Observation 1: The top-K beams are reported or predicted beams or top-K beams are groundtruth beams.

- Observation 2: For the top-K beams are groundtruth beams but not the predicted beams, may be some UE reported beams could not be verified or why should we require the RSRP accuracy of the beam that the UE doesn’t even report.

- Observation 3: What is the standard to verify the performance of AI system based on the absolute RSRP accuracy shall be considered or how to quantify the the small or large L1-RSRP difference shall be studied.

Observation 4: One question is that the proposed margin shall be maintained or smaller than legacy value. There are two options to consider the change of proposed margin:

- Option 1: Maintain the legacy measurement accuracy. In legacy, UE shall do the full beam sweeping in order to confirm the best Rx beam to receive the downlink RS and get the better communication with network. The AI system has already improve the overhead and reduce the complexity than legacy. Thus, maintain the legacy measurement accuracy could be understood.

- Option 2: Smaller than legacy measurement accuracy. This is a more direct understanding since the AI performance shall have the better performance than legacy and the measurement accuracy shall be tightened. In this way, the performance of AI method could be displayed.

- Proposal 1: For the absolute RSRP accuracy, the index i shall be the top-K beams based on the predicted beams or UE reported beams instead of the groundtruth beams.

- Proposal 2: One margin for the absolute RSRP accuracy shall be studied in order to verify the performance of AI system. The proposed margin could be equal to and smaller than the legacy measurement accuracy requirements.

Observation 5: In current RAN4 spec, we do not observe the limitation on absolute RSRP accuracy.

Proposal 3: RAN4 shall consider absolute RSRP accuracy requirement applying to Top-1 of predicted beams and other beams.

Observation 6: The whole principle for defining relative RSRP accuracy is that the predicted L1-RSRP minus ground truth beams which is similar to legacy definition.

Observation 7: If the reported L1-RSRP is reported measured RSRP, this components will disappear based on current formulation.

Proposal 4: The reported L1-RSRP shall be reported predicted L1-RSRP not the measured RSRP.

Observation 8: Top-K/1 is the relaxation of Top-1/1, UE vendors has extended margin on this performance metric and the prediction accuracy can be up to 97% even the measurement error has been added.

Observation 9: If the margin is set too large, the threshold for performance validation becomes low enough that any UE can pass the test, leading to the realization that AI’s predictive capabilities are actually not as good as legacy methods.

Proposal 5: At least x=0 shall be considered in performance metrics.

Proposal 6: The concrete value of x when x>0 shall be defined based on simulation results.

Proposal 7: The value of x shall not be set too large in order to guarantee the AI performance test and prevent UE easily passes the test.

- Observation 10: the maximum ground truth beam among Top-K beams has small difference or equal to Top-1 beam if only Top-1 beam is considered. The test will definitely pass. Nonetheless, if the rest of beams in Top-K predicted beams are worse beams, the whole AI/ML performance will be wrongly judged as successful.

- Proposal 8: RAN4 shall support to introduce Top-N beams for beam ID prediction to ensure high reliability of AI/ML.

- Proposal 9: Different interpretation of Top-K/N% could be seen as below:

- One of Top-N beams contains in Top-K predicted beams; (like RAN1’s agreement on performance monitoring)

- All of Top-N beams contains in Top-K predicted beams with corresponding order.

- All of Top-N beams contains in Top-K predicted beams without corresponding order.

- Observation 11: For interpretation A, the test will be passed easily if only one of Top-N in Top-K predicted beams.

- Observation 12: For interpretation B, compared to interpretation A, this metric will not cause test passes easily, but this metric has large probability that the test will not be passed since the conditions are quite strict.

- Observation 13: For interpretation C, only guarantee all Top-N beams contain in Top-K beams, Top-N beams are measured by UE which can be assumed as groundtruth beams. That is, if all groundtruth beams contain in AI/ML predicted Top-K beams, the test will be passed.

- Proposal 10: The definition of Top-K/N% shall be all of Top-N beams contains in Top-K predicted beams without corresponding order.

Proposal 11: The potential components in prediction delay contains three parts:

- Measurement delay: The time for measurement of SetB which is the input for inference.

- Inference delay: The time for inference from starting performing inference to generate the prediction results.

- Reporting delay: The time between the report triggering and the point when the UE starts to transmit the outputs over the air.

Proposal 12: For measurement delay, the legacy L1-RSRP measurement period shall be reused.

Proposal 13: The measurement period for inference for case 2 shall be K*N*P, where K means number of samples.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2420513, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2417890, “Discussion on core requirements of AI/ML BM use case”, Media Tek Inc.