# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521549

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for RedCap NTN

Agenda item:	6.19.3.1

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on NR NTN phase 3.

Discussion

In this section, we would like to provide our initial views on performance requirements on two different topics, which are redcap UEs in NTN scenario and downlink coverage enhancement. In this meeting, we mainly study general scope of these two topics.

For test configuration, we shall consider GSO and NGSO, FDD and HD-FDD factor for RedCap UE with FR1-NTN. Two points shall be noted as below:

- In legacy NTN test configuration, GSO and NGSO shall be both configured. If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- In legacy RedCap test configuration, both FDD and HD-FDD duplex are configured, but the UE is only required to be tested in one of the supported test configurations. Following this logic, we deem that if (e)RedCap UE supports both FDD and HD-FDD operation, the NTN capable UE is only required to be tested in one of both.

We shall consider above noted points in final test configuration. The final test configuration could be seen as below:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Proposal 1: The final test configuration could be:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

For test case list, RAN4 still have several concerning test cases as below:

Cell reselection: - FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN Conditional handover: - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN Signalling characteristics: - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN

Considering trade-off the number of test cases and comprehensiveness of testing, we have following suggestions:

For cell reselection, the FFS parts could be removed since the existing test cases have already covered location-/time-based intra-/inter- frequency measurement.

For conditional handover, the same reason as test cases defined for cell reselection and FFS part shall be removed.

For signalling characteristics, the same reason as test cases defined for cell reselection and FFS part shall be removed.

For UE transmit timing and timing advance for RedCap with satellite access, we strongly to define test cases with 1Rx. For 2Rx RedCap UE, it seems like normal UE with 2Rx. Consequently, only NTN capable RedCap UE with 1Rx has been never defined, the timing related test cases shall be considered only for 1Rx RedCap UE.

Proposal 2: Regarding FFS part in test case list, the following shall be considered:

For cell reselection, the FFS parts could be removed since the existing test cases have already covered location-/time-based intra-/inter- frequency measurement.

For conditional handover, the same reason as test cases defined for cell reselection and FFS part shall be removed.

For signalling characteristics, the same reason as test cases defined for cell reselection and FFS part shall be removed.

Proposal 3: Test cases for UE transmit timing and timing advance, only 1Rx RedCap UE shall be defined.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN phase 3:

Proposal 1: The final test configuration could be:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Proposal 2: Regarding FFS part in test case list, the following shall be considered:

For cell reselection, the FFS parts could be removed since the existing test cases have already covered location-/time-based intra-/inter- frequency measurement.

For conditional handover, the same reason as test cases defined for cell reselection and FFS part shall be removed.

For signalling characteristics, the same reason as test cases defined for cell reselection and FFS part shall be removed.

Proposal 3: Test cases for UE transmit timing and timing advance, only 1Rx RedCap UE shall be defined.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133 Clause A.14.