3GPP TSG-RAN WG4 Meeting #117	R4-2521562

# Dallas, USA, 17th – 25th

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case on RRC IDLE mobility for RedCap UE in NTN scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case for requirements in RRC IDLE state shall be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the test case for requirements in RRC IDLE state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No any test case for requirements for RedCap UEs in NTN scenario in RRC IDLE state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (NEW) A.20.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.20.1	RRC_IDLE state mobility

A.20.1.1	Cell reselection to FR1 intra-frequency NR case for 1Rx RedCap UE

A.20.1.1.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.1.1 shall apply for 1Rx RedCap UE.

A.20.1.1.2	Test Parameters

Test parameters in clause A.14.1.1.2 sahll apply except that:

- Table A.14.1.1.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.1.2-3 is replaced with A.20.1.1.2-3.

- Table A.20.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

Table A.14.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for Config 1,3 SSC.2 for Config 2,4 |  |  | NSC.1 for Config 1,3 NSC.2 for Config 2,4 |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.5 for Cell 2 |  |  | #1: SMTC.6 for Cell 1 #2: SMTC.2 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -82 | -infinity | -82 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

A.20.1.1.3	Test Requirements

Test requirements in clause A.14.1.1.3 shall apply for 1Rx RedCap UEs.

A.20.1.2	Cell reselection to FR1 intra-frequency NR case  for 2Rx RedCap UE

A.20.1.2.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.1.1 shall apply for 1Rx RedCap UE.

A.20.1.2.2	Test Parameters

Test parameters in clause A.14.1.1.2 sahll apply except that:

- Table A.14.1.1.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.1.2-3 is replaced with A.20.1.1.2-3.

A.20.1.2.3	Test Requirements

Test requirements in clause A.14.1.1.3 shall apply for 2Rx RedCap UEs.

A.20.1.3	Cell reselection to FR1 intra-frequency NR cell for 1Rx RedCap UE configured with the feature for enhanced requirements

A.20.1.3.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.2.1 shall apply for 1Rx RedCap UE.

A.20.1.3.2	Test Parameters

Test parameters in clause A.14.1.2.2 sahll apply except that:

- Table A.14.1.2.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.2.2-3 is replaced with A.20.1.3.2-3.

- Table A.20.1.3.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for Config 1,3 SSC.2 for Config 2,4 |  |  | NSC.1 for Config 1,3 NSC.2 for Config 2,4 |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.5 for Cell 2 |  |  | #1: SMTC.6 for Cell 1 #2: SMTC.2 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -82 | -infinity | -82 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

A.20.1.3.3	Test Requirements

Test requirements in clause A.14.1.2.3 shall apply for 1Rx RedCap UEs.

A.20.1.3	Cell reselection to FR1 intra-frequency NR cell for 2Rx RedCap UE configured with the feature for enhanced requirements

A.20.1.3.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.2.1 shall apply for 2Rx RedCap UE.

A.20.1.3.2	Test Parameters

Test parameters in clause A.14.1.2.2 sahll apply except that:

- Table A.14.1.2.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.2.2-3 is replaced with A.20.1.3.2-3.

A.20.1.3.3	Test Requirements

Test requirements in clause A.14.1.2.3 shall apply for 2Rx RedCap UEs.

A.20.1.4	Time-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx RedCap UE

A.20.1.4.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.3.1 shall apply for 1Rx RedCap UE.

A.20.1.4.2	Test Parameters

Test parameters in clause A.14.1.3.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.3.2-3 is replaced with A.20.1.4.2-3.

- Table A.14.1.3.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1,3 SSC.2 for Config 2,4 |  | NSC.1 for Config 1,3 NSC.2 for Config 2,4 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 and Cell 2 |  | #1: SMTC.6 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | -3.11 | -infinity | 2.79 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -52.21 | Same as parameters specified in Cell 1 columns- |  |
| Treselection | s | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 40 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

A.20.1.4.3	Test Requirements

Test requirements in clause A.14.1.3.3 shall apply for 1Rx RedCap UEs.

A.20.1.5	Time-based measurement initiation to FR1 intra-frequency NR cell reselection for 2Rx RedCap UE

A.20.1.5.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.3.1 shall apply for 2Rx RedCap UE.

A.20.1.5.2	Test Parameters

Test parameters in clause A.14.1.3.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.3.2-3 is replaced with A.20.1.4.2-3.

A.20.1.5.3	Test Requirements

Test requirements in clause A.14.1.3.3 shall apply for 2Rx RedCap UEs.

A.20.1.6	Location-based measurement initiation to FR1 inter-frequency NR cell reselection for 1Rx RedCap UE

A.20.1.6.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.8.1 shall apply for 1Rx RedCap UE.

A.20.1.6.2	Test Parameters

Test parameters in clause A.14.1.8.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.8.2-3 is replaced with A.20.1.6.2-3.

- Table A.14.1.8.2-3: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1,3 SSC.2 for Config 2,4 |  | NSC.1 for Config 1,3 NSC.2 for Config 2,4 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | 13 | -infinity | 16 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -56.84 | -70.05 | -53.94 |
| Treselection | s | 0 | 0 | 0 | 0 |
| SnonIntrasearchP | dB | 50 |  | 50 |  |
| Threshx, highP | dB | 48 |  | 48 |  |
| Threshserving, lowP | dB | 54 |  | 44 |  |
| Threshx, lowP | dB | 50 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

A.20.1.6.3	Test Requirements

Test requirements in clause A.14.1.8.3 shall apply for 1Rx RedCap UEs.

A.20.1.7	Location-based measurement initiation to FR1 inter-frequency NR cell reselection for 2Rx RedCap UE

A.20.1.7.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.8.1 shall apply for 2Rx RedCap UE.

A.20.1.7.2	Test Parameters

Test parameters in clause A.14.1.8.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.8.2-3 is replaced with A.20.1.6.2-3.

A.20.1.7.3	Test Requirements

Test requirements in clause A.14.1.8.3 shall apply for 2Rx RedCap UEs.

A.20.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion for 1Rx RedCap UE

A.20.1.8.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.9.1 shall apply for 1Rx RedCap UE.

A.20.1.8.2	Test Parameters

Test parameters in clause A.14.1.9.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.9.2-2 is replaced with A.20.1.8.2-2, and,

- Table A.14.1.9.2-3 is replaced with A.20.1.8.2-3.

- Table A.20.1.8.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2,3,4 | Cell 2 | The UE camps on Cell 2 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to Cell 1 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2,3,4 | Cell 1 | The UE shall perform reselection to Cell 1 during T1 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 2 |  |
| T2 end condition | Active cell |  | 1, 2,3,4 | Cell 2 | The UE shall perform reselection to Cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2,3,4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2,3,4 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1, 2,3,4 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1, 2,3,4 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
| DRX cycle length |  | s | 1, 2,3,4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2,3,4 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2,3,4 | Not configured |  |
| T1 |  | s | 1, 2,3,4 | 25 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2,3,4 | 25 s | T2 is defined so that cell re-selection reaction time is taken into account. |

- Table A.20.1.8.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| Satellite information |  | 1,3 | SSC.1 |  | NSC.1 |  |  |
|  |  | 2,4 | SSC.2 |  | NSC.2 |  |  |
| PDSCH RMC configuration |  | 1, 2,3,4 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2,3,4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2,3,4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2,3,4 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2,3,4 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2,3,4 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2,3,4 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2,3,4 | -140 |  | -140 |  |  |
| Pcompensation | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2,3,4 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1, 2,3,4 | 14 | 14 | -4 | 12 |  |
| Note2 | dBm/SCS | 1, 2,3,4 | -98 |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2,3,4 | -98 |  |  |  |  |
|  | dB | 1, 2,3,4 | 14 | 14 | -4 |  | 12 |
| SS-RSRP Note3 | dBm/SCS | 1, 2,3,4 | -84 | -84 | -102 |  | -86 |
| Io | dBm/9.36 MHz | 1, 2,3,4 | -55.88 | -55.88 | -68.60 |  | -57.78 |
| Treselection | s | 1, 2,3,4 | 0 | 0 | 0 |  | 0 |
| SnonintersearchP | dB | 1, 2,3,4 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2,3,4 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2,3,4 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2,3,4 | 50 |  | 50 |  |  |
| SSearchDeltaP | dB | 1, 2,3,4 | 3 |  | 3 |  |  |
| TSearchDeltaP | s | 1, 2,3,4 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2,3,4 | AWGN |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.20.1.8.3	Test Requirements

Test requirements in clause A.14.1.9.3 shall apply for 1Rx RedCap UEs.

A.20.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion for 2Rx RedCap UE

A.20.1.9.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.9.1 shall apply for 2Rx RedCap UE.

A.20.1.9.2	Test Parameters

Test parameters in clause A.14.1.9.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.9.2-2 is replaced with A.20.1.8.2-2, and,

- Table A.14.1.9.2-3 is replaced with A.20.1.8.2-3.

A.20.1.9.3	Test Requirements

Test requirements in clause A.14.1.9.3 shall apply for 2Rx RedCap UEs.

A.20.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion for 1Rx RedCap UEs

A.20.1.10.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.10.1 shall apply for 1Rx RedCap UE.

A.20.1.10.2	Test Parameters

Test parameters in clause A.14.1.10.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.10.2-2 is replaced with A.20.1.10.2-2, and,

- Table A.14.1.10.2-3 is replaced with A.20.1.10.2-3.

- Table A.20.1.10.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2,3,4 | Cell 2 | The UE camps on Cell 2 in the initial phase, it fulfills Not-at-cell edge relaxation measurements criterion, and during T1 period the UE reselects to Cell 1 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2,3,4 | Cell 1 | The UE shall perform reselection to Cell 1 during T1 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 2 |  |
| T2 end condition | Active cell |  | 1, 2,3,4 | Cell 2 | The UE shall perform reselection to Cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2,3,4 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2,3,4 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2,3,4 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2,3,4 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1, 2,3,4 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1, 2,3,4 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
| DRX cycle length |  | s | 1, 2,3,4 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2,3,4 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2,3,4 | Not configured |  |
| T1 |  | s | 1, 2,3,4 | 20 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2,3,4 | 20 s | T2 is defined so that cell re-selection reaction time is taken into account. |

- Table A.20.1.10.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| Satellite information |  | 1,3 | SSC.1 |  | NSC.1 |  |  |
|  |  | 2,4 | SSC.2 |  | NSC.2 |  |  |
| PDSCH RMC configuration |  | 1, 2,3,4 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2,3,4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2,3,4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2,3,4 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2,3,4 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2,3,4 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2,3,4 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2,3,4 | -140 |  | -140 |  |  |
| Pcompensation | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2,3,4 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2,3,4 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1, 2,3,4 | 14 | 14 | -4 | 12 |  |
| Note2 | dBm/SCS | 1, 2,3,4 | -98 |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2,3,4 | -98 |  |  |  |  |
|  | dB | 1, 2,3,4 | 14 | 14 | -4 |  | 12 |
| SS-RSRP Note3 | dBm/SCS | 1, 2,3,4 | -84 | -84 | -102 |  | -86 |
| Io | dBm/9.36 MHz | 1, 2,3,4 | -55.88 | -55.88 | -68.60 |  | -57.78 |
| Treselection | s | 1, 2,3,4 | 0 | 0 | 0 |  | 0 |
| SnonintersearchP | dB | 1, 2,3,4 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2,3,4 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2,3,4 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2,3,4 | 50 |  | 50 |  |  |
| SSearchThresholdP | dB | 1, 2,3,4 | 50 |  | 50 |  |  |
| SSearchThresholdQ | s | 1, 2,3,4 | Not Configured |  |  |  |  |
| Propagation Condition |  | 1, 2,3,4 | AWGN |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.20.1.10.3	Test Requirements

Test requirements in clause A.14.1.10.3 shall apply for 1Rx RedCap UEs.

A.20.1.11	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion for 2Rx RedCap UEs

A.20.1.11.1	Test Purpose and Environment

Test purpose and environment in clause A.14.1.10.1 shall apply for 2Rx RedCap UE.

A.20.1.11.2	Test Parameters

Test parameters in clause A.14.1.10.2 sahll apply except that:

- Table A.14.1.3.2-1 is replaced with A.20.1.1.2-1, and

- Table A.14.1.10.2-2 is replaced with A.20.1.10.2-2, and,

- Table A.14.1.10.2-3 is replaced with A.20.1.10.2-3.

A.20.1.11.3	Test Requirements

Test requirements in clause A.14.1.10.3 shall apply for 2Rx RedCap UEs.

A.20.1.12	Cell reselection to FR1 inter-RAT for NR NTN carrier for 1rx RedCap UE

A.20.1.12.1	Test purpose and Environment

Test purpose and environment in clause A.14.1.11.1 shall apply for 1Rx RedCap UE.

A.20.1.12.2	Test Parameters

- Table A.14.1.11.2-1 is replaced with A.20.1.12.2-1, and

- Table A.14.1.11.2-2 is replaced with A.20.1.12.2-2, and,

- Table A.14.1.11.2-3 is replaced with A.20.1.12.2-3, and,

- Table A.14.1.11.2-4 is replaced with A.20.1.12.2-4

- A.20.1.12.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| 4 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 6 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| NOTE 1: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases, and the UE is only required to be tested in one of the supported test configurations of the applicable scenario (GSO or NGSO). NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |  |

Table A.20.1.11.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1-6 | Cell 1 | The UE camps on Cell 1 in the initial phase and during T2 period the UE reselects to Cell 2. |
| T2 end | Active cell |  | 1-6 | Cell 2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1-6 | Cell 1 | 2 during T2. |
| Access Barring Information |  | - | 1-6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1-6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1-6 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1-6 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
| T1 |  | s | 1-6 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1-6 | 70 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.20.1.11.2-3: Cell specific test parameters for NR Cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Satellite information |  | 1,3,5 | SSC.1 |  |
|  |  | 2,4,6 | SSC.2 |  |
| TDD configuration |  | 1-6 | N/A |  |
| PDSCH parameters |  | 1-6 | SR.1.1 FDD |  |
| RMSI CORESET parameters |  | 1-6 | CR.1.1 FDD |  |
| Dedicated CORESET parameters |  | 1-6 | CCR.1.1 FDD |  |
| SSB parameters |  | 1-6 | SSB.1 FR1 |  |
| NR SMTC parameters |  | 1-6 | SMTC.2 |  |
| OCNG Pattern |  | 1-6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1-6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1-6 | ULBWP.0.1 |  |
| RLM-RS |  | 1-6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1-6 | -140 |  |
|  | dBm/SCS | 1-6 | -98 |  |
|  | dBm/15 kHz | 1-6 | -98 |  |
| SS-RSRP | dBm/SCS | 1-6 | -84 | -84 |
|  | dB | 1-6 | 14 | 14 |
|  | dB | 1-6 | 14 | 14 |
| Io | dBm/9.36 MHz | 1-6 | -55.88 | -55.88 |
| Treselection | s | 1-6 | 0 |  |
| SnonintrasearchP | dB | 1-6 | 50 |  |
| Threshx, highP (Note 2) | dB | 1-6 | 48 |  |
| Threshserving, lowP | dB | 1-6 | 44 |  |
| Threshx, lowP | dB | 1-6 | 50 |  |
| Propagation Condition |  | 1-6 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.20.1.11.2-4: Cell specific test parameters for E-UTRA Cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2,5 OP.2 FDD for test configuration 3, 4,6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -infinity | -86 |
|  | dB | -infinity | 12 |
|  | dB | -infinity | 12 |
| TreselectionEUTRAN | s | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP (Note 2) | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

A.20.1.12.3	Test requirements

Test requirements in clause A.14.1.11.3 shall apply for 1Rx RedCap UEs.

A.20.1.13	Cell reselection to FR1 inter-RAT for NR NTN carrier for 2Rx RedCap UE

A.20.1.13.1	Test purpose and Environment

Test purpose and environment in clause A.14.1.11.1 shall apply for 2Rx RedCap UE.

A.20.1.13.2	Test Parameters

- Table A.14.1.11.2-1 is replaced with A.20.1.12.2-1, and

- Table A.14.1.11.2-2 is replaced with A.20.1.12.2-2, and,

- Table A.14.1.11.2-3 is replaced with A.20.1.12.2-3, and,

- Table A.14.1.11.2-4 is replaced with A.20.1.12.2-4

A.20.1.13.3	Test requirements

Test requirements in clause A.14.1.11.3 shall apply for 2Rx RedCap UEs.

A.20.1.14	Cell re-selection to FR1 inter-frequency NR case with TN carrier for 1Rx RedCap UE

A.20.1.14.1	Test purpose and Environment

Test purpose and environment in clause A.14.1.12.1 shall apply for 1Rx RedCap UE.

A.20.1.14.2	Test parameters

- Table A.14.1.12.2-1 is replaced with A.20.1.14.2-1, and

- Table A.14.1.12.2-2 is replaced with A.20.1.14.2-2, and,

- Table A.14.1.12.2-3 is replaced with A.20.1.14.2-3, and,

- Table A.14.1.12.2-4 is replaced with A.20.1.14.2-4

- Table A.20.1.14.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 7 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 9 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 10 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 11 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 12 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE1: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases, and the UE is only required to be tested in one of the supported test configurations of the applicable scenario (GSO or NGSO). NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |  |

- TableA.20.1.14.2-2: General test parameters for inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1-12 | Cell 1 |  |
| T2 end condition | Active cell |  | 1-12 | Cell 2 |  |
|  | Neighbour cells |  | 1-12 | Cell 1 |  |
| RF Channel Number |  |  | 1-12 | 1,2 | Cell 1 is on RF channel 1 Cell 2 is on RF channel 2 |
| Time offset between cells |  |  | 1-12 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1-12 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1-12 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1-12 | SMTC.6 | Configured in SIB4 for Cell 1 and Cell 2 |
| DRX cycle length |  | s | 1-12 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1-12 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1-12 | Not configured |  |
| Ephemeris information |  |  | 1-12 | Note 1 | The detailed configuration is specified in SIB19 |
| T1 |  | s | 1-12 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1-12 | 70 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| Note 1: Detailed ephemeris information is provided in TS 38.508-1 [38] |  |  |  |  |  |

- Table A.20.1.14.2-3: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | 1,2,3,7,8,9 | SSC.1 |  | N/A |  |
|  |  | 4,5,6,10,11,12 | SSC.2 |  |  |  |
| TDD configuration |  | 1,4,7,10 | N/A |  | N/A |  |
|  |  | 2,5,8,11 |  |  | TDDConf.1.1 |  |
|  |  | 3,6,9,12 |  |  | TDDConf.2.1 |  |
| PDSCH RMC |  | 1,4,7,10 | SR.1.1 FDD |  | SR.1.1 FDD |  |
| configuration |  | 2,5,8,11 |  |  | SR.1.1 TDD |  |
|  |  | 3,6,9,12 |  |  | SR.2.1 TDD |  |
| RMSI CORESET |  | 1,4,7,10 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| RMC configuration |  | 2,5,8,11 |  |  | CR.1.1 TDD |  |
|  |  | 3,6,9,12 |  |  | CR.2.1 TDD |  |
| Dedicated CORESET |  | 1,4,7,10 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| RMC configuration |  | 2,5,8,11 |  |  | CCR.1.1 TDD |  |
|  |  | 3,6,9,12 |  |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1-12 | OP.1 defined in clause A.3.2.1 |  | OP.1 defined in clause A.3.2.1 |  |
| Initial DL BWP configuration |  | 1-12 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1-12 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1-12 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1,2,4,5,7,8,10,11 | -140 |  | -140 |  |
|  |  | 3,6,9,12 |  |  | -137 |  |
| Pcompensation | dB | 1-12 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1-12 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1-6 | 14 | 14 | -infinity | 12 |
| Note2 | dBm/SCS | 1,2,4,5,7,8,10,11 | -98 |  | -98 |  |
|  |  | 3,6,9,12 |  |  | -95 |  |
| Note2 | dBm/15 kHz | 1-12 | -98 |  |  |  |
|  | dB | 1-12 | 14 | 14 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | 1,2,4,5,7,8,10,11 | -84 | -84 | -infinity | -86 |
|  |  | 3,6,9,12 |  |  | -infinity | -83 |
| Io | dBm/Ch BW | 1,2,4,5,7,8,10,11 | -55.88 | -55.88 | -70.05 | -57.78 |
|  |  | 3,6,9,12 |  |  | -63.96 | -51.69 |
| Treselection | s | 1-12 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1-12 | 50 |  | 50 |  |
| Threshx, highP | dB | 1-12 | 48 |  | 48 |  |
| Threshserving, lowP | dB | 1-12 | 44 |  | 44 |  |
| Threshx, lowP | dB | 1-12 | 50 |  | 50 |  |
| Propagation Condition |  | 1-12 | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.20.1.14.3	Test requirements

Test requirements in clause A.14.1.12.3 shall apply for 1Rx RedCap UEs.

A.20.1.15	Cell re-selection to FR1 inter-frequency NR case with TN carrier for 2Rx RedCap UE

A.20.1.15.1	Test purpose and Environment

Test purpose and environment in clause A.14.1.12.1 shall apply for 2Rx RedCap UE.

A.20.1.15.2	Test parameters

- Table A.14.1.12.2-1 is replaced with A.20.1.14.2-1, and

- Table A.14.1.12.2-2 is replaced with A.20.1.14.2-2, and,

- Table A.14.1.12.2-3 is replaced with A.20.1.14.2-3, and,

- Table A.14.1.12.2-4 is replaced with A.20.1.14.2-4

A.20.1.15.3	Test requirements

Test requirements in clause A.14.1.12.3 shall apply for 2Rx RedCap UEs.

==============End of change==============