3GPP TSG-RAN WG4 Meeting #117		    R4-2521567
Dallas, USA, Nov. 17-21, 2025

Title:	On 6GR channel bandwidth

Source:	OPPO

Agenda item:	8.2.3

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses system parameters as per RAN4 guideline of 6GR discussion.

Discussion

The channel bandwidth discussion is proposed to be together consider the min and max CBW, FFT size and numerology. Hence below table 1 is proposed as a full view of all these parameters. Detail discussion are listed in following sub-clauses.

Table 1 Proposed 6GR SCS, minCBW and maxCBW

| Frequency range |  | SCS( kHz) | Max FFT size | MaxCBW (MHz) | MinCBW for different bands(MHz) |
| --- | --- | --- | --- | --- | --- |
| FR1 | TDD | 30 | 8192 | 200 | 5/10/20 |
|  | FDD | 15 |  | 50/100 | 3/5 |
| Around 7GHz UE |  | 30 | 8192 | 200 | 10 |
| Around 7GHz BS |  | 30 | 16384 | 400 | 10 |
| Around 15GHz |  | 60 | 8192 | 400 | 50 |
| FR2-1 |  | 120 | 8192 | 800 | 50 |

Maximum Channel bandwidth

MaxCBW for Around 7GHz and 15GHz bands

During the last RAN4#116bis meeting, the WF[2] on maximum channel bandwidth is captured as below.

- Agreement

In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths.

  - RF performance evaluation with proposed max CBW

    - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation

    - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements

    - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers

    - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum

    - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum.

    - Assess the implementation feasibility and complexity and power consumption

    - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1.

    - Study the feasibility to support different NW and UE max CBW.

  - Collaborate with RAN1 on feasibility findings

    - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

For around 7GHz frequency ranges, it has been agreed for network side RAN1 assumes 400MHz maxCBW as captured in the RAN1 chairman note. While for UE side, whether and how to enable UE to support 400MHz bandwidth is for further study.

Agreement

- RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

From RAN4 perspective, larger maxCBW i.e., 200MHz for UE needs to consider the RF front end capability including antenna and PA design as well as the UE implementation complexity and power consumption.

Antennas are optimized for specific frequency bands at specific frequency point. Increasing bandwidth requires broader frequency coverage, which compromises antenna efficiency. Wideband antennas often exhibit reduced gain, poorer impedance matching, and directional inconsistencies, especially at band edges. This leads to signal degradation, higher path loss, and unreliable connectivity, limiting practical bandwidth expansion.

Currently, the envelop tracking (ET) is used to optimize PA voltage and current to increase PA efficiency. For larger channel bandwidth the PA output change in time domain will be larger which makes the tracking hard to be precise and increase the PA inefficiency. This inefficiency increases power consumption, generates excessive heat, and drains battery life, making thermal management and energy sustainability critical barriers for high-bandwidth terminals. Also, the PA memory effect is more severe for large channel bandwidth and larger MPR will be needed which has already been agreed in NR with delta MPR for large relative bandwidth.

MIMO systems leverage spatial multiplexing to boost capacity but require independent RF chains (antennas, PAs, ADCs) per stream. Wide channel bandwidth for MIMO demands precise synchronization, complex channel estimation, and advanced algorithms, escalating computational power and energy use.

The larger maxCBW of UE aiming to higher throughput but needs to balance with antenna design, PA non-linearity, high-order modulation and MIMO. To consider the all above aspects, the maxCBW for UE at around 7GHz is proposed as 200MHz.

From antenna perspective, larger channel bandwidth means reduced gain, poorer impedance matching, and directional inconsistencies, especially at band edges which leads to signal degradation, higher path loss, and unreliable connectivity.

From PA perspective, larger channel bandwidth means PA inefficiency which increases power consumption of UE and PA non-linearity from memory effect which further reduced output power.

From MIMO perspective, large channel bandwidth needs precise synchronization and complex channel estimation leading to complex UE design and large power consumption.

Together consider the SCS and UE implementation complexity, 200MHz is more suitable for around 7GHz range.

For 6GR around 7GHz bands, the maxCBW for UE is proposed to be 200MHz corresponds further SCS study.

For around 15GHz frequency range, 30/60/120kHz SCS is proposed and hence the maxCBW can be proposed to be 400/800MHz. However as current around 15GHz range is from 14.8GHz to 15.3GHz and only 500MHz spectrum is available, so 400MHz seems more suitable for this range.

The spectrum available in 15GHz is only 500MHz

For 6GR around 15GHz bands, the maxCBW is proposed to be 400 corresponds further SCS study.

MaxCBW for NR refarming bands

For the NR re-farming bands, we look back to the NR discussion. The discussion and evolution of the maximum channel bandwidth (maxCBW) in 5G NR have been shaped by spectrum availability, regulatory constraints, and technical feasibility across different frequency ranges (FR1, FR2, and above 52.6 GHz).

Spectrum availability, regulatory constraints, and technical feasibility are used to determine the max CBW.

In FR1 (Sub-6 GHz), maxCBW was initially set at 100 MHz for bands like n41/n78, balancing spectral efficiency and UE complexity. For mmWave (FR2, 24.25–52.6 GHz), 400 MHz was standardized, leveraging wider contiguous spectrum and advanced RF front-end designs. The key factor to define the maxCBW is the SCS and FFT size. In 5G NR, it has been considered 4096 FFT size and hence with 15kHz SCS, only 50MHz can be supported as maxCBW for FDD bands and for TDD bands with 30kHz SCS, up to 100MHz can be supported.

50MHz and 100MHz has been proposed as maxCBW in FR1 considering 15kHz SCS and 30/60kHz SCS in 5G.

In 6GR, as proposed table 1, the FFT size is proposed to be increased to 8192. Hence, even with 15kHz SCS, the FDD bands can enjoy 100MHz maxCBW and TDD bands with up to 200MHz maxCBW.

Besides the FFT size and SCS, the contiguous spectrum available for one carrier of particular band as well as the implementation feasibility/complexity needs to be further discussed.

Currently the 5G FDD bands with the largest frequency span is the band n65 with UL and DL both 90MHz. Besides, take band n1 for example, in 5G since it is agreed that the maxCBW is 50MHz so even 60MHz is the frequency span, however, only 50MHz is supported for band n1. For other FDD bands below who has larger than 50MHz as available spectrum are only 9 bands out of 36 bands and hence it seems 50MHz maxCBW suits for most of the FDD bands. Furthermore, 50MHz as maxCBW can use 4096 FFT size instead of 8192 FFT size which can reduce UE implementation complexity and power

consumption.

Currently the 5G FDD bands with the largest available spectrum is band n65 with UL and DL both 90MHz.

27 of 36 FDD bands has smaller than 50MHz available spectrum.

Use 50MHz for FDD bands as maxCBW can enjoy less UE implementation complexity and power consumption.

For 6GR FR1 FDD bands, the maxCBW is proposed to be 50 MHz. Further study if 100MHz can be used for maxCBW.

Table 2 NR FDD bands

| NR band | UL (MHz) | DL (MHz) | Available spectrum UL/DL (MHz) | NR band | UL (MHz) | DL (MHz) | Available spectrum UL/DL (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 1920 - 1980 | 2110 - 2170 | 60/60 | n66 | 1710 - 1780 | 2110 - 2200 | 70/90 |
| n2 | 1850 - 1910 | 1930 - 1990 | 60/60 | n68 | 698 - 728 | 753 - 783 | 30/30 |
| n3 | 1710 - 1785 | 1805 - 1880 | 75/75 | n70 | 1695 - 1710 | 1995 - 2020 | 15/25 |
| n5 | 824 - 849 | 869 - 894 | 25/25 | n71 | 663 - 698 | 617 - 652 | 35/35 |
| n7 | 2500 - 2570 | 2620 - 2690 | 70/70 | n72 | 451 - 456 | 461 - 466 | 5/5 |
| n8 | 880 - 915 | 925 - 960 | 35/35 | n74 | 1427 - 1470 | 1475 - 1518 | 43/43 |
| n12 | 699 - 716 | 729 - 746 | 17/17 | n85 | 698 - 716 | 728 - 746 | 18/18 |
| n13 | 777 - 787 | 746 - 756 | 10/10 | n87 | 410 - 415 | 420 - 425 | 5/5 |
| n14 | 788 - 798 | 758 - 768 | 10/10 | n88 | 412 - 417 | 422 - 427 | 5/5 |
| n18 | 815 - 830 | 860 - 875 | 15/15 | n91 | 832 - 862 | 1427 - 1432 | 5/30 |
| n20 | 832 - 862 | 791 - 821 | 30/30 | n92 | 832 - 862 | 1432 - 1517 | 30/85 |
| n24 | 1626.5 - 1660.5 | 1525 - 1559 | 34/34 | n93 | 880 - 915 | 1427 - 1432 | 5/35 |
| n25 | 1850 - 1915 | 1930 - 1995 | 65/65 | n94 | 880 - 915 | 1432 - 1517 | 35/85 |
| n26 | 814 - 849 | 859 - 894 | 35/35 | n100 | 874.4 - 880 | 919.4 - 925 | 5.6/5.6 |
| n28 | 703 - 748 | 758 - 803 | 45/45 | n105 | 663 - 703 | 612 - 652 | 40/40 |
| n30 | 2305 - 2315 | 2350 - 2360 | 10/10 | n106 | 896 - 901 | 935 - 940 | 5/5 |
| n31 | 452.5 - 457.5 | 462.5 - 467.5 | 5/5 | n109 | 703 - 733 | 1432 - 1517 | 30/85 |
| n65 | 1920 - 2010 | 2110 - 2200 | 90/90 | n110 | 1390 - 1395 | 1432 - 1435 | 5/3 |

The NR TDD bands and its frequency span is shown as below table 3. Currently 100MHz is defined even we can see some of the bands have as much as 500,700 and 900MHz span. One of the reasons is that the exact spectrum holder usually doesn’t have such large spectrum and also to consider the UE implementation and complexity. With that, together consider the FFT size evolution, it is proposed to use 200MHz as maxCBW for 6GR TDD bands.

Considering the exact spectrum allocation, UE implementation and PA bandwidth, the maxCBW in 5G NR FR1 is 100MHz.

For 6GR FR1 TDD bands, the maxCBW is proposed to be 200MHz.

Table 3 NR TDD bands

| NR band | Frequency Range (MHz) | Available spectrum (MHz) | NR band | Frequency span (MHz) | Available spectrum (MHz) |
| --- | --- | --- | --- | --- | --- |
| n34 | 2010 - 2025 | 15 | n53 | 2483.5 - 2495 | 11.5 |
| n38 | 2570 - 2620 | 50 | n54 | 1670 - 1675 | 5 |
| n39 | 1880 - 1920 | 40 | n77 | 3300 - 4200 | 900 |
| n40 | 2300 - 2400 | 100 | n78 | 3300 - 3800 | 500 |
| n41 | 2496 - 2690 | 194 | n79 | 4400 - 5000 | 600 |
| n46 | 5150 - 5925 | 775 | n90 | 2496 - 2690 | 194 |
| n47 | 5855 - 5925 | 70 | n101 | 1900 - 1910 | 10 |
| n48 | 3550 - 3700 | 150 | n102 | 5925 - 6425 | 500 |
| n50 | 1432 - 1517 | 85 | n104 | 6425 - 7125 | 700 |
| n51 | 1427 - 1432 | 5 |  |  |  |

Minimum Channel bandwidth

During the last RAN4#116bis meeting, the WF[2] on minimum channel bandwidth is captured as below.

- Agreement

  - Compare options of defining min CBW, considering pros and cons

  - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress

    - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands

    - SCS-dependent framework

    - Other aspects are not precluded

  - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

The issue for 3MHz as special case for some specific bands is that for a band supporting both 5MHz and 3MHz, the sync raster will be doubled and consequently, UE needs to search SSB in two set of sync raster. Hence, we have proposed to study the minCBW together with the sync raster design. From our perspective, a unified sync raster design to cover both 3MHz and 5MHz should be introduced and in such case the sync raster will not be increased. This can be further discussed in the raster part.

The issue for 3MHz as special case for some specific bands is that for a band supporting both 5MHz and 3MHz, the sync raster will be doubled.

Besides the sync raster design, currently the irregular channel bandwidth has been fully discussed. In the survey we can see a large number of channel bandwidth allocated for operator in reality are around 4MHz. With the introduction of flexible channel bandwidth with 3MHz minCBW, it can cover almost all the cases.

With the introduction of flexible channel bandwidth with 3MHz minCBW, almost all the operator’s irregular spectrum can be covered.

It is proposed 3MHz as 6GR system minCBW.

Together to consider the discussion and evolution of the minimum channel bandwidth (minCBW) in 5G NR. Initially, proposals in 2017 suggested minCBW settings for Sub-6GHz bands: 5MHz for 15kHz SCS and 10MHz for both 30kHz and 60kHz SCS for FR1 and 50MHz for both 60 and 120kHz SCS for FR2. Subsequent evolution has addressed band-specific needs, such as introducing 5MHz CBW in n41/n90, often tied to regional spectrum fragmentation. In Rel-18, the less than 5MHz WID is accomplished to introduce 3MHz minCBW particularly for Railway communication system, public safety network as well as the legacy LTE refarming bands with fragmented 3MHz allocation. Also, the SSB for 3MHz has been developed and corresponding sync raster has been defined. Currently, band n5, n12, n26, n28, n31, n72, n85, n87, n88, n100, n106 and n110 as total 12 bands has introduced 3MHz.

5MHz for 15kHz SCS and 10MHz for 30kHz has been agreed as minCBW for FR1 in early 5G NR.

3MHz has been introduced in total 12 bands as minCBW at the end of Rel-19.

From the 6GR perspective, the minCBW should also be defined carefully especially from the following aspect:

- Initial access

  - The sync raster granularity depends on the minCBW as the larger minCBW, the coarser sync raster design corresponding to less blind detect time. From this aspect, larger minCBW is preferred.

- Spectrum fragmentation/allocation

  - In some of the LTE re-farming bands, the spectrum allocation of each operator is smaller than 5MHz. As the FR1 FDD bands are captured and shown above table 2. It can be observed that for low frequency bands, only small number of bands are limited by the spectrum allocation of 3MHz (Marked as Green with band n110).

- Spectrum utilization

  - As will also be discussed in the following chapter that with smaller CBW, the SU is always smaller than that with larger CBW. In such case, the small CBW seems not an efficient allocation. From this aspect, larger minCBW is preferred.

When define min CBW, initial access, spectrum status and spectrum utilization need to be considered.

So, to facilitate spectrum holder’s request while allowing larger SU and faster blind detection, we believe the per band basis minCBW is suitable to fulfill different needs.

For 6GR, for FDD bands, 3MHz has been introduced in NR and it is suitable to accommodate operator’s request. While still keep 5MHz for some of the bands which doesn’t need such small CBW as 3MHz is beneficial to initial access and also the spectrum utilization.

For TDD bands, 5/10/20 MHz has been agreed in NR as per band basis per operator’s request. The 5/10/20 MHz is still suitable for FR1.

5MHz for TDD and 3MHz for FDD bands as the 6GR system minCBW and each band can determine larger minCBW based on spectrum status and operator request.

CBW step size

After determining the minCBW and maxCBW, the regular/fixed CBWs between min CBW and rnaxCBW, some step sizes need to be considered. Step size too large will lead to potential spectrum low utilization especially the spectrum is irregular CBW. Step size too small will cause high implementation/test complexity. Current channel bandwidth form of 5G is captured below in table 4. It can be seen that the step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz.

Table 4 Channel bandwidth form of 5G

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |

The step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz as smaller step size in the small CBW and larger step size in the larger CBW.

Currently for the spectrum that is not exactly equal to the defined CBW, CA and irregular channel bandwidth are introduced in 5G. CA might be more proper for the large CBWs considering the complexity of supporting too many flexible CBWs. While for irregular channel bandwidth, currently it is based on operator request and will be introduced one single irregular CBW at one time like the 7MHz. Considering the operator’s spectrum are quite fragmented, flexible CBW is very useful especially when the spec rum is small and no CA is feasible or concerns on the low efficiency with CA.

In 5G, irregular bandwidth has been introduced with only one specific CBW at one time which cannot match the large number of fragmented spectrums hold by operators.

Hence, below we have proposed the flexible CBW concept. From our understanding, the flexible channel bandwidth can apply in below 20MHz range and apply 1MHz flexible granularity to helps operators with different spectrum availability.

Below CBW step size is proposed and further flexible CBW concept is discussed in next chapter 2.7.

Figure 2.5.3-1 Step size of CBW

Propose to reuse 5MHz step size for 5 to 50MHz; 10MHz step size for 50 to 100MHz and newly define 20MHz step size for 100 to 200MHz.

The flexible channel bandwidth can apply below 20MHz range and apply 1MHz flexible granularity

FFT size

During the last RAN4#116bis meeting, the WF[2] on FFT size is captured as below.

- Agreement

  - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges

  - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

The Fast Fourier Transform (FFT) size in 5G NR has been a critical parameter influencing channel bandwidth, numerology flexibility, and implementation complexity. The maximum FFT size has been agreed to be 4096 points with that the supported maximum channel bandwidth has been agreed as 50MHz for FR1 (with 30kHz SCS and 4096 points) and 400MHz for FR2 (with 120kHz SCS and 4096 points).

In 5G NR, 4096 points FFT is agreed.

In the 6GR, evolution can be foreseen to increase the largest FFT size from 4096 to 8192. In such case with 15kHz SCS, the maximum channel bandwidth can go up to 100MHz to improve the throughput of such low bands who in 5G NR the largest channel bandwidth is only 50MHz.

The FFT size connects to UE RF constraints especially to the maxCBW. In the previous maxCBW discussion, it has been proposed 200MHz for UE at around 7GHz with 30kHz SCS. In such case the 8k FFT size is enough and larger FFT size as 16k is not needed.

The FFT size connects to UE RF constraints especially to the maxCBW.

The maxCBW for UE side considering UE RF components is proposed to be 200MHz and 8k FFT size is appropriate considering also the SCS to be 30kHz.

Larger FFT size leads to complex baseband design as well as the computation power consumption. An appropriate FFT size to balance the large channel bandwidth demand and UE complexity and power consumption is necessary.

Larger FFT size brings UE complexity and power consumption.

To introduce 8192 maximum FFT size for UE in 6GR.

Numerology

General SCS

During the last RAN4#116bis meeting, the WF[2] on numerology is captured as below.

- Agreement

  - Evaluate the following proposals regarding numerology from RAN4 perspective

    - "Single numerology" proposal

    - Frequency sub-range/Band specific SCS values proposal

      - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band

      - Study numerology for SSB of initial cell search from RAN4 perspective

    - Other proposals not presented in this meeting are not precluded

  - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Below two sub-clauses discuss the general SCS and SSB SCS.

The standardization of subcarrier spacing (SCS) in 5G NR was a fundamental design choice to enable flexible deployments across diverse frequency bands and use cases. It has been agreed that to define SCS as 15×2ⁿ kHz (n=0-4), supporting 15, 30, 60, 120, and 240 kHz to accommodate different channel conditions.

Table 5 5G NR numerology

| Frequency range | Support |
| --- | --- |
| FR1 | 15/30/60kHz |
| FR2 | 60/120/240kHz |

The 15kHz SCS with longer CP matches better for delay spread hence for low FDD bands, 15kHz is used for increasing coverage. However, 30kHz and 60kHz with smaller latency have better performance in high Doppler spread scenarios. Thus higher frequency bands especially the TDD bands of n41, n77, n78 and n79 apply 30kHz SCS. As it is foreseeable that most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is proposed as for 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

In SID, it has indicated to avoid multiple numerologies for the same band / sub-range

Most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is good for MRSS as for 6GR FR1

For 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

For FR3, current 6GR new spectrum allocation is shown as below figure 2.4-1. Currently the around 7GHz and around 15GHz frequency ranges has been allocated by WRC-27 as shown below figure 2.4-1.

Figure 2.4-1 6GR new spectrum allocation

The new frequency range allocated for 6GR are around 7GHz and around 15GHz.

The frequency range for around 7GHz and around 15GHz are not typical FR2 bands, and can be considered to be harmonized to FR1.

Below table 6 has shown the CP and corresponding scenarios for different SCS. As observed also above, the around 7GHz can be treated as the extension of FR1 and hence 30kHz SCS is proposed. For around 15GHz, 60kHz is proposed to some extend to against the phase noise of higher frequency.

Table 6 SCS, CP and corresponding scenario

| SCS (kHz) | symbol length (us) | CP percentage | Scenario |
| --- | --- | --- | --- |
| 15 | 66.7 | 7% | Coverage, co-existence with LTE |
| 30 | 33.3 | 7% | Balance for mobility and capacity |
| 60 | 16.7 | 14% | Low latency |
| 120 | 8.3 | 29% | Against phase noise |

For 6GR, 30kHz SCS for around 7GHz range and 60kHz SCS for around 15GHz range is proposed.

SSB SCS

To study the numerology for SSB, also the SCS for SSB has been discussed in RAN1#122bis meeting and the agreement is captured as below.

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

The NR different SCS as default SCS for SSB has been introduced for some specific bands as band n5, n24, n34, n38, n39, n41, n53, n66 and n101 as capture currently in TS 38.101-1. Back to the NR development history, the reason why different SCS as default SCS for SSB is that different minCBW demand for different operators. Take band n5 as an example, some operator only has 5MHz spectrum and hence 15kHz SCS with 20RB SSB is appropriate. While some operator has larger available spectrum as 10MHz or 20MHz, then 30kHz SCS and 10MHz minCBW can be applied for faster UE initial access. To facilitate such request, different default SCS has been introduced.

Depends on different operator spectrum holding in the same band, different default SCS for SSB has been proposed in NR to allow the flexibility of deployment.

Hence if we want to follow RAN1 agreement, we might need to check at least RAN4 internally that the operator request for the SCS supported in different bands.

Operator is encouraged to show their proposal and concern on the same SCS agreement.

Although in current spec, the mixed numerology of SSB and data has been captured, in reality deployment, only same numerology has been deployed. This is because the mixed SCS cannot be sub-carrier aligned and hence interference is introduced. In such case, to agree on same SCS between SSB and other channels is preferred.

It is proposed to agree on single SCS for SSB and other channel in one band.

Spectrum Utilization

General

During the last RAN4#116bis meeting, the WF[2] on spectrum utilization is captured as below.

- Agreement

  - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM).

    - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G

  - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled

    - Considering trade-offs between SU, RF performance, and UE/BS complexity

    - Channel bandwidth and SCS with smaller SU should be prioritized

    - SU for larger channel bandwidth shall be evaluated based on progress on CBW

    - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

From procedure perspective, the SU will be decided finally in work item phase. For study item phase, the main work is to discuss the simulation assumption and UE RF requirement for evaluation and propose some initial simulation with candidate new waveform, modulation and PA models.

SU will be decided finally in work item phase.

In study item phase, it is proposed to agree on the simulation assumption and initial UE RF requirement for evaluation and propose some initial simulation with candidate new waveform, modulation and PA models.

Aspects for SU evaluation

The SU evaluation should consider below aspect as below table 7 summarized the evaluation consideration.

Table 7 SU evaluation consideration

| Impact factors | Considerations |
| --- | --- |
| Waveform | Waveform for 6G is still be CP-OFDM/DFT Take the agreed baseline waveform to further study the impact on SU. |
| Modulation | Take the uniform constellation modulation as baseline to study the impact on SU. |
| SEM | The existing NR bands can reuse NR requirements while for newly defined bands especially in around 7GHz and 15GHz, the requirement can be further discussed. Current SEM relaxation of MPR reduction in NR Rel-19 WID can also be considered. |
| EVM | A possible of relaxing the UE TX EVM requirement for higher order modulation can be considered. |
| ACLR | Requirement will be based on co-existence study. For FR1 re-farming bands, NR legacy requirement can be reused. |
| Demodulation | This will be waveform related and discussed together with the waveform. |
| New PA models | Together with possible new default power class: PA model needs to be updated with APT PA to be considered. Also, the default UE power class might be newly defined as higher power class has already been introduced among plenty of bands in NR. |

The waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models are factors need to be considered in the SU evaluation.

In our companion paper for waveform evaluation, the initial UE RF requirements for evaluation have been discussed. The 5G NR requirement can be used as baseline and starting point. The corresponding UE RF assumption should be aligned with waveform, modulation and SU evaluation.

The 5G NR requirement can be used as baseline and starting point.

The corresponding UE RF assumption should be aligned with waveform, modulation and SU evaluation.

All the new 6GR waveform/modulation/UE RF requirements and PA models as listed in table 7 should be considered in 6GR SU study. For around 7GHz, all these factors will be newly defined, hence it is better to wait at least some clear candidates of these new factors are settled down to further accomplish the simulation. At this stage we can further study the impacted requirement as well as the potential simulation assumption first. While for the NR re-farming bands, mostly the UE RF requirements will be reused, but with some further relaxation, i.e., ACLR, SEM and TX EVM relaxation.

For around 7GHz all the evaluation factors will be newly defined.

For NR re-farming bands, the UE RF requirements are supposed to be the same but with some relaxation, i.e., ACLR, SEM and TX EVM.

Start the SU evaluation with NR re-farming bands first to consider the requirement relaxation first. For around 7GHz bands, wait to see other system parameter and UE RF requirement progress.

Simulation assumption for SU evaluation

Besides the UE RF requirements, further simulation assumption needs to be aligned. From our understanding, the SU evaluation should be based on the RF requirements to find the largest available number of RBs of each channel bandwidth which should evaluate the PA output PSD. Hence the PSD evaluation considering the RF impairment needs to be aligned. Also as captured in the agreed WF, the PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), need to be aligned. Below table 8 is based on NR simulation assumptions and provided for SU evaluation in 6GR.

To agree on table 8 as simulation assumption:

Table 8 Simulation assumption

| Parameter | Assumption |
| --- | --- |
| Center frequency | 700MHz, 2GHz, 7GHz |
| Maximum output power | 26 dBm |
| Numerology | 15kHz for 700MHz and 2GHz 30kHz for 2GHz and 7GHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |
| PA model | As agreed in Waveform discussion |

Irregular Channel bandwidth

General

During the last RAN4#116bis meeting, the WF [2] on irregular channel bandwidth is captured as below.

- Agreement

  - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G

    - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths

    - Other options not presented in this meeting are not precluded

Below sub-clauses further introduce the background of irregular CBW and the solution in TR 38.844 and corresponding problem when using such solutions has been discussed. Following a generic solution as flexible CBW concept to be introduced in 6GR.

Background on irregular CBW

During the 5G NR evolution, irregular channel bandwidth has been introduced due to operator’s request as the spectrum allocation is often not regular enough to follow 3GPP channel bandwidth definition. In LTE, 1.4/3/5/10/20 MHz are defined. During the NR discussion, 5/10/15/20/25/30/35/40/45/50/60/70/80/90/100 MHz channel bandwidth are defined. It can be observed that 5MHz granularity is used for smaller channel bandwidth and 10MHz granularity is used for larger channel bandwidth. This will have problem when smaller granularity of 5/10MHz is needed such as 7MHz which might need 1MHz granularity to facilitate such channel bandwidth. A more detail irregular spectrum hold by operators are shown below in table 8.

Smaller granularity than 5/10MHz is needed such as 7MHz which might need 1MHz granularity,

Table 8 Irregular spectrum hold by operators

| Range of bandwidth | Irregular spectrum hold by operators (MHz) | Applicable 3GPP CBW |
| --- | --- | --- |
| <5MHz | 1, 2.4, 2.46, 2.49, 2.5, 2.51, 2.6, 3.4, 3.69, 3.7, 3.75, 3.8, 4, 4.2, 4.4, 4.6, 4.8, 4.9, 4.92 | No 3GPP CBW can be applied |
| [5MHz, 10MHz) | 5.03, 5.2, 5.4, 5.5, 5.6, 5.8, 6, 6.2, 6.25, 6.3, 6.4, 6.5, 6.6, 6.8, 7, 7.2, 7.5, 7.6, 7.8, 8, 8.2, 8.3, 8.4, 8.6, 8.8, 9, 9.1, 9.2, 9.3, 9.34, 9.6, 9.7, 9.8, 9.9 | 5MHz (Many spectrums will be wasted) |

Many operators’ spectrum doesn’t have suitable 3GPP CBW which makes the spectrum cannot be fully utilized.

Current solution for irregular CBW

To solve the irregular CBW problem, 5G has introduced different methods to solve this problem. Below has summarized those solutions.

Option 1: To choose the nearest regular CBW. For example, if the 7MHz is the exact irregular CBW, then it falls within 5MHz and 10MHz. Hence whether to choose 5MHz or 10MHz to deploy the cell. The question here is if we use 5MHz which is smaller than 7MHz, then 2MHz will be wasted. If we use 10MHz which is larger than 7MHz, then the filter for 10MHz cannot work for the range from 7 to 10MHz.

Figure 2.5.4.2-1 To choose the nearest regular CBW method

If choose the nearest small regular CBW, then the additional spectrum is wasted.

If choose the nearest large regular CBW, then the filter design does not match the bandwidth which will suffer interference.

Option 2: Base station support irregular CBW while UE uses two smaller regular CBW with overlapping. If we want to make full use of this 7MHz then the two 5MHz component carrier should be RB aligned and channel raster aligned which will not always be the case. Furthermore, the overlapping part should be large enough to place a whole SSB and Coreset0 while for smaller than 10MHz, this is also not always fulfilled.

Figure 2.5.4.2-2 BS support irregular CBW while UE support regular CBW

If BS support irregular CBW while UE support regular CBW, the component carrier RB level alignment, channel raster SCS level alignment and large enough overlapping part to put SSB and Correset0 are hard to guarantee.

Option 3: The overlapping CA method on BS side. BS use two regular CBW with overlapping CA to support the irregular CBW. Then two SSB need to be put in this BS CA range which is not possible.

Figure 2.5.4.2-3 BS CA 7MHz method

For small irregular CBW, it is difficult to put two SSB non-overlapping with the BS CA method.

So, the above 3 options are all not perfect. In the end, the 7MHz is introduced separately. If that is the method, any irregular CBW will need to be introduced one by one while as can be seen from table 8, too many number of irregular CBW are waiting to be studied.

Current solution doesn’t work well with irregular CBW.

To introduce flexible CBW in 6G to solve the 5G irregular CBW.

Flexible CBW concept in 6G for irregular CBW

The flexible channel bandwidth has been proposed and discussed during the 5G NR SI and WI. Due to time limitation companies are willing to finish the 5G NR first release in a simple way and further enhancement is agreed. However, as the frame of channel bandwidth has been agreed, it is hard to add smaller granularity as 1MHz or RB level. In 6GR, as the spectrum is more and more treasures, the flexible channel bandwidth is proposed as below figure 2.5.3-1. With the flexible channel bandwidth, operators can use fully of their irregular channel bandwidth.

Figure 2.5.3-1 Flexible channel bandwidth

Following are aspects that needs to study during the SI phase.

- UE RF requirement applicability: Some UE RF requirement is scalable with CBW while some are not.

- Guard band definition: How to calculate the guard band based on allocated RBs.

- Test burden: How to reduce test burden with small CBW granularity.

The introduction of flexible channel bandwidth need to further study the UE RF requirement applicability, guard band definition and reduce test burden.

Flexible CBW impacts to UE RF requirements

For the UE RF requirements, currently in NR, some requirements are CBW scalable while some of them are fixed without CBW change. To summarize, below table 9 shows current 5G NR UE RF requirements that can be straightforward defined either with no change or scalable change of channel bandwidth.

Table 9 Requirement corresponding to channel bandwidth

| Straightforward defined with bandwidth change |  |  |
| --- | --- | --- |
| UE maximum output power | ACLR (Measurement bandwidth and SCS) | Max input level |
| MPR | SEM (Requirement frequency range) | ACS |
| Transmit off power (Measurement bandwidth and SCS) | Spurious emission (OOB boundary) | In-band blocking |
| Transmit ON/OFF time mask | Transmit intermodulation (Wanted signal and interference signal bandwidth) | Out-of-band blocking |
| Power control | Spurious emission | Narrow band blocking |
| EVM, Carrier leakage, In-band emission | Minimum output power | Spurious response |
| Frequency error | Occupied bandwidth | RX inter-modulation |

Most of the requirements are straightforward defined either with no change or scalable change of channel bandwidth.

The only two requirements that needs to further discussed is UE RX REFSENS and A-MPR requirement which should be considered case by case. For the RX REFSENS, still the TX to RX interference can be calculated based on the requirement of regular channel bandwidth. The A-MPR requirement depends highly on the regulation and hence should be decided separately.

For the RX REFSENS, still the TX to RX interference can be calculated based on the requirement of regular channel bandwidth.

For A-MPR requirement, it depends highly on regulation and should be discussed separately.

Generally, it can be found that most of the TX requirements are non-related to channel bandwidth while for RX requirements, all of them except for spurious emission, are all related to channel bandwidth. For those requirements not related to CBW, it is straight forward to apply similarly with flexible channel bandwidth. For The requirements related to CBW besides the A-MPR and REFSENS requirements who need to be discussed separately, the other remaining RX requirements, who currently is scaled with CBW and same scaling can be remained to flexible channel bandwidth.

All requirements can either scale with bandwidth or be irrelevant with the bandwidth.

For the requirements not related to CBW, same requirement apply when introduce flexible channel bandwidth.

For requirements are scalable with bandwidth, the requirements can be scaled with flexible channel bandwidth same as NR principle.

Flexible CBW impacts to System parameter requirements

For the guard band calculation and SU for the flexible channel bandwidth, currently the SU is defined by each “regular” channel bandwidth and the minimum guard band is then calculated by formula. For flexible channel bandwidth, two options can be used to calculate the SU or number of RBs can be scheduled.

Scaling method: To scale the SU of the flexible channel bandwidth falling into the two adjacent “regular” bandwidth. Then calculate the corresponding RB number using the scaled SU.

Nearest SU method: To scale the corresponding RB numbers with the same SU of the nearest “regular” channel bandwidth.

Below we give an example on how to calculate the SU and guard band. Take 9MHz as example. Below table 10 is the SU for 3/5/10MHz. The 9MHz falls into 5 and 10MHz hence the SU for 5MHz as 90% and SU for 10MHz as 94% will be used.

Table 10 SU table for 3/5/10 MHz

| SCS (kHz) | 3 | 5 | 10 |
| --- | --- | --- | --- |
|  | MHz | MHz | MHz |
|  | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 |
| NR SU | 0.90 | 0.90 | 0.94 |

For scaling method, the SU for 9MHz will be 93.2% as shown below.

SU9M = SU5M+(SU10M-SU5M)/(10MHz-5MHz)*(9MHz-5MHz) = 93.2%

Then in this case the RB for 9MHz will be floor(9MHz * 93.2% / 0.18MHz) = 46. So 46RB is proposed to 9MHz.

For nearest SU method, the smaller SU of the two nearest SU will be used. In this case, SU for 5MHz as 90% is sued.

Then in this case the RB for 9MHz will be floor(9MHz * 90% / 0.18MHz) = 45. So 45RB is proposed to 9MHz.

Examples has been shown for different methods with different RBs for flexible CBW.

The nearest SU method as smaller SU is chosen to guarantee the guard band but the scaling method helps to guarantee the SU

This can be further studied with the 6GR SU discussion.

To further study the two methods as nearest SU method and scaling SU method and their impacts to SU.

Flexible CBW impacts to Signaling

In current 5G UE CBW signalling, the carrier bandwidth is from SIB1 of SCS-SpecificCarrier field as shown below:

Initial access

Connected

Currently the signalling of 5G already support the carrierBandwidth to be configured as any integer number of RBs and hence similar approach can be foreseen in 6GR in both initial access and RRC connected mode.

Current 5G carrierBandwidth for both initial access and RRC connected mode can support the flexible RB configuration for irregular CBW.

Further study the signalling design of 6GR to guarantee the flexible CBW work properly.

How to solve Flexible CBW test complexity

As previously indicated in this chapter the RF requirements are discussed and corresponding requirements can be defined for flexible channel bandwidth. However, as if all the flexible CBW needs to be tested, their will be large test burden.

RF performance usually is contiguous in terms of signal bandwidth. Hence nnow in UE design, usually only some of the CBWs are calibrated considering the time costs in production and CBW in middle will reuse the calibration results of nearest CBW. Therefore, it is reasonable to only test the regular CBW as provided in chapter 2.5.3.

To reduce test burden, specific regular channel bandwidth will be defined and the test only apply to regular channel bandwidth.

UL/DL asymmetric CBW

During the last RAN4#116bis meeting, the WF[2] on UL/DL asymmetric channel bandwidth is captured as below.

- Agreement

  - Study the need and feasibility to enable asymmetric CBW

In NR, the UE channel bandwidth per operating band is specified for both the TX and RX path together, the UE only supports the symmetric channel bandwidth between downlink and uplink, if no specified asymmetric UL and DL channel bandwidth combinations for this band, as shown in Figure 2.5.5-1:

Figure 2.5.5-1 Symmetric channel bandwidth in downlink and uplink

And for FDD band, the carrier centre frequency separation between UL and DL needs to meet the default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation specified for each operating bands in Spec.

In addition, NR has specified asymmetric channel bandwidth in downlink and uplink for some FDD bands, correspondingly, the default Tx-Rx carrier centre frequency separation can be with a maximum deviation (ΔFTX-RX = |(BWDL – BWUL)/2) as shown in Figure 2.5.5-2:

Figure 2.5.5-2 Asymmetric channel bandwidth in downlink and uplink

When a UE supports the asymmetric channel bandwidth in downlink and uplink in the bands as specified in the Spec, the UE need report it by the capability of asymmetricBandwidthCombinationSet.

In 5G, asymmetric channel bandwidth has already been supported with signaling.

The ΔFTX-RX = (BWDL – BWUL)/2) has been defined to limit the separation difference introduced by asymmetric channel bandwidth.

Supporting asymmetric CBW especially smaller UL CBW is benefit in UE power consumption. Current large CBW + smaller BWP approach doesn’t save much RF power due to RFFE components are configured based CBW. Besides, usually UE UL throughputs are much smaller than DL. Therefore, as an optional feature, it might be useful to allow asymmetric CBW for all FDD bands in 6G.

All bands can apply symmetric/asymmetric CBW in downlink and uplink in 6G day one with the limitation of maximum of deviation to the TX-RX carrier center frequency separation as ΔFTX-RX =  (BWDL – BWUL)/2).

Conclusion

This paper discussed the 6GR channel bandwidth, and got below observations and proposals.

MaxCBW

Observation 1: 	From antenna perspective, larger channel bandwidth means reduced gain, poorer impedance matching, and directional inconsistencies, especially at band edges which leads to signal degradation, higher path loss, and unreliable connectivity.

Observation 2: 	From PA perspective, larger channel bandwidth means PA inefficiency which increases power consumption of UE and PA non-linearity from memory effect which further reduced output power.

Observation 3: 	From MIMO perspective, large channel bandwidth needs precise synchronization and complex channel estimation leading to complex UE design and large power consumption.

Observation 4: 	Together consider the SCS and UE implementation complexity, 200MHz is more suitable for around 7GHz range.

Proposal 1: 	For 6GR around 7GHz bands, the maxCBW for UE is proposed to be 200MHz corresponds further SCS study.

Observation 5: 	The spectrum available in 15GHz is only 500MHz

Proposal 2: 	For 6GR around 15GHz bands, the maxCBW is proposed to be 400 corresponds further SCS study.

Observation 6: 	Spectrum availability, regulatory constraints, and technical feasibility are used to determine the max CBW.

Observation 7: 	50MHz and 100MHz has been proposed as maxCBW in FR1 considering 15kHz SCS and 30/60kHz SCS in 5G.

Observation 8: 	Currently the 5G FDD bands with the largest available spectrum is band n65 with UL and DL both 90MHz.

Observation 9: 	27 of 36 FDD bands has smaller than 50MHz available spectrum.

Observation 10: 	Use 50MHz for FDD bands as maxCBW can enjoy less UE implementation complexity and power consumption.

Proposal 3: 	For 6GR FR1 FDD bands, the maxCBW is proposed to be 50 MHz. Further study if 100MHz can be used for maxCBW.

Observation 11: 	Considering the exact spectrum allocation, UE implementation and PA bandwidth, the maxCBW in 5G NR FR1 is 100MHz.

Proposal 4: 	For 6GR FR1 TDD bands, the maxCBW is proposed to be 200MHz.

MinCBW

Observation 12: 	The issue for 3MHz as special case for some specific bands is that for a band supporting both 5MHz and 3MHz, the sync raster will be doubled.

Observation 13: 	With the introduction of flexible channel bandwidth with 3MHz minCBW, almost all the operator’s irregular spectrum can be covered.

Proposal 5: 	It is proposed 3MHz as 6GR system minCBW.

Observation 14: 	5MHz for 15kHz SCS and 10MHz for 30kHz has been agreed as minCBW for FR1 in early 5G NR.

Observation 15: 	3MHz has been introduced in total 12 bands as minCBW at the end of Rel-19.

Proposal 6: 	When define min CBW, initial access, spectrum status and spectrum utilization need to be considered.

Proposal 7: 	5MHz for TDD and 3MHz for FDD bands as the 6GR system minCBW and each band can determine larger minCBW based on spectrum status and operator request.

CBW step size

Observation 16: 	The step size is 5MHz from 5 to 50MHz and 10MHz from 50 to 100MHz as smaller step size in the small CBW and larger step size in the larger CBW.

Observation 17: 	In 5G, irregular bandwidth has been introduced with only one specific CBW at one time which cannot match the large number of fragmented spectrums hold by operators.

Proposal 8: 	Propose to reuse 5MHz step size for 5 to 50MHz; 10MHz step size for 50 to 100MHz and newly define 20MHz step size for 100 to 200MHz.

Proposal 9: 	The flexible channel bandwidth can apply below 20MHz range and apply 1MHz flexible granularity

FFT size

Observation 18: 	In 5G NR, 4096 points FFT is agreed.

Observation 19: 	The FFT size connects to UE RF constraints especially to the maxCBW.

Observation 20: 	The maxCBW for UE side considering UE RF components is proposed to be 200MHz and 8k FFT size is appropriate considering also the SCS to be 30kHz.

Observation 21: 	Larger FFT size brings UE complexity and power consumption.

Proposal 10: 	To introduce 8192 maximum FFT size for UE in 6GR.

Numerology

Observation 22: 	In SID, it has indicated to avoid multiple numerologies for the same band / sub-range

Observation 23: 	Most of the FR1 bands can be re-farmed to 6GR, for better co-existence with 5G, the same SCS for FR1 is good for MRSS as for 6GR FR1

Proposal 11: 	For 6GR FR1, TDD bands apply 30kHz SCS and FDD bands apply 15kHz SCS.

Observation 24: 	The new frequency range allocated for 6GR are around 7GHz and around 15GHz.

Observation 25: 	The frequency range for around 7GHz and around 15GHz are not typical FR2 bands, and can be considered to be harmonized to FR1.

Proposal 12: 	For 6GR, 30kHz SCS for around 7GHz range and 60kHz SCS for around 15GHz range is proposed.

Observation 26: 	Depends on different operator spectrum holding in the same band, different default SCS for SSB has been proposed in NR to allow the flexibility of deployment.

Observation 27: 	Operator is encouraged to show their proposal and concern on the same SCS agreement.

Proposal 13: 	It is proposed to agree on single SCS for SSB and other channel in one band.

Spectrum utilization

Observation 28: 	SU will be decided finally in work item phase.

Proposal 14: 	In study item phase, it is proposed to agree on the simulation assumption and initial UE RF requirement for evaluation and propose some initial simulation with candidate new waveform, modulation and PA models.

Observation 29: 	The waveform, modulation, SEM, EVM, ACLR, demodulation and new PA models are factors need to be considered in the SU evaluation.

Proposal 15: 	The 5G NR requirement can be used as baseline and starting point.

Proposal 16: 	The corresponding UE RF assumption should be aligned with waveform, modulation and SU evaluation.

Observation 30: 	For around 7GHz all the evaluation factors will be newly defined.

Observation 31: 	For NR re-farming bands, the UE RF requirements are supposed to be the same but with some relaxation, i.e., ACLR, SEM and TX EVM.

Proposal 17: 	Start the SU evaluation with NR re-farming bands first to consider the requirement relaxation first. For around 7GHz bands, wait to see other system parameter and UE RF requirement progress.

To agree on table 8 as simulation assumption:

Table 8 Simulation assumption

| Parameter | Assumption |
| --- | --- |
| Center frequency | 700MHz, 2GHz, 7GHz |
| Maximum output power | 26 dBm |
| Numerology | 15kHz for 700MHz and 2GHz 30kHz for 2GHz and 7GHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |
| PA model | As agreed in Waveform discussion |

Irregular Channel bandwidth

Observation 32: 	Smaller granularity than 5/10MHz is needed such as 7MHz which might need 1MHz granularity,

Observation 33: 	Many operators’ spectrum doesn’t have suitable 3GPP CBW which makes the spectrum cannot be fully utilized.

Observation 34: 	If choose the nearest small regular CBW, then the additional spectrum is wasted.

Observation 35: 	If choose the nearest large regular CBW, then the filter design does not match the bandwidth which will suffer interference.

Observation 36: 	If BS support irregular CBW while UE support regular CBW, the component carrier RB level alignment, channel raster SCS level alignment and large enough overlapping part to put SSB and Correset0 are hard to guarantee.

Observation 37: 	For small irregular CBW, it is difficult to put two SSB non-overlapping with the BS CA method.

Observation 38: 	Current solution doesn’t work well with irregular CBW.

Proposal 19: 	To introduce flexible CBW in 6G to solve the 5G irregular CBW.

Observation 39: 	The introduction of flexible channel bandwidth need to further study the UE RF requirement applicability, guard band definition and reduce test burden.

Observation 40: 	Most of the requirements are straightforward defined either with no change or scalable change of channel bandwidth.

Observation 41: 	For the RX REFSENS, still the TX to RX interference can be calculated based on the requirement of regular channel bandwidth.

Observation 42: 	For A-MPR requirement, it depends highly on regulation and should be discussed separately.

Observation 43: 	All requirements can either scale with bandwidth or be irrelevant with the bandwidth.

Proposal 20: 	For the requirements not related to CBW, same requirement apply when introduce flexible channel bandwidth.

Proposal 21: 	For requirements are scalable with bandwidth, the requirements can be scaled with flexible channel bandwidth same as NR principle.

Observation 44: 	Examples has been shown for different methods with different RBs for flexible CBW.

Observation 45: 	The nearest SU method as smaller SU is chosen to guarantee the guard band but the scaling method helps to guarantee the SU

Proposal 22: 	To further study the two methods as nearest SU method and scaling SU method and their impacts to SU.

Observation 46: 	Current 5G carrierBandwidth for both initial access and RRC connected mode can support the flexible RB configuration for irregular CBW.

Proposal 23: 	Further study the signalling design of 6GR to guarantee the flexible CBW work properly.

Proposal 24: 	To reduce test burden, specific regular channel bandwidth will be defined and the test only apply to regular channel bandwidth.

UL/DL asymmetric CBW

Observation 47: 	In 5G, asymmetric channel bandwidth has already been supported with signaling.

Observation 48: 	The ΔFTX-RX = (BWDL – BWUL)/2) has been defined to limit the separation difference introduced by asymmetric channel bandwidth.

Proposal 25: 	All bands can apply symmetric/asymmetric CBW in downlink and uplink in 6G day one with the limitation of maximum of deviation to the TX-RX carrier center frequency separation as ΔFTX-RX =  (BWDL – BWUL)/2).

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

R4-2514641	WF on [116bis][101] 6G system parameter, Huawei