3GPP TSG-RAN WG4 Meeting #117		    R4-2521568
Dallas, USA, Nov. 17-21, 2025

Title:	On 6GR channel arrangement

Source:	OPPO

Agenda item:	8.2.4

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses system parameters as per RAN4 guideline of 6GR discussion.

Discussion

Sync raster

Background of 5G sync raster design

During the last RAN4#116bis meeting, a WF [2] has been agreed and the discussion on sync raster is captured as below.

- Agreement

  - Evaluation on sync raster from RAN4 perspective:

    - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4

    - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving

  - Collaboration and planning:

    - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset.

    - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

During the NR sync raster design, two principles has been agreed and the sync raster design is based on them.

1, SCS level alignment of sync raster and channel raster

2, For each minCBW, at least one SSB to be covered

The first principle brings the floating sync raster and consequently, the Kssb is introduced in MIB to indicate the RB boundary. The second principle limits the largest sync raster point separation.

In NR sync raster design, two rules has been agreed as

- 1, SCS level alignment of sync raster and channel raster

- 2, For each minCBW, at least one SSB to be covered

Currently there is two sets of synchronization raster for different minimum channel bandwidth as 3MHz and 5MHz and corresponding SSB design has a little bit difference. The sync raster design has considered the LTE 100kHz channel raster and is combined with SCS level alignment of sync raster point and channel raster point for FR1 lower than 3GHz bands. As a result, 1.2MHz sync raster granularity has been proposed and also an LS has been sent to RAN1 to facilitate the SCS level floating. In consequence, the kssb is introduced in RAN1 specification since the RB alignment is denied and kssb is needed for an idle UE to find the exact frequency boundary of a carrier which is a compromised solution as the sync raster design agreement in RAN4 is agreed at the end of the WI and RAN1 has no further time to discuss.

Kssb is introduced to find the RB edge based on rule 1 as SCS level alignment of sync raster and channel raster.

Figure 1 SSB position with Kssb

For larger than 3GHz bands, 1.44MHz sync raster granularity is introduced based on SCS level channel raster and for different minimum channel bandwidth, the scaling factor are also different to enable larger sync raster granularity.

For 5MHz minCBW, the SS granularity is 1.2MHz for smaller than 3GHz and 1.44MHz for larger than 3GHz.

For 3MHz minCBW, the SS granularity is 0.6MHz.

The above SS granularity is based on the rule 2 as for each minCBW, at least one SSB to be covered.

In NR, the design has been considered carefully and there is no room for improvement if the listed two rules are still kept. Hence for 6GR, if less sync raster points are defined, these two rules needs to be broken.

The NR sync raster is well defined and further improvement needs to break the two rules.

How to improve the sync raster design and to what extent to break these two rules are trade-off of spectrum flexibility, system information payload and UE initial access complexity as well as latency. For example, if there is no rule 2, then a very large sync raster point separation can apply, which makes the whole band with less sync raster points to put SSB and hence less channels can be used. This might be questioned by spectrum holders. But if operator can agree on losing some of the flexibility which is not critical and then UE can enjoy the initial access gain with less sync raster points.

How to improve the sync raster design and to what extent to break these two rules are trade-off of spectrum flexibility, system information payload and UE initial access complexity as well as latency.

In our opinion, the original rule 2 is still needed while we can use larger number of channel raster, smaller SSB bandwidth and larger minCBW in different bands to reach the goal as reducing the sync raster points.

To consider larger channel raster, smaller SSB bandwidth and larger minCBW in different bands to reach the goal as reducing the sync raster points.

For rule 1, it helps to reduce the interference of SSB and data. This is good and needs to be kept. However, we can further improve it so that the Kssb can be avoided to save MIB payload.

The sync raster and channel raster design should consider the interference mitigation and improvement is needed to get rid of Kssb to save MIB payload.

From 6GR perspective, further enhancement for reducing the sync raster entries are helpful for UE initial cell search and blind detection. Currently there are different principle of sync raster for different bands, minCBW, SCS, frequency range, channel raster which makes the spec hard to read and also the complexity of UE implementation. From our understanding, a unified design will be helpful. To consider different minCBW of each band and corresponding sync raster entries can be scalable based on minCBW which follows the NR principle can be one of the solutions.

Define unified sync raster design for different bands, minCBW, SCS and channel raster in 6GR.

With the above proposals, below a detail RB based sync raster design has been proposed.

RB based sync raster design

Currently we follow the Rule 2 as for each carrier of minCBW, there should be a whole SSB included in frequency range to define the sync raster. Then the sync raster point separation should fulfill below equation 1.

Sync raster separation ≤ minCBW + Channel raster – BWSSB		(Equation 1)

Figure 2 Sync raster separation

The equation 1 is illustrated as above figure 2. For the CBW1 till CBW2, they all can use SSB1 as their carrier SSB. While for CBW3 as its offset to CBW2 is the channel raster in 5G equation 1, the SSB1 is no longer fully contained by CBW3 and hence SSB2 is proposed. With this, the largest sync raster separation equation 1 is formulated. Based on the equation 1, if we want to have larger sync raster separation compared to NR as 1.2MHz and 1.44MHz for 5MHz minCBW and 600kHz for 3MHz minCBW, we will need larger minCBW, channel raster with smaller SSB bandwidth.

- minCBW: The minCBW has been discussed. It should follow the regulation. However, we can define different minCBW for different bands as proposed previously so that larger minCBW can apply conditionally.

- Channel raster: In 5G the granularity of each initial access cell is channel raster. While enlarging channel raster might be helpful to reduce the sync raster point, this might cause less channel raster points which will limit the operator deployment and also the SU might be impacted. Also, as observation

- BWSSB: Smaller SSB helps to enlarge the sync raster separation and also improves network energy saving while the SSB performance will be impacted and need to be further studied. Currently the 12RB SSB structure has been proposed in RAN1 and we believe this could be a good starting point to enjoy the UE power saving with smaller SSB bandwidth,

MinCBW, Channel raster, BWSSB are the main factors that impact the sync raster design. Mi CBW will limit the sync raster steps. Make sure all possible channel has SSB.

With current 5G sync raster design, especially to guarantee that for every minCBW in every channel raster there should be one SSB fully contained by the minCBW, the sync raster points cannot be coarser if we still follow such rule. The most straight forward method to have coarser sync raster is to make compromise on the deployment flexibility with the sync raster searching complexity. Here we calculate the equation 1 with replacing the channel raster to a reference sparse channel raster. By using the reference sparse channel raster, then the Rule 2 as for each carrier of minCBW, there should be a whole SSB included cannot apply. But we can have coarser sync raster points and also to avoid the Kssb to save SIB1 information bits.

By replacing the channel raster to a reference sparse channel raster, coarser sync raster points can be reached.

Consider a reference sparse channel raster compared to channel raster for data carrier for initial access.

To have coarser sync raster points means faster cell search and better UE power saving. To also try to avoid the Kssb as introduced in the end of NR WID because of the SCS level floating sync raster agreed late in RAN4. These two main concerns in NR needs to be solved in 6GR new sync raster design.

6GR sync raster design to have coarser sync raster points and try to avoid Kssb.

Using 1RB as reference channel raster can naturally guarantee the SSB and channel RB grid is RB aligned. In this case, the UE after blind detection can directly know the RB grid edge and hence the Kssb is no longer needed.

Using 1RB as reference channel raster can avoid Kssb since the grid are RB level aligned.

Since the SCS level channel raster has been used in 5G, we try to make improvement by taking the pros of RB level channel raster while resolve the problem that it brings to us. A specific initial access carrier is proposed which means from initial access perspective, the IA carrier can use larger than SCS or as proposed here 1RB for reference channel raster as in the figure 3 below while after the IA process, the data carrier based on the SCS level channel raster can be established. This helps to keep the flexibility of data carrier with smaller channel raster while helps to reduce the sync raster points by enlarging the channel raster of Initial Access carrier.

Figure 3 Specific Initial Access carrier and Data carrier

The reference channel raster is coarser than the channel raster.

Here we calculate the sync raster granularity by using 15 RB configuration for 3MHz minCBW, 12 RB for SSB and 1RB as the reference channel raster. Table 2.7.1.2-1 below shows the sync raster granularity. The largest possible granularity is calculated and listed in the last by second column. Here we try to optimize the granularity as can be seen the last column based is the final proposed ones. With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz. This is further illustrated as figure 4.

Table 1 Sync raster granularity

| Minimum BW | SCS for SS | NRB,Carrier | Max ΔFSS,Raster | ΔFSS,Raster |
| --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 | 0.72 |
| 5 | 15 | 25 | 2.52 | 2.16 |
| 10 | 30 | 24 | 4.68 | 4.32 |
| 15 | 30 | 38 | 9.72 | 9.36 |
| 20 | 30 | 51 | 14.4 | 14.4 |

With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz.

This ensure with different minCBW, no new sync raster points are introduced.

Figure 4 illustration of different minCBW corresponding to the sync raster points

Below table 2 has shown the sync raster point reduction with 1RB as Initial Access carrier with 12RB SSB structure.

Table 2 Sync Raster reduction with 1RB channel raster

| minCBW | Sync raster granularity for SCS channel raster | Sync raster granularity for RB channel raster | Sync raster points reduction percentage |
| --- | --- | --- | --- |
| 3 MHz | 600 kHz | 720 kHz | 20% |
| 5 MHz | 1.2 MHz | 2.16 MHz | 80% |
| 5 MHz | 1.44 MHz | 2.16 MHz | 50% |

With 1RB reference channel raster, for different minCBW cases, large sync raster points reduction can be reached.

It is proposed to use 1RB as reference channel raster for specific initial access carrier.

The per band basis minCBW apply and the sync raster granularity can be defined accordingly.

| Minimum BW | SCS for SS | NRB,Carrier | ΔFSS,Raster |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 |
| 5 | 15 | 25 | 2.16 |
| 10 | 30 | 24 | 4.32 |
| 15 | 30 | 38 | 9.36 |
| 20 | 30 | 51 | 14.4 |

In 6GR, we propose 1RB as reference channel raster for initial access carrier that can at the same time to solve the two concerns and below table 3 shows the pros and cons of SCS channel raster and RB channel raster.

Table 3 Pros and Cons of different calculation in sync raster design

|  | Pros | Cons |
| --- | --- | --- |
| SCS level channel raster in Sync raster calculation | Larger SU: Small granularity of channel raster to make full use of spectrum Flexibility: More channel raster points to establish a carrier. | Slower Cell Search: Smaller sync raster separation leading to more blind detection time. Additional MIB information: Kssb is needed to show the RB boundary. |
| Reference sparse channel raster in Sync raster calculation | Faster Cell Search: Larger Sync raster separation leading to less blind detection time. Simplified MIB information: The sync raster and channel raster are RB level aligned and the RB boundary is naturally determined. | Lose SU: Large granularity of channel raster might lose 1RB of SU Less flexibility: Less channel raster to establish a carrier. |

To avoid the cons of Reference sparse channel raster in Sync raster calculation, the data carrier after IA can use channel raster to establish the data carrier itself than it can enjoy the full SU as well as the flexibility of deployment in frequency raster.

The data carrier can use channel raster to enjoy the full SU as well as the flexibility of deployment.

The data carrier after IA can use channel raster to establish the specific carrier.

Channel raster

Necessity of channel raster

During the last RAN4#116bis meeting, a WF [2] has been agreed and the discussion on channel raster is captured as below.

- Agreement

  - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands

    - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster

  - Investigate the interaction between the channel raster and the synchronization raster

  - Investigate the necessity of channel raster or alternative ways for the channel configuration

    - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster

    - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported

  - Study the listed main proposals especially for the migration and co-existence approaches

    - Other options not presented in this meeting are not precluded

  - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

On the necessity of channel raster, the pproblems of removing channel raster are listed as below.

- Operator issue:

How to put the base station carrier for cell deployment is a problem. the NW still need a reference of specific frequency center to locate its first initial access carrier and the following data carrier.

How to put the base station carrier for cell deployment is a problem if no channel raster defined.

- Co-existence with 5G and MRSS:

As the 5G channel raster is defined and illustrated above, the 6GR carrier frequency center needs to be SCS level aligned to mitigate interference, then an appropriate channel raster design is helpful.

Channel raster is needed for better co-existence and MRSS between 6GR and 5G.

- Sync raster design issue

The sync raster design is based on channel raster concept with two rules as we discussed above. If no channel raster is defined, there is no guidance for sync raster design.

Channel raster is needed to give guidance on sync raster design.

- Test issue, i.e, how to put the “minCBW” in the band.

Currently how to put the “5MHz” interference signal of many RX requirements are based on channel raster. If no channel raster defined, how to configure such test is questionable.

If no channel raster defined, how to configure the RX test is questionable.

- The complexity issue in indication of CBW location.

As we should agree, the channel center is indicated by channel raster with global raster of 5kHz granularity. Such payload is acceptable. However, if remove channel raster, it means 1Hz channel raster since the exact frequency of the channel center need to be signalled or indicated by signalling.

To indicate the exact frequency with 1Hz granularity needs large payload.

From above discussion and observations, we can see if no channel raster is introduced, the whole frequency framework will be broken. Furthermore, there is no gain we can reach to remove the channel raster. Hence, it is not preferred to remove the channel raster.

Removing channel raster makes the whole frequency framework broken and brings no gain.

It is proposed not to remove the channel raster.

6GR channel raster and co-existence with 5G

For the 5G-6GR co-existence, the key issue is the SCS are the same for both carriers. For adjacent channel co-existence, the problem is how to locate the 5G and 6G channels considering guard band as well as the channel raster. From our view, to keep the 5G channel raster and 6G channel raster as SCS level aligned helps to reduce the sub-carrier interference. Hence the MCD of 100kHz and 15kHz is 5kHz which also can cover 10kHz enhanced channel raster. The frequency separation of two 5G and 6G channel raster point than can be denoted as 100*m+5*n, in such case, it is simple to adjust the number of n to make it as multiple of 15.

For 5G-6GR co-existence, the same SCS is needed.

To consider the 5G and 6G channel raster as SCS level aligned helps to reduce the interference of adjacent carrier.

The 5kHz channel raster of 6G helps SCS level alignment of 5G and 6G channel raster.

During the NR discussion, the channel raster has been discussed and two proposals as 1 RB and SCS has been proposed and discussed. Due to the large granularity, the spectrum spill over might occur and 1 RB of SU might be sacrificed. So in the end the SCS has been agreed as the channel raster for 5G new bands.

While for LTE re-farming bands, 100kHz channel raster is still kept for better co-existence of NR and LTE. With the evolution, the conflict of 100kHz and 15/30kHz in FR1 has come out and 7.5kHz shift is introduced. Also, in the case of Cell bandwidth and UE specific CBW is configured with ODD/EVEN or EVEN/ODD RBs then the alignment of the two centers is impossible for 100kHz channel raster. Then the enhanced channel raster of 10kHz has been developed to solve such question.

5G has introduced 10MHz enhanced channel raster to get rid of 100kHz raster from LTE.

As from the above evolution of 5G, it seems the big question is two sets of channel raster as 100kHz from LTE and SCS for NR are used. In 6GR, we would like to get rid of the 100kHz channel raster’s impact, otherwise this 100kHz will be kept in all the following release. 5kHz as the greatest common divisor of 10kHz and 15kHz, can play a good role in channel raster for both MRSS as well as the channel spacing.

The NRARFCN is also based on 5kHz of global frequency raster which is the minimum granularity that is used for NR-ARFCN definition.

5kHz is the GCD of 10kHz and 15kHz and used as global frequency raster in NR-ARFCN.

Hence for the FR1 re-farming bands with 100kHz channel raster, to use 5kHz common channel raster can helps to get rid of LTE 100kHz which has caused lots of problem and also work as the GCD of 10/100/15kHz to achieve better MRSS.

So for 6GR, it is proposed as:

For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted

Conclusion

This paper discussed the 6GR channel arrangement, and got below observations and proposals.

Sync raster

Observation 1: 	In NR sync raster design, two rules has been agreed as

1, SCS level alignment of sync raster and channel raster

2, For each minCBW, at least one SSB to be covered

Observation 2: 	Kssb is introduced to find the RB edge based on rule 1 as SCS level alignment of sync raster and channel raster.

Observation 3: 	For 5MHz minCBW, the SS granularity is 1.2MHz for smaller than 3GHz and 1.44MHz for larger than 3GHz.

Observation 4: 	For 3MHz minCBW, the SS granularity is 0.6MHz.

Observation 5: 	The above SS granularity is based on the rule 2 as for each minCBW, at least one SSB to be covered.

Observation 6: 	The NR sync raster is well defined and further improvement needs to break the two rules.

Observation 7: 	How to improve the sync raster design and to what extent to break these two rules are trade-off of spectrum flexibility, system information payload and UE initial access complexity as well as latency.

Proposal 1: 	To consider larger channel raster, smaller SSB bandwidth and larger minCBW in different bands to reach the goal as reducing the sync raster points.

Proposal 2: 	The sync raster and channel raster design should consider the interference mitigation and improvement is needed to get rid of Kssb to save MIB payload.

Proposal 3: 	Define unified sync raster design for different bands, minCBW, SCS and channel raster in 6GR.

Observation 8: 	MinCBW, Channel raster, BWSSB are the main factors that impact the sync raster design. Mi CBW will limit the sync raster steps. Make sure all possible channel has SSB.

Observation 9: 	By replacing the channel raster to a reference sparse channel raster, coarser sync raster points can be reached.

Proposal 4: 	Consider a reference sparse channel raster compared to channel raster for data carrier for initial access.

Proposal 5: 	 6GR sync raster design to have coarser sync raster points and try to avoid Kssb.

Observation 10: 	Using 1RB as reference channel raster can avoid Kssb since the grid are RB level aligned.

Observation 11: 	The reference channel raster is coarser than the channel raster.

Observation 12: 	With the granularity of different minCBW are integer numbers of times of 3MHz granularity, the all-other sync raster points of larger minCBW are subset of sync raster points of 3MHz.

Observation 13: 	This ensure with different minCBW, no new sync raster points are introduced.

Observation 14: 	With 1RB reference channel raster, for different minCBW cases, large sync raster points reduction can be reached.

Proposal 6: 	It is proposed to use 1RB as reference channel raster for specific initial access carrier.

Proposal 7: 	The per band basis minCBW apply and the sync raster granularity can be defined accordingly.

| Minimum BW | SCS for SS | NRB,Carrier | ΔFSS,Raster |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 0.72 |
| 5 | 15 | 25 | 2.16 |
| 10 | 30 | 24 | 4.32 |
| 15 | 30 | 38 | 9.36 |
| 20 | 30 | 51 | 14.4 |

Observation 15: 	The data carrier can use channel raster to enjoy the full SU as well as the flexibility of deployment.

Proposal 8: 	The data carrier after IA can use channel raster to establish the specific carrier.

Channel raster

Observation 16: 	How to put the base station carrier for cell deployment is a problem if no channel raster defined.

Observation 17: 	Channel raster is needed for better co-existence and MRSS between 6GR and 5G.

Observation 18: 	Channel raster is needed to give guidance on sync raster design.

Observation 19: 	If no channel raster defined, how to configure the RX test is questionable.

Observation 20: 	To indicate the exact frequency with 1Hz granularity needs large payload.

Observation 21: 	Removing channel raster makes the whole frequency framework broken and brings no gain.

Proposal 9: 	It is proposed not to remove the channel raster.

Observation 22: 	For 5G-6GR co-existence, the same SCS is needed.

Observation 23: 	To consider the 5G and 6G channel raster as SCS level aligned helps to reduce the interference of adjacent carrier.

Observation 24: 	The 5kHz channel raster of 6G helps SCS level alignment of 5G and 6G channel raster.

Observation 25: 	5G has introduced 10MHz enhanced channel raster to get rid of 100kHz raster from LTE.

Observation 26: 	5kHz is the GCD of 10kHz and 15kHz and used as global frequency raster in NR-ARFCN.

Proposal 10: 	For re-farming FR1 bands with 100khz channel raster, using 5khz common channel raster, and avoid diverse channel raster in these bands. For other FR1 bands and new bands, SCS based channel raster is adopted

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

R4-2514641	WF on [116bis][101] 6G system parameter, Huawei