3GPP TSG-RAN WG4 Meeting #117                                                              R4-2521572

Dallas, USA, 17th – 21st November 2025

Source:	Nokia

Title:	On the issues for NR_LBCA_Sw_enh

Agenda item:	7.8.3

Document for:	Approval

Introduction

In previous meeting #116bis following way forward was recorded.[1]

Issue 1: Work scope clarification

-	Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19

-	Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier

Issue 2: Common understanding of the UE reference architecture

RAN4 has reached the common understanding of the following aspects related to the UE reference architecture:

-	The main path antenna is shared across Case 1 and Case 3 switching states

-	The diversity path antenna is shared across Case 1 and Case 3 switching states-	UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states

-	An example of the front end filter architecture for the case of adjacent bands is provided below for information

Issue 3: Open issues related to the UE reference architecture

-	Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation

-	Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination

-	Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above

-	Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

Issue 4: Switching period for switching between Case 1 and Case 3

-	Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations

-	Switching period capability values {35us, 70us, 140us}

-	The capability shall be per band pair per band combination

Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3

-	Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching

Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3

-	Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3

-	Whether confirmation by RAN1 is needed on any assumptions made by RAN4

-	How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests

Issue 7: Possible co-existence issues

-	The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis)

-	Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation

-	It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item

| Scenario | Type of interference | CA_n12-n29 | CA_n28-n67 |
| --- | --- | --- | --- |
| UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation) | Co-channel | UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)) |
|  | Blocking | Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out |
| BS to UE / UE to BS | Given that co-location is assumed, all those wouldn’t have an issue. |  |  |
| BS to BS | Co-channel interference and blocking | Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out. |

Issue 8: Impact on RX requirements

-	Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking

Discussion

Observing the band combination scenarios operators have sought for the switched LB CA framework in figure 1, limiting the band combinations in this work item for the two combinations makes sense as the SDL band and FDD DL bands are just adjacent each other. Aggregated reception means that there is no antenna tuning involved and DL switches between FDD DL and aggregated DL reception. There is no UL while DL aggregated reception. Aggregated total DL bandwidths are

- CA_n12A-n29A: 28MHz

- CA_n28A-n67A: 50MHz

The latter combination doesn’t need to cover full band n28 as it is associated with n67 which is EU specific band and in EU n28 DL extends only up to 788MHz.

In the previous meeting there were discussions that there might be interest in introducing this feature also for combinations that have more wide frequency separation which would be challenging for antenna design.

Figure 1 Operator provided scenarios for NR low band aggregation via switching

Reference architecture considerations

The reference architecture recorded in the WF implies that signal can be received either one or two downconversion. Following we reiterate what are the implications for these two methods when interferes are present and RFFE filter is covering two bands. In figure 2 operation with one receive chain is illustrated. As the analog channel filter is wide all the inband interferers are present without any attenuation at the ADC input. This is the same problem as identified in fragmented carrier study.

Figure 2 Aggregated DL operation with 1 DL LO and with interferers.

In figure 3 operation with two receive chains is illustrated. This case the analog channel filter can adapt to signal BW and provides attenuation to nearby interferers hence providing better resilience to interferers.

Figure 3 Aggregated DL operation with 2 DL LO and with interferers.

Two receiver implementation provides better resilience to interferers than one receiver.

Not using two receivers, adaptations from the fragmented carrier study items are needed to mitigate in-gap interference.

Switching time mask

Considering issue 5 in way forward the objectives of WI are focusing on only band combinations in close proximity of each other hence no antenna tuning is expected to be involved therefore reusing the rel-19 ON/OFF time mask as such can be seen excessive as UL transient periods are able to overlap with switching period. However, if this feature includes band combinations involving antenna tuner rel-19 ON/OFF time mask is justified.

If no antenna tuner present UL transient periods may overlap with switching period.

Co-existence

UE to UE co-existence

Our assumption is that CA_n28-n67 is restricted to EU frequencies but still considering a typical UE implementation it seems challenging to address co-existence. Maybe only way is to rely on likelihood of co-existence problem to happen as this feature is to be used away from urban areas.

BS to BS co-existence

CA_n28-n67 is having 5MHz guard band which may be enough to suppress the interference to an adequate level. But then similar guard band for CA_n12-n29 should be arranged by other means.

Reusing the rel-19 ON/OFF time mask as such is justified if the feature includes band combinations involving antenna tuner.

Mitigating the UE-to-UE co-existence issue may need to rely on the likelihood of co-existence problem to happen as the feature is to be used away from urban areas.

CA_n28-n67 is having 5MHz guard band which may be enough to suppress the interference to an adequate level but CA_n12-n29 does not.

For BS to BS co-existence, study if a minimum guard band for CA_n12-n29 between n12-UL and n29-SDL shall be considered.

Any new band combination in the future with carriers close to each other, only use cases where there is room for both a 3GPP defined channel bandwidth and sufficient guard band towards the uplink shall be considered.

Conclusion

In this paper, we express our views on the specification requirement and.

The following observations and proposals were made:

Observation 1: Two receiver implementation provides better resilience to interferers than one receiver.

Observation 2: Not using two receivers, adaptations from the fragmented carrier study items are needed to mitigate in-gap interference.

Observation 3: If no antenna tuner present UL transient periods may overlap with switching period.

Observation 4: Reusing the rel-19 ON/OFF time mask as such is justified if the feature includes band combinations involving antenna tuner.

Observation 5: Mitigating the UE-to-UE co-existence issue may need to rely on the likelihood of co-existence problem to happen as the feature is to be used away from urban areas.

Observation 6: CA_n28-n67 is having 5MHz guard band which may be enough to suppress the interference to an adequate level but CA_n12-n29 does not.

Proposal 1: For BS to BS co-existence arranging a guard band for CA_n12-n29 between n12-UL and n29-SDL should be studied.

Proposal 2: Any new band combination in the future with carriers close to each other, only use cases where there is room for both a 3GPP defined channel bandwidth and sufficient guard band towards the uplink shall be considered.

- References

- R4-2515156, Way Forward for [116bis][320] NR_low_band_CA_switching, Apple